| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0078-v2 |
| --- | --- |

| Title: | EE2-related: Modified GPM with inter and intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes modified geometric partitioning mode (GPM) with inter and intra prediction, which was proposed in JVET-W0110, to further enhance the coding performance beyond VVC. The proposed method consists of the following four modifications from the signalling and decoding process points of view against that of JVET-W0110.

- Combination of GPM-Inter/Intra and GPM-MMVD/TM

- Restriction of GPM-Intra/Intra to the inter CU

- Adaptive intra prediction mode derivation

- Application of GPM-Intra/Intra to the intra CU with option 3

Simulation results over all classes of the best coding performance and the best trade-off conditions over ECM-2.0 are shown as follows:

- The best coding performance condition over ECM-2.0:

  - RA: BD-rate YUV: -0.13%, -0.39%, -0.30%; EncT: 102%; DecT: 100%

  - LB: BD-rate YUV: -0.24%, -0.77%, -0.87%; EncT: 102%; DecT: 100%

  - LP: BD-rate YUV: -0.77%, -2.14%, -1.79%; EncT: 121%; DecT: 101%

- The best trade-off condition over ECM-2.0:

  - RA: BD-rate YUV: -0.09%, -0.30%, -0.20%; EncT: 101%; DecT: 101%

  - LB: BD-rate YUV: -0.24%, -0.60%, -0.58%; EncT: 102%; DecT: 100%

  - LP: BD-rate YUV: -0.66%, -1.71%, -1.55%; EncT: 106%; DecT: 101%

Introduction

In the previous meeting, geometry partition mode (GPM) with inter and intra prediction (GPM-Inter/Intra) was proposed by JVET-W0110 [1] to improve the coding efficiency of the GPM in VVC. With the method, pre-defined intra prediction modes (IPMs) against geometric partitioning line can be selected in addition to merge candidates for each non-rectangular split region in the GPM-applied CU. In the method, whether intra or inter prediction mode is determined for each GPM-separated region with a flag from the encoder. When the inter prediction mode, a uni-prediction signal is generated by MVs from the merge candidate list. On the other hand, when the IPM, a uni-prediction signal is generated from the neighboring pixels for the IPM specified by an index from the encoder. These two uni-prediction signals are blended with the same way of ordinary GPM.

Problem Statement

There is room to further improve the coding performance of GPM-Inter/Intra from the signalling and decoding process points of view. First, GPM with MMVD and template matching (GPM-MMVD/TM), which was proposed by JVET-W0097 [2] and adopted in ECM-2.0 [3], are not used together with GPM-Inter/Intra implemented in the current meeting EE2-3.2 software [4][5]. Next, depending on the geometric shape, a non-optimal IPM may be applied to the GPM-splitting region far from the reference pixels while the optimal IPM can be applied to those near the reference pixels as shown in Fig. 1. Then, the application of GPM-Intra/Intra to the intra CU is not available in the EE2-3.2 software. Finally, the variation of intra prediction is restricted only to the parallel and perpendicular angular modes against geometric partitioning line and planar mode in the EE2-3.2 software.

Figure 1 An example where a non-optimal IPM is applied to the GPM-splitting region far from the reference pixels while the optimal IPM is done to those near the reference pixels.

Proposal

In this contribution, the following four modifications for JVET-W0110 are proposed to further improve the coding performance of GPM-Inter/Intra. The detail of each portion is described in the corresponding sub-section.

- Combination of GPM-Inter/Intra and GPM-MMVD/TM

- Restriction of GPM-Intra/Intra to the inter CU

- Adaptive IPM derivation

- Application of GPM-Intra/Intra to the intra CU with option 3

Regarding the option 3, two different derivation schemes, which are utilized with DIMD/TIMD (w/ DIMD/TIMD) or with IPM of the neighboring block (w/ NIPM), are proposed. The derivation process is described as follows. First, the size of the IPM candidate list is 3 and the parallel angular prediction mode is always registered as the first candidate in the list. Next, the derived IPMs are registered in the second and third candidates in the list. When the derived IPM is same as the first candidate, the derived IPM is not registered in the list like the pruning process. If the list is not fulfilled by the derived IPMs, the perpendicular angular mode and then planar mode are tried to register.

Regarding the option 4, the adaptive IPM derivation based on the option 3 is utilized but the size of the IPM candidate list is 6 the same as that of VVC.

Simulation Results

The proposed methods are implemented on the top of ECM-2.0, which are defined as the anchor software in EE2 for tool-off test [6]. Simulation conditions are followed by the common test condition (CTC) [7]. In the simulations, the following 11 tests shown in the Table 0 are conducted to study the effect from the variation of possible IPMs, described in the Table 0 option 0 column, and the proposed options. A and B in the Table 0 option 0 column denote the pre-defined IPMs. The parallel angular mode (Para.) and the perpendicular angular mode (Perp.) against the geometric partitioning line can be available in the method A while the planar mode (Planar) can be also utilized besides the two modes in the method B.

Table 0 The test condition of each proposed portion.

|  | Option 0 | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- | --- |
| EE2-3.2a | A | N | N | N | N |
| EE2-3.2b | B | N | N | N | N |
| Test 1 | A | Y | N | N | N |
| Test 2 | B | Y | N | N | N |
| Test 3 | A | Y | Y | N | N |
| Test 4 | B | Y | Y | N | N |
| Test 5 | B | Y | Y | Y (w/ DIMD/TMD) | N |
| Test 6 | B | Y | Y | Y (w/ NIPM) | N |
| Test 7 | B | Y | Y | Y (w/ DIMD/TIMD/NIPM) | N |
| Test 8 | B | N | Y | Y (w/ DIMD/TIMD/NIPM) | N |
| Test 9 | N/A | N/A | N/A | N/A | Y (w/ DIMD/TIMD/NIPM) |

Note that the proposed method includes an implementation omission in ECM-2.0 regarding intra mode storage for GPM-applied blocks, which was tested in EE2-3-10 (the combination of EE2-3.3, EE2-3.4 and EE2-3.5) of the previous meeting [2][8] and was adopted.

The following Tables 1 to 9 corresponds the Tests 1 to 9 shown in Table 0. From the comparison of Tables 1 to 9, the best coding performance condition and best trade-off condition are Test 7 and Test 8, respectively.

Table 1 : Simulation results of Test 1 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.06% | -0.26% | -0.25% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.02% | -0.29% | -0.07% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.04% | -0.26% | -0.25% | 102% | 100% | -0.15% | -0.61% | -0.41% | 101% | 100% | -0.41% | -1.35% | -1.37% | 117% | 100% |
| C | -0.09% | -0.21% | -0.12% | 101% | 100% | -0.24% | -0.63% | -0.47% | 101% | 99% | -0.73% | -1.83% | -1.54% | 122% | 100% |
| E |  |  |  |  |  | 0.20% | -0.20% | 0.02% | 102% | 101% | -0.12% | -0.83% | -0.50% | 120% | 102% |
| Overall | -0.05% | -0.25% | -0.18% | 101% | 100% | -0.09% | -0.51% | -0.32% | 101% | 100% | -0.45% | -1.38% | -1.21% | 119% | 101% |
| D | -0.01% | -0.30% | 0.00% | 101% | 100% | -0.06% | -1.10% | -0.95% | 101% | 100% | -0.34% | -1.52% | -1.31% | 118% | 101% |
| F | 0.03% | -0.09% | -0.18% | 101% | 100% | -0.01% | -0.83% | -0.81% | 101% | 100% | -0.57% | -0.47% | 0.88% | 111% | 100% |

Table 2 : Simulation results of Test 2 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.04% | -0.32% | -0.19% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | 0.00% | -0.16% | 0.02% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.05% | -0.35% | -0.33% | 102% | 100% | -0.14% | -0.30% | -0.33% | 101% | 100% | -0.48% | -1.51% | -1.35% | 118% | 100% |
| C | -0.08% | -0.25% | -0.14% | 102% | 100% | -0.19% | -0.63% | -0.33% | 102% | 99% | -0.86% | -2.13% | -1.61% | 123% | 99% |
| E |  |  |  |  |  | 0.11% | -0.63% | -0.44% | 102% | 101% | -0.15% | -0.80% | -0.43% | 121% | 102% |
| Overall | -0.04% | -0.28% | -0.18% | 101% | 100% | -0.09% | -0.49% | -0.36% | 102% | 100% | -0.52% | -1.54% | -1.21% | 120% | 100% |
| D | -0.03% | -0.28% | -0.02% | 101% | 99% | -0.05% | -0.03% | -0.09% | 101% | 100% | -0.38% | -1.35% | -1.50% | 119% | 100% |
| F | 0.00% | -0.16% | -0.11% | 101% | 99% | -0.36% | -0.44% | -0.16% | 101% | 100% | -0.33% | -0.01% | 0.52% | 112% | 100% |

Table 3 : Simulation results of Test 3 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.05% | -0.07% | -0.26% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.05% | -0.24% | -0.05% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.05% | -0.21% | -0.30% | 101% | 100% | -0.15% | -0.23% | -0.67% | 101% | 100% | -0.45% | -1.36% | -1.10% | 117% | 100% |
| C | -0.11% | -0.19% | -0.18% | 101% | 101% | -0.20% | -0.72% | -0.29% | 101% | 99% | -0.77% | -1.90% | -1.40% | 122% | 99% |
| E |  |  |  |  |  | 0.26% | -0.47% | -0.32% | 102% | 101% | -0.11% | -0.89% | -0.30% | 121% | 101% |
| Overall | -0.06% | -0.18% | -0.21% | 101% | 100% | -0.06% | -0.45% | -0.46% | 101% | 100% | -0.47% | -1.42% | -1.00% | 120% | 100% |
| D | -0.03% | -0.29% | -0.16% | 100% | 100% | -0.03% | -0.65% | -0.71% | 100% | 100% | -0.43% | -1.47% | -1.48% | 118% | 101% |
| F | -0.02% | -0.16% | -0.14% | 100% | 100% | -0.19% | -0.39% | -0.52% | 100% | 100% | -0.39% | -0.97% | -0.39% | 111% | 100% |

Table 4 : Simulation results of Test 4 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.07% | -0.24% | -0.19% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.03% | -0.17% | 0.00% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.08% | -0.29% | -0.31% | 102% | 100% | -0.17% | -0.39% | -0.34% | 101% | 99% | -0.50% | -1.65% | -1.12% | 118% | 100% |
| C | -0.13% | -0.26% | -0.12% | 102% | 100% | -0.25% | -0.62% | -0.34% | 101% | 99% | -0.89% | -2.07% | -1.60% | 123% | 99% |
| E |  |  |  |  |  | 0.14% | -0.28% | -0.33% | 102% | 101% | -0.29% | -0.55% | -1.11% | 121% | 102% |
| Overall | -0.08% | -0.25% | -0.17% | 101% | 100% | -0.12% | -0.44% | -0.34% | 101% | 100% | -0.58% | -1.52% | -1.28% | 120% | 100% |
| D | -0.06% | -0.17% | 0.14% | 101% | 100% | -0.05% | -0.87% | -0.78% | 101% | 100% | -0.46% | -1.55% | -1.65% | 118% | 101% |
| F | -0.04% | -0.24% | -0.20% | 101% | 100% | -0.26% | -0.27% | -0.39% | 100% | 100% | -0.58% | -0.99% | -0.31% | 112% | 101% |

Table 5 : Simulation results of Test 5 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.12% | -0.45% | -0.28% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.07% | -0.35% | -0.22% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.11% | -0.47% | -0.39% | 102% | 100% | -0.26% | -0.60% | -0.45% | 102% | 100% | -0.64% | -1.94% | -1.55% | 118% | 101% |
| C | -0.19% | -0.44% | -0.22% | 102% | 100% | -0.37% | -0.87% | -0.70% | 101% | 100% | -1.16% | -2.58% | -2.18% | 123% | 101% |
| E |  |  |  |  |  | -0.05% | -0.47% | -0.85% | 103% | 100% | -0.32% | -1.18% | -1.08% | 122% | 102% |
| Overall | -0.13% | -0.43% | -0.29% | 102% | 100% | -0.24% | -0.66% | -0.63% | 102% | 100% | -0.74% | -1.96% | -1.64% | 121% | 101% |
| D | -0.09% | -0.45% | -0.17% | 101% | 100% | -0.19% | -0.94% | -1.30% | 101% | 101% | -0.60% | -1.09% | -1.80% | 119% | 103% |
| F | -0.06% | -0.24% | -0.32% | 101% | 100% | -0.17% | -0.32% | -0.19% | 102% | 101% | -0.44% | -1.05% | -0.56% | 112% | 100% |

Table 6 : Simulation results of Test 6 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.10% | -0.36% | -0.28% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.05% | -0.25% | -0.13% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.10% | -0.34% | -0.38% | 102% | 100% | -0.20% | -0.52% | -0.78% | 102% | 99% | -0.57% | -1.60% | -1.51% | 118% | 100% |
| C | -0.15% | -0.32% | -0.33% | 102% | 100% | -0.31% | -1.06% | -0.67% | 102% | 99% | -1.04% | -2.72% | -2.15% | 124% | 99% |
| E |  |  |  |  |  | 0.05% | -1.08% | -0.52% | 103% | 101% | -0.37% | -1.23% | -1.16% | 122% | 101% |
| Overall | -0.10% | -0.32% | -0.29% | 102% | 100% | -0.17% | -0.84% | -0.68% | 102% | 100% | -0.68% | -1.88% | -1.63% | 121% | 100% |
| D | -0.05% | -0.25% | 0.07% | 102% | 100% | -0.10% | -0.24% | -0.76% | 103% | 100% | -0.47% | -1.08% | -1.48% | 120% | 101% |
| F | -0.04% | -0.20% | -0.16% | 101% | 100% | -0.20% | -0.24% | 0.13% | 101% | 99% | -0.49% | -0.78% | -0.36% | 112% | 100% |

Table 7 : Simulation results of Test 7 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.12% | -0.35% | -0.29% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.06% | -0.34% | -0.14% | 102% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.13% | -0.43% | -0.42% | 102% | 100% | -0.29% | -0.62% | -0.91% | 101% | 100% | -0.69% | -1.90% | -1.63% | 118% | 101% |
| C | -0.20% | -0.39% | -0.25% | 102% | 100% | -0.40% | -1.07% | -0.99% | 101% | 100% | -1.15% | -2.78% | -2.23% | 124% | 100% |
| E |  |  |  |  |  | 0.06% | -0.61% | -0.67% | 103% | 100% | -0.39% | -1.68% | -1.48% | 122% | 102% |
| Overall | -0.13% | -0.39% | -0.30% | 102% | 100% | -0.24% | -0.77% | -0.87% | 102% | 100% | -0.77% | -2.14% | -1.79% | 121% | 101% |
| D | -0.05% | -0.57% | -0.26% | 101% | 100% | -0.16% | -0.77% | -1.49% | 101% | 101% | -0.67% | -2.05% | -1.79% | 119% | 102% |
| F | -0.09% | -0.24% | -0.23% | 101% | 100% | -0.26% | -0.57% | 0.08% | 101% | 100% | -0.37% | -0.92% | -1.04% | 112% | 101% |

Table 8 : Simulation results of Test 8 over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.10% | -0.15% | -0.24% | 101% | 101% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.02% | -0.22% | -0.06% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.09% | -0.33% | -0.26% | 101% | 101% | -0.29% | -0.41% | -0.65% | 101% | 100% | -0.60% | -1.54% | -1.64% | 105% | 100% |
| C | -0.14% | -0.42% | -0.20% | 101% | 101% | -0.39% | -1.00% | -0.68% | 102% | 100% | -0.99% | -2.17% | -1.71% | 106% | 100% |
| E |  |  |  |  |  | 0.05% | -0.38% | -0.35% | 102% | 101% | -0.32% | -1.38% | -1.20% | 108% | 101% |
| Overall | -0.09% | -0.30% | -0.20% | 101% | 101% | -0.24% | -0.60% | -0.58% | 102% | 100% | -0.66% | -1.71% | -1.55% | 106% | 100% |
| D | -0.03% | -0.18% | 0.07% | 101% | 100% | -0.01% | -0.20% | -1.01% | 101% | 101% | -0.46% | -1.34% | -1.90% | 106% | 101% |
| F | -0.05% | -0.08% | -0.01% | 100% | 100% | -0.41% | -0.39% | -0.69% | 100% | 100% | -0.59% | -1.24% | -0.96% | 103% | 100% |

Table 9 : Simulation results of Test 9 over ECM-2.0 (tool-off).

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT |
| A1 | -0.04% | 0.07% | -0.07% | 107% | 100% |
| A2 | -0.07% | -0.03% | -0.04% | 109% | 100% |
| B | -0.03% | -0.06% | -0.05% | 109% | 100% |
| C | -0.01% | 0.00% | -0.10% | 109% | 100% |
| E | -0.05% | -0.12% | 0.07% | 110% | 100% |
| Overall | -0.04% | -0.03% | -0.04% | 109% | 100% |
| D | 0.02% | 0.05% | 0.04% | 108% | 100% |
| F | -0.04% | -0.23% | 0.04% | 106% | 99% |

Conclusion

In this contribution, GPM with intra prediction was proposed to improve coding efficiency beyond VVC. Simulation results reportedly show the best coding performance and the best trade-off conditions of the proposed methods provide potentially -0.13%/-0.24% and -0.09/-0.24% BD-rate savings of luma component with almost no increment of encoding and decoding runtime over ECM-2.0 in the RA/LB configuration. In contrast, the results of the two conditions offer -0.77% and -0.66% BD-rate savings of luma component with 21% and 6% increments of encoding runtime over ECM-2.0 in the LP configuration. Since the GPM is enabled by the proposal in the LP configuration, the EncT is larger than those of RA/LB due to the incremental coding process of the GPM itself for the GPM-disabled anchors. Besides, in the best coding performance condition, the EncT is further increased compared to that in the best trade-off condition since the RDO of GPM-Intra is also tried for GPM-MMVD/TM. It was asserted that the coding performance is improved by the proposed GPM-Intra from the overall simulation results, we recommend it will be adopted in the next ECM.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Y. Kidani, H. Kato, and K. Kawamura, “AHG12: GPM with inter and intra prediction,” JVET-W0110, by teleconference, 7–16 July 2021.

- X. Xiu, C.-W. Kuo, X. Wang, R.-L. Liao, Y. Ye, X. Li, J. Chen, Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang, Y.-J. Chang, H. Huang, V. Seregin, C.-C. Chen, and M. Karczewicz, “EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5,” JVET-W0097, by teleconference, 7–16 July 2021.

- ECM-2.0: https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/tree/ECM-2.0

- Y. Kidani, H. Kato, and K. Kawamura, “EE2-3.2: GPM with inter and intra prediction (JVET-W0110),” JVET-X0XXX, by teleconference, 6–15 October 2021.

- EE2-3.2: https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/tree/TEST_3.2

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-W2024, by teleconference, 7–16 July 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

- EE2-3-10 based on ECM-1.0 in JVET-V meeting: https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM/-/tree/TEST_3.3_3.4_3.5

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).