| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0085-v1 |
| --- | --- |

| Title: | Non-EE2: Template Matching-based Reordering for Extended MMVD Design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Na Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com zhangna.cynthia@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

A template matching based reordering method for extended MMVD is proposed, in which the MMVD refinement positions are reordered based on template costs. In the extended MMVD, additional positions are added along k×π/8 diagonal angles, meanwhile half of distance offsets in the current MMVD design are removed. After the reordering process, the top 1/8 positions with the smallest SAD costs are kept as available positions, consequently for MMVD index coding.

Simulation results are reported as (tool-off tests, ECM-2.0 as the anchor):

RA: -0.12%, -0.17%, -0.15%, 101%, 102%; LB: -x.xx%, xxx%, xxx%.

# Introduction

The tool off test results for MMVD in current ECM-2.0 [1] show BD-rate reductions of -0.08% / -0.07 for RA and LDB respectively, implying that a more efficient design is desirable to determine and represent refinement positions in MMVD.

# Proposed method

In this document we propose a template matching based reordering method for extended MMVD (TMRE-MMVD).

First, we propose to add additional refinement positions along k×π/8 diagonal angles as depicted in Fig. 1 thus increase the number of directions from 4 to 16. Meanwhile, it is proposed to remove every other distance offsets as depicted in Fig. 2 thus reduce the number of distance offsets along each direction from 8 to 4.

Fig. 1: Additional directions along k×π/8 diagonal angles.

Fig. 2: Removing every other distance offset.

Second, based on the SAD cost between the template (one row above and one column left to the current block) and its reference for each refinement position, we reorder all the possible MMVD refinement positions (16×4) for each base candidate.

Finally, the top 1/8 (1/8 * 16 * 4 = 8) refinement positions with the smallest template SAD costs are kept as available positions, consequently for MMVD index coding. The MMVD index is binarized by the rice code with the parameter equal to 2 as showed in table 1.

Table 1: Binarization for reordered MMVD index.

| Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binarization | 00 | 01 | 100 | 101 | 1100 | 1101 | 1110 | 1111 |

# Experimental results

The tests were implemented on top of ECM-2.0. The simulations were performed following the the common test conditions [2]. Table 1 shows the overall results for RA and LDB tests compared to ECM-2.0.

Table 1: Performance of the proposed method

|  | RA |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.31% | -0.34% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.14% | -0.17% | -0.15% | 101% | 102% |  |  |  |  |  |
| Class B | -0.10% | -0.12% | -0.12% | 101% | 103% |  |  |  |  |  |
| Class C | -0.10% | -0.12% | -0.04% | 101% | 104% | -0.22% | -0.20% | 0.05% | 100% |  |
| Class E |  |  |  |  |  | -0.07% | -0.43% | -0.47% | 101% |  |
| Overall | -0.12% | -0.17% | -0.15% | 101% | 102% |  |  |  |  |  |
| Class D | -0.10% | -0.26% | 0.05% | 101% | 103% | -0.16% | -0.23% | -0.48% | 102% |  |
| Class F | -0.03% | -0.08% | -0.19% | 101% | 102% | -0.35% | -0.28% | -0.18% | 101% |  |

# Conclusions

In this contribution, TMRE-MMVD is proposed. With a negligible impact on encoder and decoder complexity, it provides a reasonable coding gain. It is recommended to further study this new method in EE2.

# References

[1] ECM-2.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).