| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0099 |
| --- | --- |

| Title: | Non-EE2: Extension of TIMD to intra chroma coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2, Hongbin Liu1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents to extend template-based intra mode derivation to intra chroma coding, known as TIMD-Chroma. Compared to ECM-2.0, simulation results of the proposed method are reported as below:

AI: ;

RA: .

Introduction

Template-based intra mode derivation (TIMD) was adopted in ECM in JVET-W meeting [1], in which the luma intra prediction mode is derived using a template. TIMD could be extended to chroma components naturally.

Proposed method

This contribution presents to extend template-based intra mode derivation to intra chroma coding, known as TIMD-Chroma. Similar to TIMD for luma, the chroma intra prediction mode is also derived using a template consisting of neighbouring samples of the current chroma block. The derived TIMD-Chroma mode may be one normal intra prediction mode (in the range of 67 modes) or one CCLM mode. A CU level flag is signalled to indicate whether the proposed TIMD-Chroma method is used.

Simulation results

The proposed method was implemented on top of the ECM-2.0 [2]. Simulations were performed following the common test conditions [3]. More detailed results could be found in the attached excel sheets. The results compared to ETM-2.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-2.0

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.12% | 0.09% | 0.02% | 100% | 106% | -0.05% | -0.06% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.10% | 0.11% | 0.03% | 101% | 108% | 0.01% | -0.17% | -0.27% | 101% | 100% |
| Class F (Optional) |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, template-based intra mode derivation is presented to extend to chroma components. It is recommended to adopt the proposed method in ECM.

Reference

- K. Cao, N. Hu, V. Seregin, M. Karczewicz, Y. Wang, K. Zhang, and L. Wang, “EE2-related: Fusion for template-based intra mode derivation,” Document of Joint Video Experts Team, JVET-W1023, by teleconference, July 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-W2017, by teleconference, July 2021.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).