| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0100-v2 |
| --- | --- |

| Title: | EE2-related: On propagating intra prediction mode for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Xiang Li, Ling Li, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In order to improve the accuracy of the construction of MPM list, intra prediction mode propagation for inter CU was adopted in JVET-W meeting and ECM-2.0 reference software. However, the proposed intra mode propagation is applied on inter CU only. In this proposal, we propose to simply extend the idea to IBC blocks. The overall performance impact of the proposed method on top of ECM-2.0 for SCC content is reported as below. 
For Class-F test sequence:

- AI: -0.06%/-0.04%/-0.02% with 100% EncT and 100% DecT

- RA:-0.02%/-0.08%/-0.07% with 101% EncT and 101% DecT

For Class-TGM test sequence:

- AI: -0.05%/-0.08%/-0.01% with 101% EncT and 101% DecT

- RA: 0.00%/-0.06%/-0.04% with 101% EncT and 101% DecT

# Introduction

In JVET-V0098[1], a template-based intra mode derivation (TIMD) method is proposed. Instead of selecting the only one intra mode with the smallest STAD cost, JVET-W0123 proposes to choose the first two modes with the smallest SATD costs which are derived with TIMD method. And then these two modes are fused with the weight, and the weighted intra prediction is used as the intra predictor for the current CU. During the generation of MPM list, intra prediction mode of the neighboring CU is derived as Planar mode in default if it is encoded as inter CU. To improve the accuracy of MPM list, an intra mode propagation method for inter CU in [3] is used in the construction of MPM list in JVET-V0098[1] and JVET-W0123[2]. In [3], the intra prediction mode could be propagated for the inter CU via motion vector and reference picture, and the propagated intra modes of the inter CU could be used in the construction of MPM list.

# Proposal

In order to improve the accuracy of the MPM list construction, the propagated intra mode of the neighboring inter CU, is derived from the motion vector and reference picture. However, the planar mode is still used for the MPM list construction when the neighboring CU is encoded as IBC prediction mode.

In this proposal, we propose to simply extend the idea of intra mode propagation not only for inter blocks but also for IBC blocks, namely deriving the intra prediction mode by using the block vector (BV) of IBC and to store the intra prediction mode information for the purpose of intra mode propagation. The propagated intra modes of IBC block is to be used for the construction of MPM list when the neighboring block is IBC block. There are two stages for the derivation of the intra prediction mode for IBC block. At the first stage, the BV is applied to the current CU to locate the reference block in the reconstructed picture. In order to get the intra prediction mode from the reference block, the reference block which an IBC referenced to via BV is divided into 4×4 units. For each 4×4 unit, we use the top-left position (0,0) to derive the corresponding intra prediction mode as shown in Figure 1. Then, the corresponding intra prediction modes which are already stored in 4×4 unit after the encoding of the collocated CU in Figure 1 are propagated to the current CU at the second stage.

Figure 1: Example of intra prediction mode propagation for IBC by using BV

# Simulation Results

The proposed method is implemented on top of ECM-2.0. Table 1 summarize the result of the proposed intra prediction mode propagation for IBC.

Table 1: Simulation results on propagating intra prediction mode for IBC on top of ECM-2.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class F | -0.06% | -0.04% | -0.02% | 100% | 100% |
| Class TGM | -0.05% | -0.08% | -0.01% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | -0.02% | -0.08% | -0.07% | 101% | 101% |
| Class TGM | 0.00% | -0.06% | -0.04% | 101% | 101% |

According to the above table, the intra prediction mode propagation for IBC block could have more than 0.05% coding gain improvement for luma component with negligible runtime increment. In RA case, it could also have 0.02% coding gain in luma channel for ClassF.

Moreover, we also evaluate the performance impact on propagating the intra prediction mode for inter CU in Table 2. The anchor is the ECM-2.0 software. We disable the intra mode propagation for inter CU in the test. Table 2 shows that the intra mode propagation for inter CU only has minor performance impact for ClassF and ClassTGM in RA case.

Table 2: Simulation results on disabling the intra prediction mode propagation for inter CU on top of ECM-2.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.05% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.03% | 0.00% | 99% | 99% |
| Class C | 0.02% | 0.03% | 0.12% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.04% | 100% | 100% |
| Class D | 0.07% | -0.36% | 0.05% | 101% | 101% |
| Class F | 0.02% | -0.01% | -0.10% | 100% | 100% |
| Class TGM | 0.00% | -0.06% | 0.00% | 100% | 100% |

# Conclusion

In order to improve the accuracy of the construction of MPM list, intra prediction mode propagation for inter CU was adopted in JVET-W meeting and ECM-2.0 reference software. In this proposal, we propose to simply extend the idea to IBC blocks. The intra prediction mode is derived by using the BV of IBC and then is stored within the intra prediction mode map of the current IBC for the purpose of intra mode propagation. The propagated intra modes of IBC block is to be used for the construction of MPM list when the neighboring block is IBC block.

# Reference

- Y. Wang, L. Zhang, K. Zhang, Z. Deng, and N. Zhang, “EE-related: Template-based intra mode derivation using MPMs”, ISO/IEC JTC1/SC29/WG11 JVET-V0098, April 2021.

- K. Cao, N. Hu, V. Seregin, M. Karczewicz, Y. Wang, K. Zhang, and L. Zhang, “EE-related: Fusion for template-based intra mode derivation”, ISO/IEC JTC1/SC29/WG11 JVET-W0123, July 2021.

- K. Zhang, L. Zhang, W.-J. Chien, and M. Karczewicz, “Intra-Prediction Mode Propagation for Video Coding”, IEEE J. Emerg. Sel. Topics Power Electron., vol. 9, no. 1, March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | inISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).