| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0102 |
| --- | --- |

| Title: | AHG11: Deep neural network for inter bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Young-Ju Choi Young-Woon Lee Byung-Gyu Kim Jongho Kim Se Yoon Jeong | Tel: Email: | yj.choi@ivpl.sookmyung.ac.kr yw.lee@ivpl.sookmyung.ac.kr bg.kim@sookmyung.ac.kr pooney@etri.re.kr jsy@etri.re.kr |
| Source: | Sookmyung Women’s University and ETRI |  |  |

_____________________________

- Abstract

This contribution proposes a deep neural network to generate a bi-prediction block by blending two motion compensated blocks. The proposed method replaces the regular bi-prediction methods. Compared with VTM-11.0, the proposed method reportedly shows on average 0.98% and 2.08% BD-rate reductions for Y component on B and D classes under RA configurations, respectively.

Introduction

In this contribution, a neural network-based inter bi-prediction scheme is proposed as a replacement of regular bi-prediction methods. In VVC [1], the bi-prediction block is generated from two different prediction blocks by utilizing several regular bi-prediction modes. The regular bi-prediction modes consist of averaging, weighted averaging called BCW (bi-prediction with CU-level weight), and averaging with bi-directional optical flow (BDOF)-based refinement. The proposed neural network-based bi-prediction mode replaces all of the regular bi-prediction modes on some of the block types. The proposed method is applied only on the luma component, and regular averaging is applied on chroma components.

Proposed Method

Architecture

The overall architecture of the proposed neural network is illustrated in Figure 1. The proposed network takes two prediction blocks P0 and P1 as the input and produces a final bi-prediction block Pf. Each of the two input prediction blocks are fed into three CNN layers to increase the number of features. The attention map between two features can be obtained by using the dot product and the sigmoid activation function. After that, the attention map is then multiplied to the original embedded features. The embedded features and attended features are then concatenated. The features pass through two CNN layers and 10 residual blocks wherein no batch normalization units [2], and two CNN layers, sequentially. Finally, the final bi-prediction block is obtained by adding the predicted residual block to an averaging block.

For all the CNN layers, kernel size of 3x3 is used. And the number of feature in each CNN layers except last layer is set to 64. The last CNN layer produces 1 feature map. For activation function, LeakyReLU is used.

Figure 1. The architecture of the proposed attention-based bi-prediction neural network.

Inference

In inference stage, PyTorch 1.7.1 is used for performing the proposed neural network-based bi-prediction mode in VTM-11.0. The inference information is summarized in Table 1 as suggested by [3]. The proposed method is applied to three block types (128x128, 64x64, and 32x32).

Table 1. Inference information.

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Quadro RTX 8000-48GB |
|  | CPU Type | Intel(R) Xeon(R) Gold 6256 CPU @ 3.60GHz |
|  | Framework: | Standalone C++, PyTorch 1.7.1 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | 1295k |
|  | Parameter Precision (Bits) | 32 (F) |
| Optional | Total Conv. Layers | 32 |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 block at a time |
|  | Patch size | Depending on the block (128x128, 64x64, 32x32) |

Training

In training stage, PyTorch 1.4.0 is used for training the proposed neural network. For different block types (128x128, 64x64, and 32x32) and QPs (22, 27, 32, 37, and 42), the proposed network is trained independently. The proposed network is trained The BVI-DVC [4] dataset is used to train the neural network. The sequences are compressed by VTM-11.0 with RA configurations. The dataset contains about 9M pair of blocks. The training information is summarized in Table 2 as suggested by [3].

Table 2. Training information.

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce RTX 2080 Ti-11GB |
|  | CPU Type | Intel(R) Xeon(R) CPU E5-2620 v4 @ 2.10GHz |
|  | Framework: | PyTorch 1.4.0 |
|  | Number of GPUs per Task: | 1 |
|  | Number of Iterations: | 600k |
|  | Batch size: | 64 |
|  | Total Parameter Number: | 1295k |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {22, 27, 32, 37, 42} |
| Optional | Patch size | 32x32x2, 64x64x2, 128x128x2 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | Charbonnier penalty function |
|  | Preprocessing: | Augmentation : Random horizontal and vertical flips, 90° rotation |

Experimental Results

The common test conditions defined in [3] are used to evaluate the proposed method. For all sequences in class B, a subset of frames equivalent to 1 second is used. The experimental results on VTM-11.0 under RA configuration are shown in Table 3. It is observed that averagely 0.98% and 2.08% BD-rate reductions are achieved for Y component on B and D classes under RA configurations, respectively.

Table 3. Experimental results on VTM-11.0 (class B: 1 sec, class C/D: 10 sec, GPU time).

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.98% | 0.25% | 0.19% | 6494% | 660% |
| Class C |  |  |  |  |  |
| Class D | -2.08% | -0.31% | -0.41% | 3029% | 1012% |
| Overall |  |  |  |  |  |

Conclusions

This contribution presented a deep neural network-based inter bi-prediction method. Experimental results demonstrated that the proposed scheme achieved the quality improvement compared with VTM-11.0.

References

- J. Chen, Y. Ye, and and S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 11 (VTM 11)”, JVET-T2002, 20th Meeting: by teleconference, 7 – 16 Oct. 2020.

- B. Lim, S. H. Son, H. W. Kim, S. J. Nah, and K. M. Lee, “Enhanced deep residual networks for single image super-resolution,” in Proceedings of the IEEE conference on computer vision and pattern recognition Workshops, 2017, pp. 136–144.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-W2016, 23rd Meeting: by teleconference, 7 – 16 Jul. 2021.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- Patent rights declaration(s)

Sookmyung Women’s University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).