| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0104-v1 |
| --- | --- |

| Title: | Update on the progress of the optimized VVC encoder implementation, Ali266 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Dong, S. Fang, Z. Huang, J. Liu, S. Xu, R. Yang, L. Yu, J. Chen, R.-L. Liao, Y. Ye | Tel: Email: | yan.ye@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports updated information on runtime and coding efficiency of Ali266, an optimized software VVC encoder developed by Alibaba. Ali266 was first presented in JVET-W0127 [1], it supports two presets, with its fast preset aimed at real-time encoding applications. In JVET-W0127, Ali266 was able to achieve encoding speed of 37.7 fps for 720p sequences.

In this document, we report further improvement in Ali266’s encoding speed at its fast preset, and also the addition of 8-bit encoding to Ali266 with the continued support of 10-bit encoding. We report that a 1.9x encoding speedup has been achieved in the past three months for video sequences at 1920x1080 or 1080x1920 resolutions. Currently, when configured with fast preset, Ali266 achieves average encoding speeds of 36.3 fps for classes 1920x1080 / 1080x1920 and 63.4 fps for classes 1280x720 / 720x1080. Based on such data, it is our new conclusion that Ali266 is a VVC encoder capable of real-time encoding for 1080p sequences, and Ali266 is more ready for real-world commercial applications.

Besides the speed improvement, Ali266’s encoding performance is also improved. For the JVET CTC content, Ali266 at slow preset achieves BD rates for {Y, U, V, YUV} of {10.95%, -6.11%, -4.50%, 7.11%} under the Random Access configuration with an average speedup factor of 227 times over VTM-13.0. For the internal VOD content, Ali266 at fast preset achieves BD rates for {Y, U, V, YUV} of {-36.25%, -45.73%, -43.02%, -38.10%} over the widely used open source HEVC software encoder x265 at medium preset, which is better than the result that was previously reported as {-32.87%, -46.07%, -42.70%, -35.35%} in JVET-W0127 over the same anchor.

Introduction

In JVET-W0127, we presented Ali266, a software VVC encoder implementation developed by Alibaba aimed at achieving real-time encoding speed, and claimed that Ali266 was capable of real-time encoding for 720p sequences. To accelerate VVC’s adoption by a wider range of video applications, we have taken more efforts to improve Ali266’s encoding speed. Under the configuration of fast preset with 8-bit internal bit depth, Ali266 has achieved real-time encoding speed for 1080p sequences.

The main differences from JVET-W0127 include:

- More fast encoding algorithms suitable for VVC encoder were employed.

- More extensive and adequate SIMD instructions were used to reduce the required clock cycles, especially for the 8-bit internal bit depth.

- Redesigned data structure to improve memory access efficiency and overcome performance bottlenecks.

- Less memory consumption.

- Better support of parallel encoding to reduce CPU idle time.

- More rate control mode such as Constant QP (CQP), Constant Rate Factor (CRF), and Average Bitrate (ABR).

Both 8-bit encoding and 10-bit encoding are supported in Ali266. Bitstreams encoded by Ali266 conform to VVC main10 profile and can be decoded by VTM-11.0 reference software decoder.

Simulation results

Performance and speed test: JVET CTC content

The encoding performance of Ali266 at slow preset focuses on class A1, A2, B, and C with random access encoding as defined in the JVET common test conditions (CTC) [2]. The anchor is VTM-13.0. YUV-PSNR is a weighted average with weighting factors of 6, 1 and 1 for the Y-PSNR, U-PSNR and V-PSNR, respectively.

The PSNR BD-rates of Ali266 over VTM-13.0 is shown in Table 1. Here positive number represents bit-rate increase at the same PSNR with respect to the VTM-13.0 anchor, while negative number represents bit-rate saving.

As a good trade-off between encoder runtime and PSNR BD-rate performance, it could be seen that with the slow preset, the BD-rate loss over VTM-13.0 is only 7.11%, with an average speedup of more than 227x.

Table 1: Encoding performance of the Ali266 encoder @ slow preset

| Class | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to VTM-13.0 |  |  |  | Speedup |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs. VTM |
| A1 | 12.82 | -2.81 | 1.88 | 8.55 | 317 |
| A2 | 15.34 | -4.10 | -2.08 | 11.08 | 288 |
| B | 9.07 | -8.56 | -8.64 | 5.18 | 239 |
| C | 8.60 | -7.02 | -5.93 | 5.47 | 138 |
| Overall | 10.95 | -6.11 | -4.50 | 7.11 | 227 |

Performance and speed test: internal VOD content

The encoding performance of Ali266 at fast preset focuses on 720p and 1080p sequences in both landscape and portrait modes. These two kinds of resolution are widely used for the real-world VOD content in our cloud computing business. We selected the widely-used open source HEVC software encoder x265 at the medium preset as anchor.

With 8-bit internal bit depth, The PSNR BD-rates of Ali266 fast preset over x265 at medium preset is shown in Table 2. With the fast preset, the BD-rate gain over x265 is approx. 38.10%, which is better than the result that was previously reported as 35.35% in JVET-W0127 over the same anchor. Compared with the previous version, an average of 1.9x speedup has been made in the past three months for classes 1920x1080 / 1080x1920. Currently, Ali266 achieves average encoding speeds of 39.13 and 33.46 fps for classes 1920x1080 and 1080x1920, and 64.52 and 62.23 fps for classes 1280x720 and 720x1280, respectively. Especially, the average encoding speed reaches 36.3 fps for 1080p sequences. Therefore, it is our conclusion that software VVC encoder is capable of real-time encoding for 1080p sequences, and is ready for real-world commercial applications.

Table 2: Encoding performance of the Ali266 encoder @ fast preset

| Class | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to x265 @ medium |  |  |  | Speed (fps) |  |
|  | Y (%) | U (%) | V (%) | YUV (%) | Previous version | Current version |
| 1920x1080 | -37.73% | -51.31% | -47.93% | -40.58% | 19.99 | 39.13 |
| 1080x1920 | -36.39% | -41.04% | -36.75% | -36.86% | 18.61 | 33.46 |
| 1280x720 | -38.35% | -46.63% | -45.00% | -40.27% | 38.77 | 64.52 |
| 720x1280 | -32.54% | -43.93% | -42.39% | -34.68% | 36.66 | 62.23 |
| Overall | -36.25% | -45.73% | -43.02% | -38.10% | -- | -- |

Conclusion

This contribution presents the progress of Alibaba’s optimized VVC encoder implementation, Ali266. It shows Ali266’s encoding performance and speed for the JVET CTC content and internal VOD content. With the slow preset of Ali266, the average YUV-PSNR-based BD-rate over VTM-13.0 is 7.11%, and the average speedup factor achieves 227 times over VTM. With the fast preset, the average YUV-PSNR-based BD-rate over x265 at the medium preset is -38.10%, and the average encoding speed of 1080p sequences achieves 36.3 fps. The simulation results demonstrate that Ali266 is capable of real-time encoding for 1080p sequences, and is ready for real-world applications. In addition to long-term efforts to improve encoding speed and performance, we are also committed to applying Ali266 to Alibaba’s video business.

References

- S. Fang el al., “Ali266: an optimized VVC software encoder implementation”, JVET-W0127, Joint Video Experts Team (JVET), July 2021.

- F. Bossen el al., “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Joint Video Experts Team (JVET), October 2020.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).