| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0105-v3 |
| --- | --- |

| Title: | AHG12: Edge classifier for Cross-component Sample Adaptive Offset (CCSAO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Nan Hu Vadim Seregin Marta Karczewicz C.-W. Kuo, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, W. Chen, Ning Yan, Xianglin Wang | Tel: Email: | akotra@qti.qualcomm.com cheweikuo@kwai.com |
| Source: | Qualcomm Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

The cross-component sample adaptive offset (CCSAO) in ECM-2.0 [2] only uses the band classifier to classify a given sample. This contribution extends the CCSAO design by adding the edge-based classifier.

The objective results, over ECM-2.0 Anchor for CTC configuration are as follows:

When the edge-based classifier is applied only to “Chroma” components:

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.06% | -1.14% | -0.88% | 101% | 102% |
| RA | 0.04% | -0.88% | -0.67% | 99% | 99% |
| LDB | -0.02% | -0.93% | -0.53% | 98% | 98% |

When the edge-based classifier is applied only to both “Luma” and “Chroma” components:

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.01% | -1.03% | -0.82% | 101% | 103% |
| RA | -0.14% | -0.73% | -0.48% | 100% | 100% |
| LDB | -0.39% | -0.64% | -0.06% | 101% | 101% |

# Introduction

In the VVC standard [1], Sample Adaptive Offset (SAO) filter uses either the edge or band classifier during sample classification. Cross-component Sample Adaptive offset (CCSAO) in ECM-2.0 [2] leverages the correlation between the different components to improve the reconstructed quality of the signal. However, the CCSAO in ECM-2.0 only uses the band classifier for sample classification. 
To achieve a better complexity/performance trade-off for the whole CCSAO filter, this proposal extends the CCSAO design by also using the edge-based classifier during sample classification.

# Proposal

In the proposed design, the edge-based classifier is applied to both the Luma and Chroma components, Moreover, the main difference between CCSAO edge-based classifier and the Chroma SAO edge classifier in VVC [1] is that, in the former, Chroma samples use the co-located Luma samples for deriving the edge information whereas in the later Chroma samples use its own neighboring samples for deriving the edge information.

Like the edge classifier of SAO, the proposed edge-based classifier of CCSAO also uses the four 1-D directional patterns for sample classification: horizontal, vertical, 135° diagonal and 45° diagonal, as shown in the figure below.

Figure1. Four 1-D directional patterns for CCSAO EO sample classification: horizontal (EO class = 0), vertical (EO class =1), 135° diagonal and 45° diagonal

For every 1-D pattern, each sample is classified based on the sample difference between the luma sample value labeled as “c” and its two neighbor luma samples labeled as “a” and “b” along the selected 1-D pattern.

Similar to SAO, the encoder may decide the best 1-D directional pattern using the rate-distortion optimization and signal this additional information in each classifier/set. Both the sample differences “a-c” and “b-c” are compared against a pre-defined threshold value (Th) to derive the final “class_idx” information.

Ea=(a-c<0)?(a-c<(-Th)?0:1):(a-c<(Th)?2:3) 				– (1)

Eb=(b-c<0)?(b-c<(-Th)?0:1):(b-c<(Th)?2:3) 				– (2)

class_idx = band * 16 + pa * 4 + pb 					– (3)

variable “band” in equation 3 is derived based on the sample value of either Luma or Chroma sample

# Simulation results

The proposed edge classifier for CCSAO is implemented on top of ECM-2.0 [2] and the simulation is performed under common test conditions as described in JVET-V2017 [4]. SIMD optimizations were not used in the implementations. The BD-Rate impact of applying the “Edge-based” classifier only to the Chroma components is summarized in Table 3.1 below. The BD-Rate impact of applying the “Edge-based” classifier both to the Luma and Chroma components is summarized in Table 3.2 below

## Edge-based classifier applied only to Chroma components:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -1.25% | -0.87% | 102% | 101% |
| Class A2 | 0.05% | -1.29% | -4.66% | 99% | 102% |
| Class B | 0.08% | -2.25% | -0.24% | 101% | 103% |
| Class C | 0.04% | 0.28% | 0.20% | 100% | 101% |
| Class E | 0.04% | -0.93% | 0.37% | 101% | 102% |
| Overall | 0.06% | -1.14% | -0.88% | 101% | 102% |
| Class D | 0.01% | -0.04% | 0.01% | 100% | 100% |
| Class F | 0.04% | -1.29% | -0.78% | 100% | 102% |
| Class TGM | 0.05% | -1.77% | -1.54% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.39% | -0.04% | 100% | 99% |
| Class A2 | 0.03% | -1.23% | -3.16% | 97% | 99% |
| Class B | 0.06% | -1.62% | -0.14% | 99% | 99% |
| Class C | 0.03% | -0.07% | 0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.88% | -0.67% | 99% | 99% |
| Class D | 0.01% | -0.14% | -0.07% | 100% | 100% |
| Class F | 0.01% | -0.65% | -0.50% | 102% | 101% |
| Class TGM | 0.00% | -0.82% | -0.63% | 101% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -1.29% | -0.61% | 101% | 101% |
| Class C | -0.06% | -0.24% | 0.06% | 95% | 96% |
| Class E | -0.06% | -1.23% | -1.18% | 98% | 97% |
| Overall | -0.02% | -0.93% | -0.53% | 98% | 98% |
| Class D | 0.05% | -0.32% | 0.19% | 98% | 99% |
| Class F | -0.08% | -0.53% | -0.59% | 98% | 97% |
| Class TGM | 0.07% | -0.65% | -0.46% | 100% | 99% |

## Edge-based classifier applied to both Luma and Chroma components:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -1.14% | -0.83% | 102% | 103% |
| Class A2 | -0.05% | -1.26% | -4.66% | 99% | 106% |
| Class B | 0.02% | -2.13% | -0.13% | 101% | 103% |
| Class C | 0.02% | 0.42% | 0.30% | 100% | 100% |
| Class E | -0.07% | -0.77% | 0.39% | 102% | 102% |
| Overall | -0.01% | -1.03% | -0.82% | 101% | 103% |
| Class D | 0.01% | -0.04% | 0.01% | 101% | 101% |
| Class F | -0.12% | -1.06% | -0.60% | 101% | 103% |
| Class TGM | -0.29% | -1.70% | -1.45% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.40% | 0.16% | 101% | 101% |
| Class A2 | -0.21% | -1.19% | -3.03% | 101% | 100% |
| Class B | -0.24% | -1.36% | 0.06% | 101% | 100% |
| Class C | -0.03% | 0.15% | 0.28% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.73% | -0.48% | 100% | 100% |
| Class D | -0.01% | -0.13% | -0.03% | 99% | 97% |
| Class F | -0.19% | -0.30% | -0.19% | 102% | 100% |
| Class TGM | -0.44% | -0.62% | -0.59% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -1.24% | -0.48% | 101% | 102% |
| Class C | -0.19% | 0.05% | 0.33% | 100% | 101% |
| Class E | -0.42% | -0.55% | 0.14% | 104% | 100% |
| Overall | -0.39% | -0.64% | -0.06% | 101% | 101% |
| Class D | -0.07% | -0.99% | -0.12% | 100% | 99% |
| Class F | -0.52% | -1.14% | -0.84% | 101% | 100% |
| Class TGM | -0.74% | -0.66% | -0.52% | 100% | 100% |

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, Jun. 2020, Teleconference

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “EE2-5.1: Cross-component Sample Adaptive offset”, JVET-W0066, July 2021, Teleconference

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, May 2021, Teleconference.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).