| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0109-v1 |
| --- | --- |

| Title: | On maximum bit-rates for high bit-depth profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Steve Keating Adrian Browne Karl Sharman | Email: | steve.keating@sony.com adrian.browne@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution compares the maximum specified bit-rates at certain levels of VVC with the bit-rates and PSNRs of VTM14.0 for appropriate CTCs.  Comparison is also made with HEVC/HM bit-rates.

# Introduction

The maximum bit-rate for VVC is a function of level, tier, CpbVclFactor and the HBR factor of the profile.  The accompanying spreadsheets plot PSNR vs. bit-rate for various CTCs compared with the maximum specified bit-rates for main and high tiers at the appropriate level. Some plots from these are included in this document for illustration, figures are for Random Access unless otherwise stated.

# Plots

## Main 10 Profile

The plot below shows PSNR vs. bit-rate for the JVET-M1010 CTC along with the specified maximum bit-rate for level 4.1 (2k @ 60fps)* and level 5.1 (4k @ 60fps).  As can be seen, all the 2k content is within the main tier’s maximum bit-rate and most of the 4k content is within the main tier’s bit-rate although some requires the high tier’s bit-rate for this CTC.

* note that the Campfire sequence is only 30fps so level 4 could be used for this.

## Main 12 Profile

The plot below shows PSNR vs. bit-rate for the JVET-U2018-420LOWQP CTC (QP=17..QP=2) along with the specified maximum bit-rate for level 4.1 (2k @ 60fps)*. As can be seen, much of the content requires bit-rates well beyond high tier, especially the HLG content.  If this level is used, the maximum PSNR can be below 40dB for some content.

*note that some PQ sequences could use level 4 due to their lower frame rate

If the CpbVclFactor is increased as proposed in JVET-X0108 the maximum bit-rate is as shown below:

This still only allows 40dB PSNR for the most difficult HLG content and approximately 50dB PSNR for the most difficult PQ content at high tier.

## Main 12 4:4:4 Profile

The plot below shows PSNR vs. bit-rate for the JVET-U2018-MIDQP CTC (QP=32..QP=7) along with the specified maximum bit-rate for level 4.1 (2k @ 60fps). As can be seen, much of the HLG and SVT content requires bit-rates well beyond high tier, although most of the PQ content can be encoded within high tier maximum bit-rates for level 4.1.

If the CpbVclFactor is increased as proposed in JVET-X0108 the maximum bit-rate is as shown below:

If this level is used, the maximum PSNR is between 45 and 50dB for the HLG content but below 35dB for all the SVT content.

## High Throughput Profiles

High throughput profiles have been discussed but not yet formally proposed for VVC, it may therefore be instructive to examine the high throughput profiles in HEVC.  The plots below show PSNR vs. bit-rate for HM with the JVET-U2018-LOWQP CTC along with the specified maximum bit-rate for level 4.1 (2k @ 60fps).  These figures are for All Intra.

# Conclusion

The maximum specified bit-rate for 12-bit profiles can limit the achievable PSNR to sometimes as low as 40dB even in high tier.  Although a higher level could be used to allow higher picture quality this might require, e.g., a 4k or 8k capable decoder for a 2k bit-stream.  Consideration could be given to increasing either the HBR factor or the high tier maximum bit-rate in order to achieve higher image quality within the existing levels.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “VVC operation range extensions (Draft 4)” JVET-W2005-v1, F Bossen, B Bross, T Ikai, D Rusanovskyy, G Sullivan, Y-K Wang

- “ ITU-T Rec. H.265v8 High Efficiency Video Coding (08/2021)”