| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0111_v3 |
| --- | --- |

| Title: | [AHG11: MPEG NNR compressed bias update for the CNN based post-filter of EE1-1.1] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Alireza Zare, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela. | Email: | {maria.santamaria_gomez.ext, jani.lainema, francesco.cricri, ramin.ghaznavi-youvalari, honglei.1.zhang, alireza.zare, goutham.rangu, hamed.rezazadegan_tavakoli, homayun.afrabandpey, miska.hannuksela}@nokia.com |
| Source: | [Nokia Technologies] |  |  |

_____________________________

# Abstract

This contribution presents a Convolutional Neural Network (CNN) based content-adaptive post-processing filter. The filter is content-adaptive in the sense that it is trained offline on general video sequences, and later fine-tuned on the test video sequence. The work is based on the filter described in JVET-X0110, where only the bias terms of the CNN are fine-tuned resulting in a weight-update that is compressed with Lempel-Ziv-Markov chain algorithm 2 (LZMA2). In this study the weight-update is switched to use the MPEG Neural Network compression and Representation (NNR) standard. The weight-update was compressed with NNR’s reference model, the Neural network Compression Test Model (NCTM). The proposed technique was implemented on top of the EE1 anchor software (VVC Test Model (VTM) 11.0 with Neural Network for Video Coding (NNVC) technology 1.0). It is reported that compressing the weight-update by using NCTM achieves overall BD-rate gains in Random Access (RA) configuration of -2.64% (Y), -5.94% (U), -3.97% (V). Furthermore, when compared against the weight-update compression with LZMA2 in RA, the average gains are -0.13% (Y), -0.08% (U), -0.10% (V).

# Introduction

This contribution studies the CNN content-adaptive post-processing filter in JVET-X0110 [1]. The same fine-tuned CNNs are used, but in this case the content-adaptive component (bias term weight-update) is compressed with NNR [2].

The architecture of the CNN is illustrated in Figure 1. The inputs are the YUV channels and an additional channel that contains the normalised frame Quantisation Parameter (QP). The first block comprises a convolutional layer (excluding the biases), a bias layer and a LeakyReLU as the non-linear activation function. There are four following blocks with similar layers as in the first block, but where the input of each block is added to the block’s output (by using a skip connection). The convolutional layers in the first block and in the following four blocks use 64 kernels. The last block is similar to the first block (no skip connection), but with 3 convolutional kernels. The input YUV channels are summed to the output of the NN.

The CNN contains 151188 parameters and the computational complexity in MACs per pixel is 151488.

Figure 1. CNN architecture.

The CNN is pre-trained offline using a subset of training video sequences available for the JVET Common Test Conditions (CTC) for NNVC technology [3]. The BVI-DVC dataset [4] was used for classes A, B, C and D, while screen content video sequences were used for class F.

The video sequences were encoded with VTM 11.0 with NNVC 1.0, QPs 22, 27, 32, 37 and 42, and the Random Access (RA) configuration. For each epoch, all the frames from each video sequence are selected once randomly. From each frame in these video sequences, a 128x128 patch from the reconstructed sequence was used as the input to the CNN filter, and the corresponding uncompressed patch was used as the ground-truth. The patches were cropped by choosing their position randomly within the frames. Whenever necessary, padding was performed by using boundary pixels. The normalised frame-QP was used as an additional input channel, stacked to the Y, U, V channels. The chroma channels were up-sampled by duplicating the neighbouring rows and columns.

Table I shows the network information in pre-training stage.

Table I. Network information in pre-training stage

| Network Information in Pre-Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 300 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: | Class A: ~ 89 hours Class B: ~75 hours Class C: ~63 hours Class D: ~25 hours Class F: ~54 hours |
|  | Training data information: | BVI-DVC and screen content sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in JVET CTC NNVC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

The bias terms of the pre-trained CNNs are fine-tuned for each pair of [sequence, sequence QP]. Once the fine-tuning is done, the weight-update is calculated as the difference between the fine-tuned bias terms and the pre-trained bias terms. Finally, the weight-update is compressed with NNR’s [2] NCTM [5] using uniform quantisation, QP -40, QP density 2, CABAC unary length 10 and CABAC optimisation. The bitrate of the weight-update is considered in the bitrate and in the BD-rate computation.

Table II shows the network information in fine-tuning stage.

Table II. Network information in fine-tuning stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: | Class A: ~19.2 hours per video sequence Class B: ~5.5 hours per video sequence Class C: ~1.3 hours per video sequence Class D: ~0.4 hours per video sequence Class F: ~2.7 hours per video sequence |
|  | Training data information: | JVET-CTC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in JVET CTC NNVC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

# Experimental results

Table III displays the network information in inference stage.

Table III. Network information in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task |  |
|  | Total Parameter Number | 151811 |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0,579113007 (per NN) 2,895565035 (all NNs) |
|  | Multiply Accumulate (MAC) | 151488 |
| Optional |  |  |
|  | Total Conv. Layers | 6 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 8 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points | 1 NN for each [sequence, QP] pair |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Padding with boundary pixels when necessary |
|  | Other information: |  |

Table IV reports the BD-rates for the proposed technique, where NCTM is used for compressing the weight-update.

Table IV. Coding performance of proposed technique over VTM 11.0 with NNVC 1.0

Table V reports the BD-rates for JVET-X0110, where LZMA2 is used to compress the weight-update.

Table V. Coding performance of JVET-X0110 over VTM 11.0 with NNVC 1.0

Table VI reports the BD-rates for the proposed technique over JVET-X0110.

Table VI. Coding performance of proposed approach over JVET-X0110

The reported results state that using NCTM to compress the weight-update improves the content-adaptive post-processing filter performance.

# References

- M. Santamaria, J. Lainema, F. Cricri, R. G. Youvalari, H. Zhang, Alireza Zare, H. R. Tavakoli, M. Hannuksela. EE1-1.1: Content-adaptive neural network post-processing filter, JVET-X0110. October 2021.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao. Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-W2016. July 2021.

- Zhang, F. (Creator), Ma, D. (Creator), Bull, D. (Creator) (31 Mar 2020). BVI-DVC. University of Bristol. 10.5523/bris.3hj4t64fkbrgn2ghwp9en4vhtn.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis, N0128. July 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).