| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0112-v1 |
| --- | --- |

| Title: | AHG9: On post-filter SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, Hamed. R. Tavakoli Maria Santamaria Nokia Technologies Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following SEI messages into a working draft of a VSEI amendment:

- An NNR post-filter SEI message for the carriage of an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream that specifies a neural-network-based post-filter.

- A post-filter purpose SEI message that currently specifies quality enhancement and super resolution as purposes and allows other purposes to be specified in the future.

Additionally, this contribution proposes constraints to be included in VVC regarding the use of the proposed SEI messages in VVC bitstreams.

# Introduction

## Related contributions

JVET-T0076 [1] proposed enabling the usage of ALF as a post-filter and presented additional sharpening and bokeh effect as use cases. The contribution pointed out that ALF post-filtering enables selective region-wise sharpening or blurring.

JVET-T0092 [2] proposed neural network (NN) based super resolution post-filtering and presented a concept to convey the NN parameters in an SEI message. MPEG NNR, ONNX (Open Neural Network eXchange), and NNEF (Neural Network Exchange Format) were mentioned as potential formats for the NN parameters in the SEI message.

JVET-U0091 [3] presented syntax and semantics for a neural-network-based post-filtering SEI message, which included the following aspects:

- Purpose for the post-filtering

| nn_postfilter_type_idc | Post filtering type |
| --- | --- |
| 0 | Visual quality improvement with single input picture |
| 1 | Visual quality improvement with multiple input pictures |
| 2 | Super resolution with single input picture |
| 3 | Super resolution with multiple input pictures |
| 4..15 | Reserved |

- Neural network topology and parameter storage formats, e.g. NNEF or ONNX

- Either a URI identifying the neural network topology or a coded representation of the topology

- Either a URI identifying the neural network parameter values or a coded representation of the parameter values

JVET-V0058 [4] proposed a post-filter nesting SEI message, which contains inter-related SEI messages jointly specifying a post-filtering operation. The contribution envisioned that the nested SEI messages could include the following:

- An SEI message specifying the purpose for filtering, such as quality improvement or super-resolution, and parameters specific to the purpose, such as an applicable scaling ratio range for a super-resolution filter.

- An SEI message specifying the filter itself. Such an SEI message could for example contain or reference (through a URI) an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream.

JVET-V0058 was discussed in a joint meeting during which the following guidance was noted (quote from the meeting minutes):

It was suggested to continue with the intent to not significantly extend existing SEI messages (at least if there was not a clear plan to do so that was established in advance, and that the existence of a reserved value is not sufficient to indicate such a plan). A NOTE in the text to describe future plans could be an approach to consider for new SEI messages.

JVET-W0063 [5] proposed a NN-based super-resolution post-filter. Two sets of results were provided, where test 1 used constant neural network parameters and test 2 used content-adaptive neural network parameters per each sequence and QP that were coded with the NCTM of MPEG NNR. Test 1 was shown to provide an average BD-rate reduction over the use of VVC RPR for upsampling. Test 2 was shown to provide a consistent BD-rate reduction compared to test 1. An SEI message or systems layer signaling was proposed to convey the network parameters to the decoder.

JVET-X0111-v1 [6] proposes a content-adaptive NN-based quality enhancement post-filter where the NN parameters are coded with MPEG NNR. The results are compared with the same NN parameters coded with Lempel-Ziv-Markov chain algorithm 2 (LZMA2). A consistent BD-rate reduction with the MPEG NNR coding compared to LZMA2 coding is reported in all tested classes (B, C, D, F).

## Observations

We have the following observations based on the related contributions:

- Post-filtering can be performed for multiple purposes, including but not limited to those mentioned in JVET-U0091 and JVET-T0076.

- Different types of post-filters (e.g. ALF or a neural-network-based filter) can be used for the same purpose, such as generic visual quality improvement.

- Content-adaptive NN-based post-processing for quality enhancement and super-resolution has been shown to provide a BD-rate reduction compared to pre-trained NN-based post-processing with the same neural network topology.

- Coding neural network parameters with MPEG NNR has been shown to be a viable approach for content-adaptive NN-based post-processing for quality enhancement and super-resolution.

# Proposal

## Overview

This contribution proposes the following SEI messages into a working draft of a VSEI amendment:

- An NNR post-filter SEI message for the carriage of an MPEG NNR bitstream that specifies a neural-network-based post-filter.

- A post-filter purpose SEI message that currently specifies quality enhancement and super resolution as purposes and allows other purposes to be specified in the future.

Additionally, this contribution proposes constraints to be included in VVC regarding the use of the proposed SEI messages in VVC bitstreams.

## NNR post-filter SEI message

| nnr_post_filter( payloadSize ) { | Descriptor |
| --- | --- |
| nnrpf_id | u(21) |
| nnrpf_mode_idc | u(2) |
| if( nnrpf_mode_idc > 0 ) |  |
| nnrpf_persistence_flag | u(1) |
| if( nnrpf_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do { |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ¦ ¦ nnrpf_mode_idc = = 3 ) { |  |
| for( i = 0; i < payloadSize − 3 ; i++ ) |  |
| nnrpf_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnrpf_id).

nnrpf_id contains an identifying number that may be used to identify a post-processing filter.

Values of nnrpf_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnrpf_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

nnrpf_mode_idc equal to 0 indicates that no post-processing filter associated with the nnrpf_id value is specified in this SEI message. nnrpf_mode_idc equal to 1 specifies that the post-processing filter associated with the nnrpf_id value is a neural network identified by a given Uniform Resource Identifier (URI). nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnrpf_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message. nnrpf_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnrpf_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnrpf_persistence_flag specifies the persistence of this SEI message for the current layer.

nnrpf_persistence_flag equal to 0 specifies that the this SEI message applies to the current decoded picture only.

nnrpf_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnrpf_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and nnrpf_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnrpf_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and any nnrpf_mode_idc value is output.

The neural network that may be used as a post-processing filter for a cropped decoded output picture is the filter that i) is identified by the NNR post-filter SEI message that has a particular nnrpf_id value, has nnrpf_mode_idc equal to 1 or 2, and pertains to the picture and ii) is updated by the NNR post-filter SEI message, if any, that has the same nnrpf_id value, has nnrpf_mode_idc equal to 3, and pertains to the picture.

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string that contains a URI identifying the neural network used as the post-processing filter.

nnrpf_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnrpf_payload_byte[ i ] for all values of i in the range of 0 to payloadSize − 3, inclusive, shall be a complete bitstream that conforms to ISO/IEC 15938-17.

## Post-filter purpose SEI message

| post_filter_purpose( payloadSize ) { | Descriptor |
| --- | --- |
| pfp_id | u(21) |
| pfp_purpose | ue(v) |
| if( pfp_purpose = = 1 ) { /* super resolution */ |  |
| pfp_pic_width_in_luma_samples | ue(v) |
| pfp_pic_height_in_luma_samples | ue(v) |
| } |  |
| } |  |

This SEI message may be used for indicating a purpose for the identified post-processing filter. In this version of this Specification, the identified post-processing filter is specified by the NNR post-filter SEI message that has nnrpf_id equal to the value of pfp_id.

NOTE 1 – In a future version of this Specification, other mechanisms, such as another post-filter SEI message, could be specified. The value of pfp_id could be used as an identification number for the post-processing filter specified through such other mechanism.

This SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

pfp_id contains an identifying number used to identify a post-processing filter.

Values of pfp_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of pfp_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

pfp_purpose indicates the purpose of post-processing filter identified by pfp_id as specified in Table XYZ. The value of pfp_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of pfp_purpose that do not appear in Table XYZ are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore post-filter purpose SEI messages that contain reserved values of pfp_purpose.

Table XYZ – Definition of pfp_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Visual quality improvement |
| 1 | Increase the width or height of cropped decoded output picture |

pfp_pic_width_in_luma_samples and pfp_pic_height_in_luma_samples, when present, specify the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by pfp_id to a cropped decoded output picture.

NOTE 2 – When a reserved value of pfp_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by pfp_purpose being equal to that value.

## Use of NNR post-filter and post-filter purpose SEI messages (subclause D.9.x of VVC)

The variables croppedPicWidthInLumaSamples and croppedPicHeightInLumaSamples are derived for the referenced PPS as follows:

croppedPicWidthInLumaSamples = 	pps_pic_width_in_luma_samples − pps_conf_win_left_offset − pps_conf_win_right_offset	(1)

croppedPicHeightInLumaSamples = 	pps_pic_height_in_luma_samples − pps_conf_win_top_offset − pps_conf_win_bottom_offset	(2)

When pfp_pic_width_in_luma_samples and pfp_pic_height_in_luma_samples are present in a post-filter purpose SEI message, all of the following constraints apply:

–	pfp_pic_width_in_luma_samples shall be greater than or equal to croppedPicWidthInLumaSamples.

–	pfp_pic_height_in_luma_samples shall be greater than or equal to croppedPicHeightInLumaSamples.

–	At least one of the following constraints apply:

–	pfp_pic_width_in_luma_samples shall be greater than croppedPicWidthInLumaSamples.

–	pfp_pic_height_in_luma_samples shall be greater than croppedPicHeightInLumaSamples.

When a post-filter purpose SEI message and an NNR post-filter SEI message with nnrpf_id equal to pfp_id are present in the same picture unit, they shall be present in the same SEI NAL unit.

# References

- Han Boon Teo, Hai Wei Sun, Chong Soon Lim, "AHG9: ALF control SEI message", JVET-T0076, Oct. 2020.

- Takeshi Chujoh, Eiich Sasaki, and Tomohiro Ikai, "AHG9/AHG11: Neural network based super resolution SEI", JVET-T0092, Oct. 2020.

- Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", JVET-U0091, Jan. 2021.

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, and Hamed R. Tavakoli, "AHG9: On post-filter SEI", JVET-V0058, Apr. 2021.

- Takeshi Chujoh, Yukinobu Yasugi, and Tomohiro Ikai, "EE1.2.2: NN-based super resolution (JVET-V0073)", JVET-W0063, Jul. 2021.

- Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Alireza Zare, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela, "AHG11: MPEG NNR compressed bias update for the CNN based post-filter of EE1-1.1", JVET-X0111, Oct. 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).