| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0116-v1 |
| --- | --- |

| Title: | AHG10: Suggestion to enable GOP-based temporal filtering for low-delay configurations in CTC for HM and VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, P. Wennersten | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

GOP-based temporal filtering / Motion Compensated Temporal Filtering (MCTF) is enabled in CTC for random-access for HM and VTM. For low-delay MCTF has so far been disabled due to bad performance for smaller resolutions. In this contribution we show that by enabling MCTF for low-delay configurations in HM-16.24rc1 and VTM-14.0 a BD-rate impact of -0.7% and -0.8% can be achieved for HM respectively VTM for low-delay B and -1.1% for low-delay P. We have also checked visually that it works well.

We therefore suggest to also enable temporal filtering for low-delay configurations in CTC for HM and VTM.

# Introduction

MCTF was firstly introduced in VTM/HM using two pictures before and two pictures after the current picture for every eight picture in random access configuration [1]. It was later further improved in [2] and then also using four pictures before and four pictures after the current picture in random access configuration. For low-delay configuration it has been present in reference software to only use pictures before the current picture but it has been disabled in CTC because of bad performance for small resolutions, e.g. Class C and Class D. In this contribution we present results for the improved MCTF which is part of VTM-14.0 and HM-16.24rc1 but here with more conservative setting.

# Proposal

In this contribution we suggest enabling MCTF in low-delay configurations by default and use the following settings:

HM:

TemporalFilter                             : 1                # Enable/disable GOP Based Temporal Filter

TemporalFilterFutureReference  : 0                # Enable/disable reading future frames

TemporalFilterStrengthFrame8   : 0.1             # Enable filter at every 8th frame with strength

VTM:

TemporalFilter                             : 1                # Enable/disable GOP Based Temporal Filter

TemporalFilterFutureReference  : 0                # Enable/disable reading future frames

TemporalFilterStrengthFrame8   : 0.2             # Enable filter at every 8th frame with strength

Compared to the current settings in HM and VTM the filtering strength have been reduced to be more conservative. The strength for HM is set lower than for VTM to have a similar effect. The application of the filter is also aligned with the GOP structure for low-delay configurations.

# Results

We provide results for both HM and VTM for low-delay test conditions [5][6]. MCTF when enabled for low-delay configurations can provide a significant benefit for both HM and VTM. Anchor and test are not run on the same machine, but it is expected that the encoding and decoding time is similar or lower than for the anchor because bitstream size is reduced by MCTF.

Table 1: MCTF with proposed settings on VTM-14.0 [3] for low-delay B:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,14% | -1,09% | -1,38% | 100% | 108% |
| Class C | -0,10% | -0,08% | 0,05% | 97% | 95% |
| Class E | -1,06% | -1,29% | -1,12% | 98% | 96% |
| Overall | -0,77% | -0,80% | -0,84% | 99% | 100% |
| Class D | 0,07% | 0,10% | -0,61% | 100% | 94% |

Table 2: MCTF with proposed settings on VTM-14.0 [3] for low-delay P:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,68% | -1,46% | -1,76% | 97% | 106% |
| Class C | -0,19% | -0,30% | 0,15% | 101% | 108% |
| Class E | -1,29% | -2,00% | -2,60% | 103% | 115% |
| Overall | -1,09% | -1,21% | -1,33% | 100% | 109% |
| Class D | -0,02% | -0,23% | -0,40% | 104% | 105% |

Table 3: MCTF with proposed settings on HM-16.24rc1 [4] for low-delay B:

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -1,18% | -0,73% | -0,94% |
| Class C | -0,20% | -0,15% | -0,03% |
| Class E | -0,68% | -0,82% | -0,79% |
| Overall | -0,73% | -0,56% | -0,60% |
| Class D | 0,12% | 0,01% | -0,07% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4: MCTF with proposed settings on HM-16.24rc1 [4] for low-delay P:

|  | Low delay P Main10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -1,87% | -1,50% | -1,48% |
| Class C | -0,20% | -0,05% | -0,02% |
| Class E | -1,20% | -1,24% | -1,33% |
| Overall | -1,15% | -0,95% | -0,95% |
| Class D | -0,03% | -0,39% | -0,15% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 115% |  |  |

# Conclusions

In this contribution we present results that shows that MCTF can be helpful to reduce bitrate in low-delay configurations for both HM and VTM.

We therefore suggest enabling MCTF with proposed settings also in low-delay configurations in CTC for HM and VTM.

# References

[1] P. Wennersten, R. Sjöberg, J. Enhorn, “AHG10: Encoder-only GOP-based temporal filter,” Joint Video Experts Team, doc. JVET-O0549, version 2, Gothenburg, July 2019.

[2] P. Wennersten, C. Hollmann, J. Ström, AHG10: GOP-based temporal filter improvements. JVET-V0056, April 2021

[3] VTM-14.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[4] HM-16.24rc1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/HM.git

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[6] K. Sühring, K. Sharman, “Common Test Conditions for HM Video Coding Experiments”, JVET-U1100, Meeting by Teleconference, April 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).