| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0118 |
| --- | --- |

| Title: | EE1-3.1: BD-rate gains vs complexity of NN-based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T.Dumas, F.Galpin P.Bordes F. Leléannec | Tel: Email: | thierry.dumas@interdigital.com franck.galpin@interdigital.com philippe.bordes@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the EE1 results for the contribution JVET-W0081 presented during the 23rd JVET meeting.

Mean BD-rate reductions of -2.87% -2.32% -2.37% and -1.38% -0.55% -0.82% in AI and RA configurations respectively are reported when comparing VTM-11.0-NNVC including the low complexity version of the neural network-based intra prediction mode with respect to VTM-11.0-NNVC. The complexity (in MAC/pixel) of the low complexity version of the neural network-based intra prediction mode is about 12 times smaller than its regular version.

# Introduction

Matrix-based Intra Prediction (MIP) [1] was adopted in VVC following the 14th JVET meeting. Several minor refinements to MIP have then been proposed [2]. MIP consists of multiple learning-based intra prediction modes whose parameters are learned off-line, then fixed on both encoder and decoder sides. -0.63% -0.17% -0.18% of mean BD-rate reduction in AI configuration and -0.33% -0.35% -0.37% of mean BD-rate reduction in RA configuration are reported when comparing VTM-10.0 with MIP to VTM-10.0 without MIP [3]. When predicting a given block from its decoded reference samples, MIP requires only four multiplications per sample.

During the 22nd JVET meeting, a single neural network-based intra prediction mode inside VVC was presented [4]. Like MIP, its parameters are learned off-line, then fixed on both encoder and decoder sides. -3.47% -3.24% -3.25% of mean BD-rate reduction in AI configuration and -1.70% -0.62% -0.96% of mean BD-rate reduction in RA configuration are observed when comparing VTM-11.0 with this single neural network-based mode to VTM-11.0. The complexity of the single neural network-based mode can be estimated at 100 kMAC/pixel.

Given the gap in complexity between MIP and the single neural network-based mode, this contribution suggests a low-complexity version of the single neural network-based mode.

# Composition of the neural network-based intra prediction mode

Before presenting the contribution, this section recalls the basic characteristics of the single neural network-based intra prediction mode. The single neural network-based mode contains 8 neural networks, each predicting blocks of a different size in S=4×4, 8×4, 16×4, 32×4, 8×8, 16×8, 16×16, 32×32. The neural network predicting w×h blocks, denoted fh, w . , θh,w, is parametrized by θh,w.

Furthermore, the single neural network-based mode can predict blocks of sizes that do not belong to S thanks to the following steps. The extended set of decoded reference samples, called context, surrounding a given 32×16 block is downsampled horizontally by 2 before being fed into the neural network predicting 16×16 blocks and the neural network prediction is interpolated by 2. The same goes for a 64×64 block, but the horizontal and vertical downsampling factors are 2, and the neural network predicting 32×32 blocks is used for prediction. For h, w∈8, 4, 16, 4, 32, 4, 16, 8, 32, 16, the context of a given w×h block is transposed before being fed into the neural network and the neural network prediction is transposed, allowing the prediction via the neural network designed to predict h×w blocks.

# Proposal

## Inherently sparse learned intra prediction function

The neural network-based intra prediction function obtained after training exhibits an inherent sparsity. For instance, when the neural network predicting 4×4 blocks, belonging to the single neural network-based intra prediction mode, is trained without the widely used lp-norm weight decay, p∈1, 2, at the end of the training, the weights in its first and second layers have the histograms displayed in Figure 1 and Figure 2 respectively. Similarly, when the neural network predicting 16×8 blocks, belonging to the single neural network-based intra prediction mode, is trained without the lp-norm weight decay, at the end of the training, the weights in its first and second layers have the histograms displayed in Figure 3 and Figure 4.

Figure 1: weights in the first layer of the neural network predicting 4×4 blocks, belonging to the single neural network-based intra prediction mode.

Figure 2: weights in the second layer of the neural network predicting 4×4 blocks.

Figure 3: weights in the first layer of the neural network predicting 16×8 blocks, belonging to the single neural network-based intra prediction mode.

Figure 4: weights in the second layer of the neural network predicting 16×8 blocks.

## Building the low-complexity version of the neural network-based mode

In the hardware implementation of a multiplication between two tensors, any multiplication involving 0 can be omitted. Therefore, weights sparsity can decrease the complexity on hardware implementation.

Given this and the remark on the inherently sparse neural network-based intra prediction function, see Section 1.1, a low-complexity version of the neural network-based intra prediction mode in a hardware implementation can be created as follows.

First, to increase the sparsity of the weights during the training phase, each neural network belonging to the single neural network-based mode is trained using the l1-norm weight decay. Then, all the weights inside the trained neural network whose absolute values are below a threshold are set to 0.

# Information on the neural networks in the experiments

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 1.9.0 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 100 |
|  | Training time | 8 hours |
|  | Training data information | The set of images used to create the training sets combines frames extracted from the BVI-DVC dataset and the RGB images in the CLIC2020 training set for low-rate image compression, converted into YCbCr. |
|  | Training configurations for generating compressed training data | VTM-8.0; QP {22, 27, 32, 37} |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context. |
|  | Learning rate | 1e-4 |
|  | Optimizer | ADAM |
|  | Loss function | Mean-squared error. A l1-norm weight decay term is added to the loss function for training the neural networks in the “low-complexity neural network-based mode”. |
|  | Preprocessing | See [6] |

| Network information in the inference stage |  |  |
| --- | --- | --- |
| Parameter number | “regular neural network-based mode” | “low-complexity mode” (weights at 0 not counted) |
|  | θ4,4: 1574430 parameters. θ4,8: 1632046 parameters. θ4,16: 1747278 parameters. θ4,32: 1977742 parameters. θ8,8: 1920078 parameters. θ8,16: 2150542 parameters. θ16,16:1256282 parameters. θ32,32: 3904782 parameters. | θ4,4: 99187 parameters. θ4,8: 110769 parameters. θ4,16: 134149 parameters. θ4,32: 180907 parameters. θ8,8: 142904 parameters. θ8,16: 189662 parameters. θ16,16:1256282 parameters. θ32,32: 3904782 parameters. |
| Parameter precision | 32-bit float for Test-1 and Test-2; 16-bit signed integer for Test-3 and Test-4. |  |
| MAC per pixel | “regular neural network-based mode” | “low-complexity mode” |
|  | 100 kMAC/pixel: 4×4 block. 50 kMAC/pixel: 8×4 block. 26562 MAC/pixel: 16×4 block. 15625 MAC/pixel: 32×4 block. 29687 MAC/pixel: 8×8 block. 16406 MAC/pixel: 16×8 block. 16797 MAC/pixel: 16×16 block. 11230 MAC/pixel: 32×32 block. | 6199 MAC/pixel: 4×4 block. 3465 MAC/pixel: 8×4 block. 2098 MAC/pixel: 16×4 block. 1414 MAC/pixel: 32×4 block. 2234 MAC/pixel: 8×8 block. 1482 MAC/pixel: 16×8 block. 16797 MAC/pixel: 16×16 block. 11230 MAC/pixel: 32×32 block. |
| Total conv. layers | See Figure 3, Table 2, Table 3, Table 5, Table 6 in [6] |  |
| Total FC layers | See Table 1, Table 4, Table 7 in [6] |  |
| Mem.T (MB) | 1.44 MB (1200×1200) |  |
| GPU Type | No GPU |  |
| CPU Type | Xeon 2.4GHz |  |
| Framework | C++ with a single thread in both all tests |  |

# Experimental results

Now that a low-complexity version of the neural network-based intra prediction mode can be created, the loss in terms of rate-distortion of the low-complexity version of the neural network-based mode inside VVC with respect to the regular neural network-based mode inside VVC must be measured.

In this section, the neural network architectures and the signaling of the single neural network-based intra prediction mode in VVC for both luminance and chrominance in [4] are re-used. The only difference between the integration of the single neural network-based mode in VVC in [4] and the current one lies in the fact, here, the neural network-based intra prediction mode neither infers nor predicts the implicit LFNST signaling for a block predicting via this mode.

In the single neural network-based intra prediction mode, only the fully-connected neural networks are targeted by the proposed reduction of complexity, i.e. the 6 neural networks predicting blocks of sizes in 4×4, 8×4, 16×4, 32×4, 8×8, 16×8.

The conditions of the experiments follow [5]. The table below details, in each rate-distortion test, which modified version of VTM-11.0-NNVC is compared to VTM-11.0-NNVC.

|  | regular neural network-based mode | low-complexity neural network-based mode |
| --- | --- | --- |
| float NNs | Test-1 |  |
| 16-bit signed integer NNs | Test-3 | Test-4 |

Note that, to generate DecT in the six tables below, a computation in the standard reported template of the AHG11 was corrected. Indeed, in AI, the decoder running times are averaged over the video sequences of the classes A, B, C, and H, whereas any other metric is averaged over the video sequences of the classes A, B, C, and E. Therefore, to produce DecT in these six tables, the decoder running times are averaged other the video sequences of the classes A, B, C, and E.

Table 1: AI results, Test-1 (float NNs, regular neural network-based mode) versus VTM 11.0-NNVC

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -3.99% | -3.25% | -3.45% | -4.17% | -2.78% | -3.13% | 344% | 3318% | 3% |
| Class A2 | -2.44% | -2.36% | -2.75% | -2.49% | -1.67% | -2.18% | 370% | 3540% | 2% |
| Class B | -2.93% | -2.41% | -2.68% | -2.68% | -2.01% | -1.79% | 373% | 3860% | 2% |
| Class C | -3.09% | -2.48% | -2.70% | -3.10% | -1.84% | -1.78% | 367% | 4527% | 3% |
| Class E | -4.58% | -4.31% | -3.40% | -4.72% | -4.03% | -1.95% | 379% | 4629% | 4% |
| Overall | -3.34% | -2.87% | -2.94% | -3.33% | -2.38% | -2.10% | 367% | 3962% | 3% |
| Class D | -2.99% | -2.64% | -2.80% | -2.87% | -1.81% | -1.69% | 359% | 4745% | 2% |
| Class F | -2.25% | -1.64% | -1.72% | -2.32% | -1.39% | -1.14% | 212% | 2272% | 2% |
| Class H |  |  |  |  |  |  |  |  |  |

Table 2: AI results, Test-3 (16-bit NNs, regular neural network-based mode) versus VTM 11.0-NNVC

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -3.93% | -3.10% | -3.37% | -4.14% | -2.65% | -3.10% | 211% | 1845% | 3% |
| Class A2 | -2.43% | -2.33% | -2.69% | -2.48% | -1.70% | -2.16% | 217% | 1805% | 2% |
| Class B | -2.92% | -2.41% | -2.57% | -2.67% | -1.98% | -1.76% | 219% | 1903% | 2% |
| Class C | -3.08% | -2.48% | -2.78% | -3.11% | -1.77% | -1.86% | 217% | 1927% | 3% |
| Class E | -4.55% | -4.29% | -3.53% | -4.69% | -3.94% | -1.98% | 222% | 2198% | 4% |
| Overall | -3.31% | -2.84% | -2.93% | -3.32% | -2.33% | -2.11% | 217% | 1927% | 3% |
| Class D | -2.98% | -2.51% | -2.73% | -2.89% | -2.09% | -1.81% | 208% | 1926% | 2% |
| Class F | -2.22% | -1.58% | -1.63% | -2.34% | -1.15% | -1.28% | 139% | 1134% | 2% |
| Class H |  |  |  |  |  |  |  |  |  |

Table 3: AI results, Test-4 (16-bit NNs, low-complexity neural network-based mode) vs. VTM 11.0-NNVC

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -3.76% | -2.87% | -3.21% | -4.02% | -2.38% | -3.02% | 207% | 1802% | 3% |
| Class A2 | -2.09% | -1.91% | -2.24% | -2.21% | -1.38% | -1.74% | 222% | 1767% | 2% |
| Class B | -2.49% | -1.97% | -2.10% | -2.32% | -1.60% | -1.38% | 221% | 1735% | 2% |
| Class C | -2.53% | -1.86% | -1.95% | -2.59% | -1.12% | -1.03% | 218% | 1790% | 2% |
| Class E | -3.88% | -3.39% | -2.67% | -4.10% | -3.18% | -1.20% | 222% | 2023% | 3% |
| Overall | -2.87% | -2.32% | -2.37% | -2.94% | -1.85% | -1.60% | 218% | 1809% | 2% |
| Class D | -2.47% | -2.05% | -2.08% | -2.44% | -1.53% | -1.22% | 211% | 1749% | 2% |
| Class F | -1.82% | -1.19% | -1.18% | -1.90% | -0.83% | -0.60% | 137% | 993% | 2% |
| Class H |  |  |  |  |  |  |  |  |  |

Table 4: RA results, Test-1 (float NNs, regular neural network-based mode) versus VTM 11.0-NNVC

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -2.18% | -0.81% | -1.67% | -2.06% | -0.43% | -1.49% | 138% | 550% | 2% |
| Class A2 | -1.17% | -0.36% | -0.84% | -1.13% | 0.49% | -0.25% | 130% | 380% | 1% |
| Class B | -1.57% | -0.36% | -0.46% | -1.38% | 0.09% | 0.66% | 137% | 491% | 1% |
| Class C | -1.52% | -0.83% | -1.08% | -1.43% | -0.22% | -0.15% | 146% | 749% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.60% | -0.57% | -0.94% | -1.48% | -0.02% | -0.17% | 138% | 534% | 1% |
| Class D | -1.35% | -0.72% | -0.82% | -1.53% | -0.41% | 0.01% | 137% | 654% | 1% |
| Class F | -1.14% | -0.38% | -0.59% | -0.90% | 0.25% | 0.58% | 133% | 356% | 1% |
| Class H |  |  |  |  |  |  |  |  |  |

Table 5: RA results, Test-3 (16-bit NNs, regular neural network-based mode) versus VTM 11.0-NNVC

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -2.15% | -0.68% | -1.55% | -2.09% | -0.33% | -1.45% | 111% | 337% | 2% |
| Class A2 | -1.16% | -0.41% | -0.88% | -1.11% | 0.39% | -0.40% | 103% | 211% | 1% |
| Class B | -1.57% | -0.53% | -0.44% | -1.41% | -0.11% | 0.62% | 111% | 291% | 1% |
| Class C | -1.50% | -0.73% | -0.95% | -1.40% | -0.12% | 0.13% | 113% | 354% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.58% | -0.59% | -0.89% | -1.48% | -0.06% | -0.13% | 110% | 296% | 1% |
| Class D | -1.34% | -1.24% | -1.09% | -1.32% | -0.88% | -0.07% | 108% | 315% | 1% |
| Class F | -1.09% | -0.53% | -0.67% | -0.99% | -0.14% | 0.56% | 107% | 217% | 1% |
| Class H |  |  |  |  |  |  |  |  |  |

Table 6: RA results, Test-4 (16-bit NNs, low-complexity neural network-based mode) vs. VTM 11.0-NNVC

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -2.02% | -0.78% | -1.47% | -1.95% | -0.60% | -1.39% | 110% | 327% | 1% |
| Class A2 | -1.00% | -0.36% | -0.73% | -1.00% | 0.40% | -0.18% | 102% | 208% | 1% |
| Class B | -1.35% | -0.43% | -0.59% | -1.22% | -0.04% | 0.48% | 108% | 264% | 1% |
| Class C | -1.24% | -0.69% | -0.70% | -1.22% | -0.25% | 0.25% | 112% | 341% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.38% | -0.55% | -0.82% | -1.32% | -0.12% | -0.09% | 108% | 281% | 1% |
| Class D | -1.11% | -0.67% | -0.60% | -1.18% | 0.30% | 0.25% | 108% | 283% | 1% |
| Class F | -0.93% | -0.36% | -0.53% | -0.76% | -0.28% | 0.28% | 106% | 191% | 1% |
| Class H |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution introduced a low-complexity version of the single additional neural network-based intra prediction mode inside VVC for a hardware implementation. It is reported that the complexity of the neural network-based intra prediction mode in VVC, measured in MAC/pixel, can be divided by a factor of about 12 while giving up only 0.40% of mean BD-rate reduction approximatively in luminance in AI.

Thanks to Johannes Sauer and Elena Alshina from Huawei for cross-checking this contribution!

# References

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Affine linear weighted intra prediction”, JVET-N0217.

- J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand, “8-bit implementation and simplification of MIP”, JVET-O0084.

- W. Chen, Y. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y. Huang, H. Jang, R. Liao, S. Liu, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-T0013.

- T. Dumas, F. Galpin, P. Bordes, P. Nikitin, “AHG11: neural network-based intra prediction: updated signaling”, JVET-V0105.

- S. Liu, A. Segall, E. Alshina, R. Liao , “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-U2016.

- T. Dumas, F.Galpin, P. Bordes, F. Le Léannec, “AHG11: neural network-based intra prediction with transform selection in VVC”, JVET-T0073.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).