| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0119r1 |
| --- | --- |

| Title: | Non-EE2: On pairwise merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Romain Bellessort Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the merge candidates derivation. It is proposed to derive the pairwise candidate during the reordering process of the merge candidates list (AMCR-TM, JVET-W0090). In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate.

On top of the ECM-2.0, simulation results of the proposed methods are reported as follows:

RA: -0.13%, 101%, 101%

LB: -0.10%, 99%, 100%

# Introduction

The pairwise candidate in ECM is the same as VVC. This candidate combines the two first candidates of the regular merge list in order to create a combined candidate and/or an average candidate. This candidate is added at the end of the regular merge list when it is not a duplicate.

# Proposed method

In this contribution, the pairwise candidate is removed from the regular merge candidate derivation. It is added, when it is not a duplicate, during the adaptive reordering of merge candidates with template matching (ARMC-TM) [1] as depicted in Figure 1. The pairwise is built with the two first reordered candidates. The number of reordered candidates is the same as in the ECM2.0 implementation as well as the number of the template matching costs computed.

The pairwise candidate is also restricted to use only the average candidate when the reference frames of the first and second reordered candidate are the same.

In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate if the created pairwise is not a duplicate.

Figure 1. Pairwise in the merge candidate reoredering ARMC-TM and additional pairwise candidates after reordering.

# Results

The proposed modification is implemented on top of the ECM-2.0 [2] and tested according to the common test condition (CTC) [3].

Table 1 Coding performance of the proposed modifications

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.13% | -0.09% | 101% | 101% |
| Class A2 | -0.11% | -0.04% | -0.04% | 101% | 101% |
| Class B | -0.13% | -0.14% | -0.12% | 100% | 101% |
| Class C | -0.15% | -0.10% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.11% | -0.06% | 101% | 101% |
| Class D | -0.11% | -0.19% | -0.03% | 99% | 100% |
| Class F | -0.02% | 0.03% | -0.06% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| ssClass A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.19% | 0.05% | 100% | 99% |
| Class C | -0.22% | -0.13% | -0.07% | 98% | 99% |
| Class E | -0.01% | -0.46% | -0.57% | 99% | 101% |
| Overall | -0.10% | -0.08% | -0.14% | 99% | 100% |
| Class D | -0.02% | -0.15% | -0.41% | 98% | 103% |
| Class F | -0.21% | 0.04% | -0.21% | 103% | 102% |

# Conclusion

In this contribution, a modification of the pairwise merge candidate is proposed. A coding efficiency of more than -0.1% has been achieved for both RA and LB configurations without additional encoding and decoding time increase. We therefore suggest investigating this proposal in an EE for the next meeting.

# References

[1]	Na Zhang, Kai Zhang, Li Zhang, Hongbin Liu1, Zhipin Deng, Yang Wang, “EE2-3.1/EE2-3.2: Adaptive Reordering of Merge Candidates with Template/Bilateral Matching,” JVET-W0090, July 2021.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-2.0

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-W2017, Apr. 2021.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).