| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 11–15 October 2021 | Document: JVET-X0122-v2 |
| --- | --- |

| Title: | Non-EE2: Unification of negative modes processing in TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Kalyan Goswami, Damian Ruiz Coll, Young-Yoon Lee, Tae Meon Bae, Esmael Dinan | Tel: Email: | {afilippov, vrufitskiy} @ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents a modification to the inverse angle processing that is performed when TIMD mode is selected. Reportedly, the proposed modification provides the following overall results: -0.09% (Y) /-0.13% (U) /-0.14% (V) and x.xx% (Y) / x.xx% (U) / x.xx% (V) in AI and RA configurations, respectively. The coding performance improvement is achieved at no complexity and runtime increase for both encoder and decoder.

# Introduction

In VVC and ECM-2.0, when directional intra prediction is performed for negative angles, a main reference side is extended to the left. This extension is performed by calculation of fractional positions on the side reference for each of the extension positions of the main reference. This process is currently performed as follows:

The values of prediction angle parameters “IntraPredAngle” and “IntraPredAngleExt” are defined in Table 1 and Table 2.

Table 1. IntraPredAngle values

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 512 | 341 | 256 | 171 | 128 | 102 | 86 | 73 | 64 | 57 | 51 | 45 | 39 | 35 | 32 | 29 | 26 |
| predModeIntra | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| intraPredAngle | 23 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 3 | 2 | 1 | 0 | −1 | −2 | −3 |
| predModeIntra | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| intraPredAngle | −4 | −6 | −8 | −10 | −12 | −14 | −16 | −18 | −20 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −20 |
| predModeIntra | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| intraPredAngle | −18 | −16 | −14 | −12 | −10 | −8 | −6 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | 6 |
| predModeIntra | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
| intraPredAngle | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 |
| predModeIntra | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 73 | 86 | 102 | 128 | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |

Table 2. IntraPredAngleExt values

| predModeIntra | -28 | -27 | -26 | -25 | -24 | -23 | -22 | -21 | -20 | -19 | -18 | -17 | -16 | -15 | -14 | -13 | -12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredAngleExt | 1024 | 853 | 682 | 597 | 512 | 427 | 342 | 299 | 256 | 230 | 204 | 188 | 172 | 159 | 146 | 137 | 128 |
| predModeIntra | -11 | -10 | -9 | -8 | -7 | -6 | -5 | -4 | -3 | -2 | -1 | 2 | 3 | 4 | 5 | 6 | 7 |
| IntraPredAngleExt | 121 | 114 | 108 | 102 | 96 | 90 | 84 | 78 | 74 | 70 | 67 | 64 | 61 | 58 | 55 | 52 | 49 |
| predModeIntra | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| IntraPredAngleExt | 46 | 43 | 40 | 38 | 36 | 34 | 32 | 30 | 28 | 26 | 24 | 22 | 20 | 18 | 16 | 14 | 12 |
| predModeIntra | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| IntraPredAngleExt | 10 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 |
| predModeIntra | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 |
| IntraPredAngleExt | -8 | -10 | -12 | -14 | -16 | -18 | -20 | -22 | -24 | -26 | -28 | -30 | -32 | -34 | -36 | -38 | -40 |
| predModeIntra | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 |
| IntraPredAngleExt | -43 | -46 | -49 | -52 | -55 | -58 | -61 | -64 | -61 | -58 | -55 | -52 | -49 | -46 | -43 | -40 | -38 |
| predModeIntra | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 |
| IntraPredAngleExt | -36 | -34 | -32 | -30 | -28 | -26 | -24 | -22 | -20 | -18 | -16 | -14 | -12 | -10 | -8 | -7 | -6 |
| predModeIntra | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 |
| IntraPredAngleExt | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 10 | 12 | 14 |
| predModeIntra | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 |
| IntraPredAngleExt | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 43 | 46 | 49 | 52 |
| predModeIntra | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| IntraPredAngleExt | 55 | 58 | 61 | 64 | 67 | 70 | 74 | 78 | 84 | 90 | 96 | 102 | 108 | 114 | 121 | 128 | 137 |
| predModeIntra | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 |  |  |
| IntraPredAngleExt | 146 | 159 | 172 | 188 | 204 | 230 | 256 | 299 | 342 | 427 | 512 | 597 | 682 | 853 | 1024 |  |  |

The value of “bExtIntraDir” flag determines precision of the inverse angle. It is set equal to true when an intra prediction for a block was derived using TIMD.

invAngle =Round512*32intraPredAngle, if bExtIntraDir is falseRound512*64intraPredAngleExt, otherwise

If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- The value of fractional offset frac32precision is derived as follows:

frac32precision = bExtIntraDir ? ((x *invAngle + 16) >> 5) : ((x * invAngle + 8) >> 4)

, with x = −nTbH..1

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx =  frac32precision >>  5

iFact = frac32precision & 31

The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = fC[ iFact ][ j ]

The value of sample ref[ -x ][ -refidx ] is derived as follows:

ref[[ -x ][ -refidx ] = Clip1Y( ( ( i=03fT[ i ]*p[ -refIdx][-refIdx+iIdx+i ] ) + 32 )  >>  6 )

# Proposed method

In this contribution, the following modification is proposed (highlighted):

…

invAngle =Round512*32intraPredAngle, if bExtIntraDir is falseRound512*64intraPredAngleExt, otherwise

If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- The value of fractional offset frac32precision is derived as follows:

frac32precision = (x * invAngle + 8) >> 4

, with x = −nTbH..1

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx =  frac32precision >>  5

iFact = frac32precision & 31

The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = fC[ iFact ][ j ]

The value of sample ref[ -x ][ -refidx ] is derived as follows:

ref[[ -x ][ -refidx ] = Clip1Y( ( ( i=03fT[ i ]*p[ -refIdx][-refIdx+iIdx+i ] ) + 32 )  >>  6 )

# Results

The results obtained for AI and RA configurations in accordance with [3] are presented in Table 3 and Table 4, respectively.

Table 3. Performance of the proposed method (AI configuration)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.19% | 101% | 100% |
| Class A2 | -0.08% | -0.09% | -0.10% | 100% | 100% |
| Class B | -0.07% | -0.10% | -0.09% | 100% | 101% |
| Class C | -0.10% | -0.07% | -0.15% | 100% | 100% |
| Class E | -0.18% | -0.28% | -0.23% | 100% | 100% |
| Overall | -0.09% | -0.13% | -0.14% | 100% | 100% |
| Class D | -0.05% | 0.03% | -0.09% | 100% | 101% |
| Class F | -0.08% | -0.26% | -0.20% | 100% | 100% |
| Class TGM | N/A | N/A | N/A | N/A | N/A |

Table 4. Performance of the proposed method (RA configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.03% | 101% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.07% | 0.01% | 0.09% | 100% | 100% |
| Class C | -0.05% | 0.09% | 0.09% | 101% | 100% |
| Class E | - | - | - | - | - |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | -0.21% | 0.08% | 100% | 100% |
| Class F | -0.06% | 0.05% | 0.08% | 100% | 100% |
| Class TGM | N/A | N/A | N/A | N/A | N/A |

# Conclusions

This contribution presents a unification of main reference side extension for TIMD and non-TIMD intra prediction modes. We recommend to adopt this modification into the ECM reference software.

# References

- Y. Wang, L. Zhang, K. Zhang, Z. Deng, and N. Zhang, “EE-related: Template-based intra mode derivation using MPMs”, ISO/IEC JTC1/SC29/WG11 JVET-V0098, April 2021.

- K. Cao, N. Hu, V. Seregin, M. Karczewicz, Y. Wang, K. Zhang, and L. Zhang, “EE-related: Fusion for template-based intra mode derivation”, ISO/IEC JTC1/SC29/WG11 JVET-W0123, July 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-W2017, July 2021.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).