| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: by teleconference, 6 – 15 Oct. 2021 | Document: JVET-X0126-v1 |
| --- | --- |

| Title: | [AHG11] Neural Network-based Adaptive Model Selection for CNN In-Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Kazushi Sato Luhang Xu Zhihuang Xie Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution presents a convolutional neural network-based in-loop filter (CNNLF) wherein a neural network-based, adaptive model selection (NNAMS) method is introduced. The proposed CNNLF+NNAMS is integrated in the code base of VTM-11.0 + new MCTF. Compared with VTM-11.0 + new MCTF, the proposed method demonstrates {X.XX%, X.XX%, X.XX%} and { X.XX%, X.XX%, X.XX%} BD-rate savings with the AI and RA configurations, respectively.

# Introduction

There have been several contributions on using multiple models of CNNLF to further improve coding performance [1][2].

In this contribution, a convolutional neural network-based in-loop filter (CNNLF), also comprising multiple models, is introduced. The CNNLF is placed between SAO and ALF in the in-loop filter process, and these individual CNNLF models are trained separately with visual content that have been processed by using different QPs. Furthermore, a dedicated neural network, which is trained in conjunction with the CNNLF models, is added to the video codec. Using this neural network, a flag-free method of neural network-based, adaptive model selection (NNAMS) is developed that can adaptively select at the CTU level one of the CNNLF models for the best coding performance without sending CNNLF model information in the bitstream. More details of the CNNLF as well as NNMAS are described in the next section.

# Proposed method

## Network architecture of CNNLF

The structure of the proposed CNNLF is shown in Fig. 1. The first layer and the last layer, labeled as 3x3 Conv, are regular convolutional layers with kernel size 3. Between these two convolutional NN layers, there are N Residual Blocks, where N is set to 10 in this contribution. Inside each Residual Block, two regular convolutional NNs, also with kernel size 3, are connected through a PReLU activation function. A bypass construct, which allows combining the output of the last 3x3 Conv layer with the input of the network to make the final output, is added to the design of CNNLF and Residual Block. For the case of CNNLF, the bypass connection enables blending the output of the last 3x3 Conv layer with the input reconstructed frame to make the final CNNLF reconstructed frame. For all the convolutional layers, the number of feature maps is set as 64. Different groups of CNNLF models are trained for Y, U, V, respectively.

Fig. 1: CNNLF’s architecture

## Network architecture of NNAMS

The structure of the proposed NNAMS is shown in Fig. 2. The network starts off with three layers of regular convolutional NN with kernel size 3. Each of these three convolution layers is followed by a 2x2 MaxPooling layer. Then, a fully connected layer is placed followed by a PReLU activation function. Finally, a second fully connected layer is placed and then followed by a Softmax activation function to derive the index of the selected CNNLF model. For all the convolutional layers, the number of feature maps is set as 64. Different groups of NNAMS models are trained for Y, U, V, respectively.

Fig. 2 NNAMS’s architecture

## Neural network-based adaptive model selection (NNAMS)

In the proposed method, separate CNNLF models are trained with individual CTC QPs{22, 27, 32, 37, 42} for Y, U, V, respectively. Therefore, a candidate list of 5 CNNLF models can be set up for each color component. During the encoding and decoding process, only one CNNLF model should be selected out of 5 models in the candidate list for the current CTU. To this end, a neural network-based adaptive model selection method (NNAMS) is proposed to predict the most suitable CNNLF model for the current CTU. Since the NNAMS method is used in both the encoder and the decoder, there’s no need to signal the selected model index in the bitstream. After selecting the CNNLF model to be used for the current CTU, the encoder can further decide whether to apply the CNNLF to the current CTU through dedicated flags.

## Training information

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel(R) Xeon(R) Gold 6142 CPU @ 2.60GHz GPU: Tesla V100 (32GB) |
|  | SW environment: | OS: Ubuntu 16.04.4 |
|  | Framework: | Pytorch v1.6.0 |
|  | Epoch: | 20 |
|  | Batch size: | 32 |
|  | Training time: | 48h |
|  | Training data information: | DIV2K |
|  | Configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Patch size | 128X128x1 for Y, 64x64x1 for U or V |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: |  |
|  | Other information: |  |

## Inference information

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | Pytorch v1.6.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Conv. Layers | 22 for CNNLF, 3 for NNAMS |
|  | Total FC Layers | 3 for NNAMS |
|  | Total Parameter Number | CNNLF: 739k NNAMS: 41k(Y), 39k(U or V) |
|  | Parameter Precision | 32 (F) |
|  | Memory Parameter (MB) | CNNLF: 3.30MB/model, 15models NNAMS: 273KB/model, 15models |
|  | Memory Temp (MB) |  |
|  | Multiplay Accumulate (MAC)/pixel | CNNLF: 740k NNAMS: 9.76k |
| Optional |  |  |
|  | Patch size | 128X128x1 for Y, 64x64x1 for U or V |
|  | Other information: |  |

# Experiment results

In this contrition, the proposed CNNLF and CNNLF+NNAMS are tested with the code base of VTM-11.0 + new MCTF, and 5 CNNLF models {modelQP22, modelQP27, modelQP32, modelQP37, modelQP42} for each color component were applied in these tests. These CNNLF models were trained individually with the content that were compressed by VTM software with the CTC AI configuration and the corresponding QP. For example, the material used for training modelQP22 was a compressed version of DIV2K, where the VTM was configured for CTC AI and QP=22.

All the anchors in this section were produced by VTM-11.0+new MCTF software with the CTC configurations.

## Test 1: CNNLF only

For the CTC AI configuration, a fixed CNNLF model, selected based on the encoding QP, is used in the coding of the entire sequence. For example, if the coding QP is 27, the modelQP27 will be selected from the list of {modelQP22, modelQP27, modelQP32, modelQP37, modelQP42}. Table 3 shows the coding results. On average, under the AI configuration, the proposed CNNLF can bring {X.XX%, X.XX%, X.XX%} BD-rate savings for Y, U, V, respectively.

Table 3. Performance of the proposed CNNLF on code base of VTM-11.0 + new MCTF (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.24% | -4.14% | -6.15% | 169% | 34652% |
| Class A2 |  |  |  |  |  |
| Class B | -2.62% | -3.91% | -5.57% | 135% | 28627% |
| Class C | -3.82% | -5.40% | -7.13% | 120% | 21496 |
| Class E | -4.91% | -6.15% | -6.60% | 141% | 29431% |
| Overall |  |  |  |  |  |
| Class D | -4.39% | -4.24% | -6.46% | 116% | 20846% |
| Class F | -1.82% | -4.88% | -5.75% | 117% | 24211% |

For the CTC RA configuration, the following model selection method is applied:

if the current coding qp <=22: CNNLF model is modelQP22;

else if the current coding qp <=27: CNNLF model is modelQP27;

else if the current coding qp <=32: CNNLF model is modelQP32;

else if the current coding qp <=37: CNNLF model is modelQP37;

else: CNNLF model is modelQP42;

Table 4 shows the coding results. On average, under the RA configuration, the proposed CNNLF can bring {X.XX%, X.XX%, X.XX%} BD-rate savings for Y, U, V, respectively.

Table 4. Performance of the proposed CNNLF on code base of VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.33% | -4.73% | -5.69% | 144% | 56814% |
| Class C | -1.50% | -3.97% | -4.95% | 133% | 50323% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -2.29% | -3.56% | -4.47% | 138% | 31287% |
| Class F | -0.36% | -3.78% | -4.12% | 163% | 41848% |

## Test 2: CNNLF + NNAMS with CTC RA configuration

In this test, the NNAMS method is applied to every frame, and both the encoder and the decoder use the same CNNLF model selected by NNAMS at the CTU level whenever the CNNLF flag is set to true in the bitstream. Table 5 shows the coding results. On average, under the RA configuration, the proposed CNNLF can bring {X.XX%, X.XX%, X.XX%} BD-rate savings for Y, U, V, respectively.

Comparing Table 4 with Table 5, around x.xx% of average coding gain has been observed.

Table 5. Performance of the proposed CNNLF+NNAMS on code base of VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.78% | -5.50% | -6.56% | 145% | 61700% |
| Class C | -2.23% | -5.36% | -6.24% | 132% | 57148% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -3.19% | -4.85% | -5.92% | 137% | 40594% |
| Class F | -0.72% | -4.73% | -4.88% | 171% | 50340% |

# Conclusions

This contribution presents a convolutional neural network-based in-loop filter (CNNLF) wherein a neural network-based, adaptive model selection (NNAMS) method is introduced. When working alone, the proposed CNNLF shows interesting coding gains compared to the VTM-11.0 + new MCTF. It is observed that the coding gains can be further increased when using the proposed CNNLF together with the proposed NNAMS method. It is proposed to further study neural network-based adaptive model selection for NN-based in-loop filters in an Ad-Hoc group.

# References

- AHG11: Deep In-Loop Filter with Adaptive Model Selection and External Attention, JVET-W0100, Y. Li, K. Zhang, L. Zhang. (Bytedance)

- EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer, JVET-W0130, H. Wang, J. Chen, K. Reuzé, A. M. Kotra, M. Karczewicz. (Qualcomm)

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).