| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0128-v2 |
| --- | --- |

| Title: | AHG8: On History-Based Rice Parameter Derivations for Wavefront Parallel Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Haoping Yu Zhihuang Xie Fan Wang Luhang Xu Dong Wang | Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

A history-based Rice parameter derivation method [3] was adopted in JVET V-meeting. A counter per color component is utilized to maintain a history record with data relating to the Rice parameter and may be updated TU by TU. The counter in the preceding CTU row will be passed to the first TU in the current CTU row. As a result, this process will break the parallelism of WPP when WPP is enabled at the same time. This contribution proposes two methods to address the issue. The first method proposes that the counter per color component is saved after the last TU of the first CTU in each CTU row is coded. For each CTU row except the first CTU row, prior to the first TU coding, the synchronization process is applied, and the counter is synchronized with the saved counter from the preceding CTU row. The second method proposes that a fixed initial value for counter is used prior to the first TU coding in each CTU row. Under the high bit-depth common test condition (CTC), the simulation results show minor loss in BD-bitrate for both methods. Comparing method 2 with method 1, method 2 does not require any extra memory to save extra variable while method 2 has the same coding performance with method 1.

Method 1:

Low QP

PQ: AI: 0.04%, 0.04%, 0.02%; LDB: 0.09%, 0.08%, 0.03%; RA: 0.09%, 0.08%, 0.01%;

HLG: AI: 0.02%, 0.01%, 0.01%; LDB: 0.08%, 0.04%, 0.00%; RA: 0.10%, 0.06%, 0.02%;

SVT: AI: 0.01%, 0.01%, 0.01%; LDB: 0.02%, 0.02%, 0.01%; RA: 0.03%, 0.02%, 0.01%;

Lossless:

PQ: AI: 0.01%; LDB: 0.01%; RA: 0.01%;

HLG: AI: 0.02%; LDB: 0.03%; RA: 0.03%;

SVT: AI: 0.01%; LDB: 0.02%; RA: 0.02%;

Method 2:

Low QP

PQ: AI: 0.04%, 0.04%, 0.02%; LDB: 0.09%, 0.08%, 0.03%; RA: 0.09%, 0.08%, 0.01%;

HLG: AI: 0.02%, 0.01%, 0.01%; LDB: 0.08%, 0.04%, 0.00%; RA: 0.10%, 0.06%, 0.02%;

SVT: AI: 0.01%, 0.01%, 0.01%; LDB: 0.02%, 0.02%, 0.01%; RA: 0.03%, 0.02%, 0.01%;

Lossless:

PQ: AI: 0.01%; LDB: 0.01%; RA: 0.01%;

HLG: AI: 0.02%; LDB: 0.03%; RA: 0.03%;

SVT: AI: 0.01%; LDB: 0.02%; RA: 0.02%;

# Introduction

A history-based Rice parameter derivation method JVET-V0106 [3] was adopted in VVC version 2 [2]. To improve coding efficiency, this method enhances the Rice parameter derivation process defined in VVC version 1 [1] by using a history derived value, instead of 0, for template positions outside of the current TU in the computation of localSumAbs.

To maintain a history, a single counter per color component, StatCoeff [3] is utilized. It is being updated once per TU from the first, non-zero, Golomb-Rice coded transform coefficient (abs_remainder[ ] or dec_abs_level[ ]) through a process of exponential moving average.

When the first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as the following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(abs_remainder[ ])) + 2 ) >> 1

When the first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as dec_abs_level, the history counter for color component cIdx is updated as the following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(dec_abs_level[ ])) ) >> 1

Prior to TU decoding and history counter update, the variable HistValue is being initialized as:

HistValue = 1 << StatCoeff[cIdx]

If the current CTU is the first CTU in a slice or tile, the StatCoeff[cIdx] is initialized as follows.

StatCoeff[ idx ] = 2 * Floor( Log2( BitDepth − 10 )

where BitDepth specifies the bit depth of the samples of the luma and chroma arrays, Floor( x ) represents the largest integer less than or equal to x and Log2( x ) is base-2 logarithm of x. The locSumAbs is re-derived according to HistValue [3].

Wavefront parallel process (WPP) was designed to provide a parallel coding mechanism [1]. When WPP is enabled in VVC [1], each CTU row of a picture constitutes a separation partition, and the content of the adapted CABAC context variables and palette predictor are propagated from the first coded CTU of the preceding CTU row to the first CTU of the current CTU row.

When history-based Rice parameter derivation [3] is enabled for high bit depth and high bit rate video coding, the last counter in the preceding CTU row will be passed to the first TU in the current CTU row. As a result, this process will break the parallelism of WPP when WPP is enabled at the same time.

# Proposed Methods

## Method 1

Method 1 proposes that the StatCoeff[cIdx], where cIdx is index for each color component, are saved as StatCoeffWpp[cIdx] after the last TU of the first CTU in each CTU row is coded.

Starting from the second CTU row, prior to the first TU coding, the synchronization process for Rice parameter derivation is applied to the first CTU for each CTU row. StatCoeff[cIdx] is synchronized with the saved StatCoeffWpp[cIdx] from the preceding CTU row.

Based upon the current Draft of VVC operation range extension [2], possible specification changes highlighted in yellow are specified as follows.

9.3.1 General

When sps_persistent_rice_adaptation_enabled_flag is equal to 1, the storage process for Rice parameter derivation using statistics accumulated from previous TUs is applied as follows:

- When ending the parsing of the CTU syntax in clause 7.3.11.2 and the decoding process of the last CU in the CTU in clause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for Rice parameter derivation as speficied in clause 9.3.2.x1 is invoked.

9.3.2.1 General

The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in clause 9.3.2.7 is invoked.

–	When sps_persistent_rice_adaptation_enabled_flag is equal to 1, the synchronization process for Rice parameter derivation as specified in clause 9.3.2.x2 is invoked.

#### 9.3.2.x1 Storage process for Rice parameter derivation

This process stores the values of the arrays StatCoeff in the arrays StatCoeffWpp as follows:

for( cIdx = 0; cIdx < 3; cIdx++ ) {

StatCoeffWpp [ cIdx ] = StatCoeff [ cIdx ]
}

#### 9.3.2.x2 Synchronization process for Rice parameter derivation

This process synchronizes the values of the arrays StatCoeff in the arrays StatCoeffWpp as follows:

for( cIdx = 0; cIdx < 3; cIdx++ ) {
	StatCoeff [ cIdx ] = StatCoeffWpp[ cIdx ]
}

## Method 2

Method 2 proposes that a fixed initial value for StatCoeff[cIdx], where cIdx is index for each color component, is used for coding the first abs_remainder[ ] or dec_abs_level[ ] in each CTU row.

An initial value can be as follows,

StatCoeff[ idx ] = 2 * Floor( Log2( BitDepth − 10 )

where BitDepth specifies the bit depth of the samples of the luma or chroma arrays, and Floor( x ) represents the largest integer less than or equal to x.

Based upon the current Draft of VVC operation range extension [2], possible specification changes highlighted in yellow are specified as follows.

9.3.2.1 General

The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp and TableStateIdx1Wpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in clause 9.3.2.7 is invoked.

–	When sps_persistent_rice_adaptation_enabled_flag is equal to 1, the synchronization process for Rice parameter derivation as specified in clause 9.3.2.x3 is invoked.

#### 9.3.2.x3 Synchronization process for Rice parameter derivation

This process synchronizes the values of the arrays StatCoeff as follows:

for( cIdx = 0; cIdx < 3; cIdx++ ) {
	StatCoeff [ cIdx ] = 2 * Floor( Log2( BitDepth − 10 )
}

# Simulation results

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [4]. The anchor is generated with VTM-14.0, the tests are method 1 and method 2, respectively.

## Comparison of method 1

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.03% | 0.04% | 0.02% | 105% | 106% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.04% | 0.04% | 0.02% | 107% | 106% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.04% | 0.04% | 0.02% | 106% | 106% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0_cebase |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.06% | 0.06% | 0.03% | 91% | 100% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.12% | 0.10% | 0.04% | 89% | 93% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.09% | 0.08% | 0.03% | 90% | 96% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.08% | 0.11% | 0.01% | 0.07% | 0.11% | 0.02% | 104% | 105% |
| PQ422 | 0.12% | 0.06% | 0.01% | 0.11% | 0.06% | 0.01% | 101% | 102% |
| Overall | 0.10% | 0.09% | 0.01% | 0.09% | 0.08% | 0.01% | 102% | 103% |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | 0.07% | 0.07% | 0.02% | 99% | 102% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.01% | 0.01% | 108% | 114% |
| HLG422 | 0.02% | 0.01% | 0.01% | 103% | 106% |
| Overall | 0.02% | 0.01% | 0.01% | 106% | 110% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.08% | 0.04% | 0.00% | 89% | 75% |
| HLG422 | 0.09% | 0.04% | 0.01% | 91% | 81% |
| Overall | 0.08% | 0.04% | 0.00% | 90% | 78% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.09% | 0.06% | 0.01% | 100% | 93% |
| HLG422 | 0.10% | 0.06% | 0.02% | 101% | 91% |
| Overall | 0.10% | 0.06% | 0.02% | 100% | 92% |
| Overall HLG | 0.07% | 0.04% | 0.01% | 99% | 93% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.02% | 0.01% | 104% | 99% |
| SVT12 | 0.01% | 0.01% | 0.01% | 107% | 101% |
| Overall | 0.01% | 0.01% | 0.01% | 105% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.02% | 0.02% | 94% | 94% |
| SVT12 | 0.02% | 0.01% | 0.01% | 93% | 94% |
| Overall | 0.02% | 0.02% | 0.01% | 94% | 94% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.02% | 0.02% | 105% | 98% |
| SVT12 | 0.02% | 0.01% | 0.01% | 105% | 103% |
| Overall | 0.03% | 0.02% | 0.01% | 105% | 100% |
| Overall RGB | 0.02% | 0.01% | 0.01% | 101% | 98% |

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| PQ444 | 2.6 | 2.6 | 0.01% | 3.1 | 3.1 | 0.01% | 3.1 | 3.1 | 0.01% |
| PQ422 | 2.4 | 2.4 | 0.01% | 2.9 | 2.9 | 0.01% | 2.9 | 2.9 | 0.01% |
| Overall | 2.5 | 2.5 | 0.01% | 3.0 | 3.0 | 0.01% | 3.0 | 3.0 | 0.01% |
| Enc Time[%] | 95% |  |  | 106% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 106% |  |  | 108% |  |  |

| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| HLG444 | 1.8 | 1.8 | 0.02% | 2.0 | 2.0 | 0.02% | 2.0 | 2.0 | 0.02% |
| HLG422 | 1.7 | 1.7 | 0.02% | 1.9 | 1.9 | 0.04% | 1.9 | 1.9 | 0.04% |
| Overall | 1.7 | 1.7 | 0.02% | 2.0 | 2.0 | 0.03% | 2.0 | 2.0 | 0.03% |
| Enc Time[%] | 91% |  |  | 102% |  |  | 95% |  |  |
| Dec Time[%] | 89% |  |  | 101% |  |  | 103% |  |  |

| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| SVT16 | 1.2 | 1.2 | 0.01% | 1.2 | 1.2 | 0.02% | 1.2 | 1.2 | 0.02% |
| SVT12 | 1.3 | 1.3 | 0.01% | 1.3 | 1.3 | 0.02% | 1.3 | 1.3 | 0.02% |
| Overall | 1.2 | 1.2 | 0.01% | 1.3 | 1.3 | 0.02% | 1.3 | 1.3 | 0.02% |
| Enc Time[%] | 99% |  |  | 94% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 83% |  |  | 95% |  |  |

Normal QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 1.01% | 1.07% | 1.09% | 1.19% | 1.03% | 1.09% | 1.14% | 1.17% | 99% | 99% |
| Class H2 |  |  |  |  |  | 1.24% | 1.54% | 0.76% | 99% | 98% |
| Overall | 1.01% | 1.07% | 1.09% | 1.19% | 1.03% | 1.16% | 1.34% | 0.96% | 99% | 98% |

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.19% | 0.14% | #VALUE! | #VALUE! | #VALUE! | 0.14% | -0.11% | -0.40% | 99% | 98% |
| Class H2 |  |  |  |  |  | 0.13% | -0.23% | -0.37% | 106% | 101% |
| Overall | -0.19% | 0.14% | #VALUE! | #VALUE! | #VALUE! | 0.14% | -0.17% | -0.39% | 103% | 100% |

## Comparison of method 2

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.03% | 0.04% | 0.02% | 102% | 107% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.04% | 0.04% | 0.02% | 106% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.04% | 0.04% | 0.02% | 104% | 107% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.06% | 0.06% | 0.03% | 92% | 98% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.12% | 0.10% | 0.04% | 90% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.09% | 0.08% | 0.03% | 91% | 95% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.08% | 0.11% | 0.01% | 0.07% | 0.11% | 0.02% | 97% | 95% |
| PQ422 | 0.12% | 0.06% | 0.02% | 0.11% | 0.06% | 0.01% | 94% | 92% |
| Overall | 0.10% | 0.09% | 0.01% | 0.09% | 0.08% | 0.01% | 96% | 94% |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | 0.07% | 0.07% | 0.02% | 97% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.01% | 0.01% | 106% | 109% |
| HLG422 | 0.02% | 0.01% | 0.01% | 102% | 112% |
| Overall | 0.02% | 0.01% | 0.01% | 104% | 110% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.08% | 0.04% | 0.00% | 93% | 81% |
| HLG422 | 0.09% | 0.04% | 0.01% | 92% | 84% |
| Overall | 0.08% | 0.04% | 0.00% | 92% | 82% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.09% | 0.06% | 0.01% | 92% | 84% |
| HLG422 | 0.10% | 0.06% | 0.02% | 94% | 86% |
| Overall | 0.10% | 0.06% | 0.02% | 93% | 85% |
| Overall HLG | 0.07% | 0.04% | 0.01% | 97% | 92% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.02% | 0.01% | 101% | 95% |
| SVT12 | 0.01% | 0.01% | 0.01% | 105% | 98% |
| Overall | 0.01% | 0.01% | 0.01% | 103% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.02% | 0.02% | 98% | 93% |
| SVT12 | 0.02% | 0.01% | 0.01% | 97% | 96% |
| Overall | 0.02% | 0.02% | 0.01% | 98% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.03% | 0.02% | 0.02% | 94% | 96% |
| SVT12 | 0.02% | 0.01% | 0.01% | 92% | 98% |
| Overall | 0.03% | 0.02% | 0.01% | 93% | 97% |
| Overall RGB | 0.02% | 0.02% | 0.01% | 98% | 96% |

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| PQ444 | 2.6 | 2.6 | 0.01% | 3.1 | 3.1 | 0.01% | 3.1 | 3.1 | 0.01% |
| PQ422 | 2.4 | 2.4 | 0.01% | 2.9 | 2.9 | 0.01% | 2.9 | 2.9 | 0.01% |
| Overall | 2.5 | 2.5 | 0.01% | 3.0 | 3.0 | 0.01% | 3.0 | 3.0 | 0.01% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 94% |  |  | 98% |  |  |

| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| HLG444 | 1.8 | 1.8 | 0.02% | 2.0 | 2.0 | 0.02% | 2.0 | 2.0 | 0.02% |
| HLG422 | 1.7 | 1.7 | 0.02% | 1.9 | 1.9 | 0.04% | 1.9 | 1.9 | 0.04% |
| Overall | 1.7 | 1.7 | 0.02% | 2.0 | 2.0 | 0.03% | 2.0 | 2.0 | 0.03% |
| Enc Time[%] | 104% |  |  | 99% |  |  | 97% |  |  |
| Dec Time[%] | 104% |  |  | 89% |  |  | 93% |  |  |

| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| SVT16 | 1.2 | 1.2 | 0.01% | 1.2 | 1.2 | 0.02% | 1.2 | 1.2 | 0.02% |
| SVT12 | 1.3 | 1.3 | 0.01% | 1.3 | 1.3 | 0.02% | 1.3 | 1.3 | 0.02% |
| Overall | 1.2 | 1.2 | 0.01% | 1.3 | 1.3 | 0.02% | 1.3 | 1.3 | 0.02% |
| Enc Time[%] | 105% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 111% |  |  | 91% |  |  | 104% |  |  |

Normal QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 1.01% | 1.07% | 1.09% | 1.19% | 1.03% | 1.09% | 1.14% | 1.17% | 101% | 108% |
| Class H2 |  |  |  |  |  | 1.24% | 1.54% | 0.76% | 101% | 106% |
| Overall | 1.01% | 1.07% | 1.09% | 1.19% | 1.03% | 1.16% | 1.34% | 0.96% | 101% | 107% |

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.19% | 0.14% | #VALUE! | #VALUE! | #VALUE! | 0.14% | -0.11% | -0.40% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.13% | -0.23% | -0.37% | 103% | 104% |
| Overall | -0.19% | 0.14% | #VALUE! | #VALUE! | #VALUE! | 0.14% | -0.17% | -0.39% | 102% | 102% |

Both methods show minor coding loss when history-based Rice parameter derivation and WPP are enabled together. It is worth to note that method 1 is already implemented in the VTM-14.0 base software. Comparing method 2 with method 1, the coding performance is basically the same from the two methods. However, method 2 does not require the synchronization process for the value of StatCoeff for each color component in the preceding CTU row when WPP is enabled, which results in memory saving.

# Conclusion

This contribution proposes two methods to allow history-based Rice parameter derivation when WPP is enabled. The first method proposes that a counter for StatCoeff per color component is saved after the last TU of the first CTU in each CTU row is coded. Starting from the second CTU row, prior to the first TU coding, the synchronization process for Rice parameter derivation is applied to the first CTU for each CTU row where the StatCoeff counter is synchronized with saved StatCoeff counter from the preceding CTU row. The second method proposes that a fixed initial value for the StatCoeff counter is used prior to the coding of first TU in each CTU row. Comparing method 2 and method 1, method 2 does not require the synchronization process for StatCoeff, which results in memory saving, while both methods show the same coding performance. This contribution proposes to adopt method 2.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “Versatile Video Coding Editorial Refinements on Draft 10,” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-T2001_v2, 20th Meeting: by teleconference, 7–16 Oct. 2020.

[2] F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, G. J. Sullivan, Y.-K. Wang, “VVC operation range extensions (Draft 4),” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-W2005, 22nd Meeting, by teleconference, 7–16 July. 2021.

[3] Dmytro Rusanovskyy, Marta Karczewicz, Luong Pham Van, Muhammed Coban “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths,” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-V0106, 22nd Meeting, by teleconference, 20–28 Apr. 2021.

[4] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).