| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 11–15 October 2021 | Document: JVET-X0130-v1 |
| --- | --- |

| Title: | AHG11: Cross-component prediction based on a neural network model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Young-Yoon Lee, Tae Meon Bae, Damian Ruiz Coll, Kalyan Goswami, Alexey Filippov, and Vasily Rufitskiy | Tel: Email: | {yylee, taemeon.bae, druizcoll, kgoswami, afilippov, vrufitskiy} @ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents cross-component prediction based on a neural network model (CCNN). As an alternative mode of intra chroma prediction, the CCNN may further reduce the cross-component redundancy. Based on the AHG11 anchor (VTM11+V0056), the proposed CCNN method reportedly provides the compression performance improvements of -0.86%/0.82%/1.04% and -0.32%/0.35%/0.62% (Y/Cb/Cr) in AI and RA configurations, respectively. The additional test results reportedly present the compression performance improvements of -0.37%/-2.93%/ -2.70% (Y/Cb/Cr) for AI configuration after redistributing the compression performance improvements between luma and chroma using lambda adjustment for the chroma components.

# Introduction

In VVC, the cross-component linear model prediction (CCLM) is exploited to reduce the cross-component redundancy. In CCLM, the chroma samples are predicted based on the reconstructed luma samples based on a linear model. On top of the CCLM, an alternative mode of intra chroma prediction is presented based on a neural network model.

# Proposed method

In this contribution, the neighbouring reconstructed samples of the luma and the chroma components, and the reconstructed luma samples in the co-located area of the current block are used to predict samples of two chroma blocks based on the CCNN. The output of CCNN is processd and compared with other intra chorma predition modes to decide the best mode based on the RDO cost.

## Network architecture

Figure 1 illustrates the CCNN architecture. The CCNN takes the samples in the YUV 420 format as the input. To predict chroma samples of the size (W, H), the inputs of the CCNN are the luma reconstruction samples of the size (2*W, 2*H) and the neighboring reconstructed samples of the luma and the chroma components. The luma component down-sampling is performed by the Bicubic interpolation filter.  The neighboring reconstructed samples of the size (1, H) or (W, 1) are transformed to the size (W, H) based on the duplication along the axis. These input samples are concatenated to form a tensor of the size (W, H, 7). The first, the last, and the skip layers are 2D convolutional layers with kernel size 5. The structure of residual block (ResBlock) consists of two convolutional layers with kernel size 3, where N denotes the number of ResBlock. N is set to 7 in this contribution. The instance normalization (InstanceNorm) and the leaky ReLU activation function are combined to convolutional layers. As for the convolutional layers, the number of feature maps is set as 128.

Fig. 1: CCNN architecture

## Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: RTX 3090 x 2 x 24GB |
|  | Framework: | PyTorch v1.7.1 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 48 |
|  | Batch size: | 512 x 2 |
|  | Loss function: | L2 |
|  | Training time: | 37h |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 32*32 |
|  | Learning rate: | 3.16e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | normalization |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 2.65M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 10.12 |
|  | Multiplay Accumulate (MAC) /pixel | 2.65M |
| Optional |  |  |
|  | Total Conv. Layers | 18 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 32x32, 16x16 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

The anchor is VTM11+V0056 [, ]. CTC results of test 1 and test 2 are provided. Two tests are launched as follows:

Test 1: CCNN is enabled for 16x16 and 32x32 chroma blocks.

Test 2: CCNN is enabled for 16x16 and 32x32 chroma blocks with lambda adjustment.

Test 2a: The anchor with the same lambda adjustment as Test 2.

Table 3. Performance of the proposed method (AI configuration, Test 1)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -1.19% | 0.16% | 0.79% | -1.42% | 0.38% | 0.35% | 478% | 8442% |
| Class A2 | -1.89% | 1.69% | 1.66% | -2.21% | 1.01% | 1.71% | 374% | 6959% |
| Class B | -0.54% | 0.91% | 1.27% | -0.67% | 0.70% | 1.57% | 317% | 6397% |
| Class C | -0.42% | 0.88% | 1.61% | -0.63% | 1.25% | 2.76% | 272% | 5228% |
| Class E | -0.62% | 0.35% | -0.50% | -0.80% | 0.96% | 0.30% | 355% | 9782% |
| Overall | -0.86% | 0.82% | 1.04% | -1.07% | 0.86% | 1.44% | 344% | 6973% |
| Class D | -0.34% | 0.49% | 1.89% | -0.58% | 1.08% | 3.74% | 228% | 6028% |
| Class F | -0.30% | 0.41% | 0.52% | -0.26% | 0.99% | 1.39% | 200% | 4637% |

Table 4. Performance of the proposed method (RA configuration, Test 1)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.38% | -0.37% | 0.10% | -0.47% | 0.24% | -0.03% | 220% | 772% |
| Class A2 | -0.72% | 0.78% | 0.67% | -0.83% | 0.61% | 1.18% | 182% | 494% |
| Class B | -0.23% | 0.47% | 0.95% | -0.27% | 0.31% | 1.24% | 164% | 439% |
| Class C | -0.10% | 0.42% | 0.56% | -0.17% | 0.21% | 1.28% | 166% | 425% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.32% | 0.35% | 0.62% | -0.40% | 0.33% | 0.98% | 178% | 499% |
| Class D | -0.10% | 0.74% | 1.11% | -0.23% | 0.73% | 1.92% | 192% | 734% |
| Class F | -0.11% | 0.23% | 0.03% | -0.09% | 0.85% | 0.69% | 206% | 345% |

Table 5. Performance of the proposed method with lambda adjustment (AI configuration, Test 2)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.56% | -3.71% | -2.44% | -0.66% | -2.83% | -2.62% | 493% | 9518% |
| Class A2 | -0.90% | -2.55% | -2.17% | -1.19% | -3.26% | -2.32% | 383% | 7965% |
| Class B | -0.19% | -3.01% | -2.91% | -0.33% | -3.54% | -3.01% | 324% | 6900% |
| Class C | -0.04% | -2.60% | -2.03% | -0.26% | -3.02% | -1.93% | 274% | 3974% |
| Class E | -0.36% | -2.83% | -4.02% | -0.54% | -2.56% | -3.68% | 360% | 9704% |
| Overall | -0.37% | -2.93% | -2.70% | -0.55% | -3.10% | -2.70% | 351% | 6982% |
| Class D | 0.03% | -2.93% | -1.76% | -0.18% | -3.30% | -0.96% | 231% | 4269% |
| Class F | 0.10% | -1.94% | -1.90% | 0.00% | -2.24% | -1.65% | 203% | 3803% |

Table 6. Performance of the anchor with lambda adjustment (AI configuration, Test 2a)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | 0.66% | -4.04% | -3.32% | 0.79% | -3.44% | -3.10% | 101% | 95% |
| Class A2 | 1.04% | -4.04% | -3.74% | 1.10% | -4.13% | -3.94% | 101% | 103% |
| Class B | 0.38% | -3.87% | -4.13% | 0.37% | -4.38% | -4.60% | 101% | 102% |
| Class C | 0.37% | -3.64% | -3.61% | 0.41% | -4.77% | -4.75% | 101% | 104% |
| Class E | 0.29% | -3.29% | -3.77% | 0.23% | -3.64% | -4.22% | 101% | 99% |
| Overall | 0.52% | -3.78% | -3.76% | 0.55% | -4.14% | -4.21% | 101% | 101% |
| Class D | 0.37% | -3.43% | -3.54% | 0.34% | -4.50% | -4.56% | 101% | 102% |
| Class F | 0.39% | -2.33% | -2.41% | 0.40% | -3.09% | -3.12% | 100% | 94% |

# Conclusions

This contribution presents cross-component prediction model based on a neural network. By applying the proposed method, additional BD rate saving can be achieved. We recommend to study the proposed method in Exploration Experiment (EE) 1.

# References

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[1] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[2] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-V2016, 22nd Meeting, by teleconference, 20–28 Apr. 2021.