| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0133-v1 |
| --- | --- |

| Title: | EE2-related: MV candidate type-based ARMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Han Huang, Vadim Seregin, Chun-Chi Chen, Marta Karczewicz | Email: | {yjchang, hanhuang, vseregin} @qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, an MV candidate type-based ARMC is proposed to reorder the merge candidates in a candidate type, e.g., TMVP, NA-MVP, etc., based on the TM cost values. It is operated before the merge candidate list is constructed. Compared to ECM-2.0 anchor, simulation results of the proposed method are summarized as follows:

RA: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 10x% (DecT)

LB: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 10x% (DecT)

# Introduction

In ECM [1], the regular and TM merge candidate lists are constructed by including the following six types of candidates in order:

- Spatial MVP (SMVP) from spatial neighbour CUs

- Temporal MVP (TMVP) from collocated CUs

- Non-adjacent MVP (NA-MVP) from non-adjacent CUs

- History-based MVP (HMVP) from a FIFO table

- Pairwise average MVP (PA-MVP)

- Zero MVs.

The following four types of subblock merge candidates are used in order to form the subblock merge candidate list, where the first entry is SbTMVP, and the other entries are Affine merge candidates:

- SbTMVP

- Inherited affine merge candidates (I-AffineMVP) that extrapolated from the CPMVs of the neighbour CUs

- Constructed affine merge candidates (C-AffineMVP) that are derived using the translational MVs of the neighbour CUs

- Zero MVs

In 23rd JVET meeting, adaptive reordering of merge candidates (ARMC) with template matching (TM) in JVET-W0090 [2] was agreed to be integrated into ECM-2.0. The ARMC is applied to regular merge candidate list, TM merge candidate list, and affine merge candidate list (subblock merge candidate list excluding the SbTMVP candidate). After a merge candidate list is constructed, merge candidates are divided into several groups. The group size is set to 5 for regular merge mode and TM merge mode. The group size is set to 3 for affine merge mode. Merge candidates in each group are reordered ascendingly according to the cost values based on TM. For simplification, merge candidates in the last but not the first group are not reordered. For the TM merge mode, ARMC is applied before the TM refinement process.

# Proposal

The MV candidate type-based ARMC is proposed to reorder the merge candidates in a candidate type, e.g., TMVP, NA-MVP, etc., based on the TM cost values (the same as the ARMC TM cost used in ECM2.0). M candidates are then selected out of the reordered candidates in the candidate type when constructing the merge candidate list, where M varies depending on candidate types (for example, M = 1 for TMVP type and M = 9 for NA-MVP type). Some candidate types add more merge candidates to perform the reordering and the selection. For example, TMVP candidates derived from different collocated positions including the bottom-right and center positions currently used in ECM2.0 are reordered together, and 1 candidate is selected out of these reordered TMVP candidates. Some candidate types, such as SMVP, may not be reordered by the proposed ARMC.

# Simulation results

The MV candidate type-based ARMC was implemented on top of the ECM-2.0 and tested using the common test conditions as defined in JVET-W2017 [3]. Table 1 shows the overall BD-rate performance.

Table 1. Performance of the MV candidate type-based ARMC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.18% | -0.19% | -0.19% | 100% | 107% |
| Class C | -0.24% | -0.33% | 0.00% | 97% | 108% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.13% | -0.18% | 0.09% | 99% | 108% |
| Class F | -0.14% | -0.20% | -0.19% | 98% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.26% | -0.35% | -0.23% | 101% | 117% |
| Class E | 0.02% | -0.05% | -0.25% | 102% | 110% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.10% | 0.26% | 101% | 118% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

The contribution proposes an MV candidate type-based ARMC to reorder the merge candidates in a candidate type based on the TM cost values before the merge candidate list is constructed. It is recommended to test the proposed method in the next EE2.

# References

- M. Coban, F. Le Léannec, J. Ström, “Algorithm description of Enhanced Compression Model 2 (ECM 2),” JVET-W2025, Jul. 2021.

- N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang, “EE2-3.1/EE2-3.2: Adaptive Reordering of Merge Candidates with Template/Bilateral Matching,” JVET-W0090, Jul. 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, Jul. 2021.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).