| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0134-v1 |
| --- | --- |

| Title: | Non-EE2: On the number of TM merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | {yjchang, vseregin} @qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, the number of TM merge candidates is changed from 4 to 7. Compared to ECM-2.0 anchor, simulation results are summarized as follows:

RA: -0.11% (Y), -0.17% (U), -0.09% (V), 103% (EncT), 100% (DecT)

LB: -0.14% (Y), -0.32% (U), -0.27% (V), 102% (EncT), 101% (DecT)

# Proposal

The number of TM merge candidates is changed from 4 to 7 by modifying the parameter TM_MRG_MAX_NUM_CANDS in ECM-2.0 software as follows:

static const int    TM_MRG_MAX_NUM_CANDS =           7;

# Simulation results

The proposed change was tested using the common test conditions as defined in JVET-W2017. Table 1 shows the overall BD-rate performance.

Table 1. Performance of the number of TM merge candidates set as 7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.24% | -0.12% | 103% | 100% |
| Class A2 | -0.11% | -0.20% | -0.12% | 103% | 100% |
| Class B | -0.11% | -0.20% | -0.13% | 103% | 100% |
| Class C | -0.12% | -0.05% | 0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.17% | -0.09% | 103% | 100% |
| Class D | -0.05% | -0.16% | -0.07% | 102% | 99% |
| Class F | -0.04% | 0.10% | -0.06% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.22% | -0.30% | 103% | 101% |
| Class C | -0.20% | -0.28% | -0.14% | 101% | 102% |
| Class E | -0.01% | -0.53% | -0.39% | 102% | 99% |
| Overall | -0.14% | -0.32% | -0.27% | 102% | 101% |
| Class D | 0.01% | 0.20% | -0.12% | 101% | 101% |
| Class F | -0.18% | -0.23% | -0.01% | 101% | 99% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).