| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0134-v2 |
| --- | --- |

| Title: | EE2-related: On the number of TM merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | {yjchang, vseregin} @qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, the number of TM merge candidates is changed from 4 to 7. Compared to ECM-2.0 anchor, simulation results are summarized as follows:

RA: -0.11% (Y), -0.17% (U), -0.09% (V), 103% (EncT), 100% (DecT)

LB: -0.14% (Y), -0.32% (U), -0.27% (V), 102% (EncT), 101% (DecT)

# Proposal

The number of TM merge candidates is changed from 4 to 7 by modifying the parameter TM_MRG_MAX_NUM_CANDS in ECM-2.0 software as follows:

static const int    TM_MRG_MAX_NUM_CANDS =           7;

# Simulation results

The proposed change was tested using the common test conditions as defined in JVET-W2017. Table 1 shows the overall BD-rate performance.

Table 1. Performance of the number of TM merge candidates set as 7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.24% | -0.12% | 103% | 100% |
| Class A2 | -0.11% | -0.20% | -0.12% | 103% | 100% |
| Class B | -0.11% | -0.20% | -0.13% | 103% | 100% |
| Class C | -0.12% | -0.05% | 0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.17% | -0.09% | 103% | 100% |
| Class D | -0.05% | -0.16% | -0.07% | 102% | 99% |
| Class F | -0.04% | 0.10% | -0.06% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.22% | -0.30% | 103% | 101% |
| Class C | -0.20% | -0.28% | -0.14% | 101% | 102% |
| Class E | -0.01% | -0.53% | -0.39% | 102% | 99% |
| Overall | -0.14% | -0.32% | -0.27% | 102% | 101% |
| Class D | 0.01% | 0.20% | -0.12% | 101% | 101% |
| Class F | -0.18% | -0.23% | -0.01% | 101% | 99% |

# Patent rights declaration(s)

Qualcomm Inc. does not have any current or pending patent rights relating to the technology described in this contribution.