| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0137-v1 |
| --- | --- |

| Title: | AHG8 and AHG10: On derivation of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes methods for determining the signalled values sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 which are used to decode coefficient values in the tools adopted in VVC v2 described in JVET-V0121 and JVET-W0046.

The methods described here operate on the original image data and therefore can be used before coefficients are coded. This allows the values of the signalled parameters to be determined using image data for the first frame of a sequence unlike the current implementations in VTM which rely on QP and bit depth based formulas for the first frame of a sequence.

Gains are reported as follows

For the sh_reverse_last_sig_coeff_flag modification:

| Low QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | 0.01% | -0.01% | 0.00% | 0.02% | 0.00% | 0.03% | 0.01% | 0.00% | 0.01% |
| HLG | -0.01% | -0.02% | -0.02% | -0.03% | -0.13% | -0.06% | -0.03% | -0.10% | -0.05% |
| SVT-12 | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 0.01% | -0.01% | -0.01% | -0.01% |
| SVT-16 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

| Mid QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | -0.04% | 0.02% | -0.08% |  |  |  |  |  |  |
| HLG | -0.01% | 0.00% | 0.00% |  |  |  |  |  |  |
| SVT-12 | -0.05% | -0.03% | -0.04% |  |  |  |  |  |  |
| SVT-16 | 0.00% | 0.00% | 0.00% |  |  |  |  |  |  |

For the sh_ts_residual_coding_rice_idx_minus1 modification:

| Low QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| SCC | -0.39% | -0.38% | -0.37% |  |  |  |  |  |  |
| PQ | -0.01% | -0.02% | -0.03% | 0.01% | -0.02% | 0.00% | -0.01% | -0.01% | -0.01% |
| HLG | 0.00% | -0.02% | -0.02% | 0.00% | -0.01% | -0.02% | -0.01% | 0.01% | 0.00% |
| SVT-12 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| SVT-16 | 0.00% | 0.01% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

| Very | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low QP | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | -0.07% | -0.31% | -0.31% |  |  |  |  |  |  |
| HLG | -0.40% | -0.75% | -0.73% |  |  |  |  |  |  |
| SVT-12 | -0.01% | -0.00% | -0.01% |  |  |  |  |  |  |
| SVT-16 | 0.00% | 0.00% | 0.00% |  |  |  |  |  |  |

# Introduction

VVC version 2 [1] contains a number of modifications for high bit depth coding. Amongst them are changes to the coding of the last significant coefficient position [2] and the Rice parameter used for transform skip residual coding [3]. Both of these modifications signal a value in the bitstream to control the decoding of coefficient data. In both cases, the current VTM implementations of these modifications derive the signalled values when encoding the previous I-frame. For the first frame of a sequence, or the only frame if using a still profile, the signalled value is determined using a formula which uses only the bit depth and the slice QP. Such an approach cannot therefore adapt for the actual frame being processed if it has characteristics atypical of those used to determine the formula currently implemented. In addition the current approach is not sensitive to scene changes if the characteristics of the current I-frame are different from that of the previous I-frame.

The approach taken in this proposal is to introduce encoder only (non-normative) modifications to determine a suitable value for the signalled values. For both values, this modification represents a pre-encoder phase which operates only on the original image data and thus determines a value for the current frame being encoded not the next frame. Thus values can be signalled for the first or only frame of a sequence that are not purely based on bit depth and slice QP.

# Background

sh_reverse_last_sig_coeff_flag is signalled in the slice header and, if set, the position of the signalling of the last significant coefficient in each transform block in the slice is reversed, i.e. the position is calculated relative to the bottom right of the transform block rather than the top left. If not present in the bitstream sh_reverse_last_sig_coeff_flag is inferred to be 0 which is equivalent to VVC version 1.

sh_ts_residual_coding_rice_idx_minus1 is signalled in the slice header. The Rice parameter used when coding TSRC coefficients is sh_ts_residual_coding_rice_idx_minus1 + 1. If not present in the bitstream sh_ts_residual_coding_rice_idx_minus1 is inferred to be 0 and thus the Rice parameter used is 1 which is equivalent to VVC version 1.

A mechanism exists within VTM which allows the signalled value of  sh_ts_residual_coding_rice_idx_minus1 to be reset to zero and is triggered if certain conditions related to IBC are detected. This mechanism is still operational with this modification.

# Approach

The approach taken for deriving values for both sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 is to estimate the residual for each pixel in the original image. This estimate is calculated as the minimum absolute difference between the pixel and the pixels above and to the left:

residualx,y = min( abs( pixelx,y  –  pixelx-1,y ), abs( pixelx,y – pixelx,y-1 ) )

When calculating sh_ts_residual_coding_rice_idx_minus1, if residualx,y  is equal to zero a counter ignored is incremented and no further action is done for this pixel. This extra step has been implemented as TSRC is typically used for screen content which can feature large areas of zero residuals with small zones with very large residuals and special treatment is required.

This residual is then converted into a bin in a histogram as follows:

binx,y = max(0, round( ( residualx,y > 0 ? log2( residualx,y ) : 0 ) * 6 ) – qp_offset)

Where qp_offset is a function of the slice QP and bit depth. Thus although this technique still uses the slice QP and bit depth in the calculations these are then modified by the actual pixel data. qp_offset is required as the optimal value of both signalled parameters relies on the quantization of the coefficients as well as the original pixel data. Therefore the value of sh_ts_residual_coding_rice_idx_minus1 will tend to increase as QPs decrease.

The final stage of the common algorithm is to increment the bin in the histogram

hist[ binx,y  ] ++

At this point the techniques for computing sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 diverge.

sh_reverse_last_sig_coeff_flag is set to 1 if the value of hist[0] is smaller than the sum of all other hist[] values as this indicates that a large number of coefficients should be expected when coding transform blocks.

sh_ts_residual_coding_rice_idx_minus1 is calculated in a similar manner to the shift value n in the technique proposed for RRC [4] and involves picking the bins for the 33.3%, 50%, and 66.6% centiles and computing an average before clipping the value to 7 which is the maximum value that may be signalled.

Finally if ignored is greater than the total number of entries in the histogram the value of sh_ts_residual_coding_rice_idx_minus1 is clipped to bit_depth – 9. Clipping the Rice parameter limits the cost of coding zero residuals when large numbers of such residuals are anticipated.

# Results

## sh_ts_residual_coding_rice_idx_minus1

Using the low QP section of the HBD CTC [5], the following results were obtained.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.02% | -0.03% | -0.01% | -0.02% | -0.03% | 100% | 100% |
| PQ422 | -0.02% | -0.04% | -0.03% | -0.02% | -0.04% | -0.03% | 101% | 101% |
| Overall | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | -0.03% | 100% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | -0.02% | 0.00% | 0.00% | -0.02% | 0.00% | 102% | 101% |
| PQ422 | 0.02% | -0.01% | -0.01% | 0.01% | -0.01% | -0.01% | 99% | 99% |
| Overall | 0.01% | -0.02% | 0.00% | 0.00% | -0.02% | 0.00% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 98% | 99% |
| PQ422 | 0.00% | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% | 99% | 99% |
| Overall | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | 99% | 99% |
| Overall PQ | 0.00% | -0.02% | -0.01% | -0.01% | -0.02% | -0.01% | 100% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.02% | -0.02% | 100% | 100% |
| HLG422 | 0.00% | -0.01% | -0.01% | 99% | 100% |
| Overall | 0.00% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | -0.01% | 99% | 98% |
| HLG422 | 0.00% | -0.01% | -0.02% | 100% | 99% |
| Overall | 0.00% | -0.01% | -0.02% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | 0.00% | 100% | 99% |
| HLG422 | -0.01% | 0.01% | 0.00% | 99% | 99% |
| Overall | -0.01% | 0.01% | 0.00% | 99% | 99% |
| Overall HLG | 0.00% | -0.01% | -0.01% | 99% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.01% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.00% | 0.00% | 98% | 98% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 98% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 99% | 100% |

Whilst the gains are very small, it should be recognised that the TSRC modification offered only small gains in this configuration. In addition it can be seen from the timings given in the result that the pre-encode stage takes no more time than the previous method embedded in the encoder itself.

Additional tests have been completed using a slightly modified anchor of VTM 14.0. The aim of the modifications is to test still picture performance. Thus the signalled value of sh_ts_residual_coding_rice_idx_minus1 is always computed using the slice QP and bit depth based formula implemented in VTM. The results for normal QPs are very similar to those given above which indicates that the formula works well for the QPs and content within the CTC.

Further testing outside the standard QP range was also completed with a range of QPs from 2 to -23 for 12 bit and -23 to -48 for 16 bit. It can be expected that at the lowest QPs in these ranges the number of coefficients coded with TSRC will be significantly increased. This should also highlight the effects of correctly deriving the values of sh_ts_residual_coding_rice_idx_minus1. The results are given below:

| HDR PQ |  |  | AI (QP -23 to 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.31% | -0.31% | -0.06% | -0.29% | -0.29% | 101% | 99% |
| PQ422 | -0.15% | -0.43% | -0.43% | -0.14% | -0.43% | -0.43% | 101% | 99% |
| Overall | -0.11% | -0.37% | -0.37% | -0.10% | -0.36% | -0.36% | 101% | 99% |

| HDR HLG |  |  | AI (QP -23 to 2) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.37% | -0.76% | -0.74% | 102% | 98% |
| HLG422 | -0.44% | -0.75% | -0.72% | 102% | 99% |
| Overall | -0.40% | -0.75% | -0.73% | 102% | 98% |

| SVT RGB |  |  | AI (QP-48 to -23 for SVT-16, QP -23 to 2 for SVT-12) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | -0.01% | 0.00% | -0.01% | 100% | 99% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 99% |

The results indicate that outside the QP range defined in the CTC, gains are achieved for both PQ and HLG content. The current formula used in VTM for calculating sh_ts_residual_coding_rice_idx_minus1 for still pictures is less suitable for content coded at QPs outside the current range defined in the CTC.

The following are results for the screen content section of the HBD CTC.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.74% | -0.71% | -0.68% | 96% | 98% |
| RGB 4:4:4 - TGM 720p | -0.05% | -0.06% | -0.07% | 99% | 101% |
| Overall | -0.39% | -0.38% | -0.37% | 98% | 99% |

Screen content tends to favour TSRC and therefore higher gains are evident. The gains for TGM 720p content are lower as it is generally older, easier to compress, and activates the IBC reset mechanism described earlier which is common to the proposed modification and VTM.

Results for the standard QP section of the HBD CTC are given a separate spreadsheet. The gains / losses are uniformly zero as at these QPs as sh_ts_residual_coding_rice_idx_minus1 is always zero for both VTM14.0 and the modification.

Lossless results are not provided as TSRC is not used in lossless coding.

## sh_reverse_last_sig_coeff_flag

Using the low QP section of the HBD CTC [5], the following results were obtained.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 100% | 100% |
| PQ422 | 0.01% | -0.02% | -0.01% | 0.01% | -0.02% | -0.01% | 101% | 101% |
| Overall | 0.01% | -0.01% | 0.00% | 0.01% | -0.01% | 0.00% | 100% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.02% | 0.05% | 0.03% | 0.02% | 0.05% | 101% | 100% |
| PQ422 | 0.02% | -0.02% | 0.01% | 0.02% | -0.03% | 0.01% | 100% | 100% |
| Overall | 0.02% | 0.00% | 0.03% | 0.02% | 0.00% | 0.03% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.04% | 0.04% | 0.02% | 0.04% | 0.04% | 99% | 98% |
| PQ422 | 0.00% | -0.04% | -0.02% | 0.00% | -0.04% | -0.02% | 100% | 99% |
| Overall | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 100% | 99% |
| Overall PQ | 0.02% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 100% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.02% | -0.03% | 101% | 100% |
| HLG422 | 0.00% | -0.02% | -0.01% | 99% | 99% |
| Overall | -0.01% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.09% | -0.04% | 98% | 97% |
| HLG422 | -0.05% | -0.17% | -0.08% | 100% | 101% |
| Overall | -0.03% | -0.13% | -0.06% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.03% | -0.05% | -0.07% | 100% | 99% |
| HLG422 | -0.02% | -0.14% | -0.03% | 99% | 99% |
| Overall | -0.03% | -0.10% | -0.05% | 99% | 99% |
| Overall HLG | -0.02% | -0.08% | -0.04% | 99% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| SVT12 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| SVT12 | -0.04% | -0.03% | -0.03% | 101% | 101% |
| Overall | -0.02% | -0.01% | -0.02% | 100% | 101% |
| Overall RGB | -0.01% | -0.01% | -0.01% | 100% | 100% |

Although the gains for the low QP range are very small and there are also occasional small losses, the modification is targeted at the first frame of a sequence where at the present in VTM the value of sh_reverse_last_sig_coeff_flag swaps from 0 to 1 at QP 12 for all content. Using a modified anchor of VTM which uses this default switch for all frames in an AI sequence to simulate still frames, the following results were obtained.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | 0.00% | -0.02% | -0.01% | 0.00% | -0.02% | 99% | 99% |
| PQ422 | -0.02% | 0.01% | -0.03% | -0.02% | 0.02% | -0.01% | 101% | 100% |
| Overall | -0.01% | 0.01% | -0.03% | -0.02% | 0.01% | -0.01% | 100% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| HLG422 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 98% | 99% |
| SVT12 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 98% |

Again the gains are small when comparing against the modified anchor, but the losses are removed. The new modification is working at least as well for single frames as VTM 14.0.

In order to properly test the modification it is necessary to test over a wider range of QPs. As the current algorithm in VTM switches sh_reverse_last_sig_coeff_flag at QP 12 which is at the extreme end of the low QP section of the CTC, additional tests were carried over a range of QP 2 to QP 27. This is also a range of QP which might be considered a more practical operating point for VVC v2. The results are given below.

| HDR PQ |  |  | AI (QP 2 to 27) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.02% | -0.08% | -0.03% | -0.02% | -0.07% | 100% | 99% |
| PQ422 | -0.05% | 0.06% | -0.08% | -0.05% | 0.08% | -0.04% | 101% | 100% |
| Overall | -0.04% | 0.02% | -0.08% | -0.04% | 0.03% | -0.05% | 101% | 100% |

| HDR HLG |  |  | AI (QP 2 to 27) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.01% | 0.01% | 99% | 99% |
| HLG422 | -0.01% | -0.02% | -0.01% | 101% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 99% |

| SVT RGB |  |  | AI (QP 2 to 27 for SVT-12, QP -18 to 7 for SVT-16) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| SVT12 | -0.05% | -0.03% | -0.04% | 98% | 97% |
| Overall | -0.03% | -0.02% | -0.02% | 98% | 98% |

Small gains are apparent in these results for both PQ and SVT-12. The results suggest that the current switch at QP 12 is suitable for the current HLG content. Even though small, the gains for PQ should be considered in terms of the -0.02%/-0.04%/-0.08% gains reported in the V meeting CE.

Results for the standard QP section of the HBD CTC are given a separate spreadsheet. The gains / losses are uniformly zero as at these QPs as sh_reverse_last_sig_coeff_flag is never set for both VTM14.0 and the modification as the number of coefficients in a transform block is typically small.

# Conclusions

This contribution has described non-normative pre-encoder methods for determining the values of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 which are signalled in the slice header. These methods allow these values to be derived and used for the same frame, whereas the current implementation in VTM derives a value to be used in the next frame.

Currently for the first frame of a sequence default formulas based only on the slice QP and bit depth are used to determine the values to signal. As the characteristics of content can vary, there is a need for a better method for determining the values of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 for the first frame. This is particularly true for still frame profiles.

Gains for both methods have been reported, particularly outside the current low QP section of the high bit depth CTC.

It is proposed that the pre-encoder methods described in this document are adopted into VTM for the determination of the signalled values of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 for the first frame of a sequence in the Main 12 4:4:4 and Main 16 4:4:4 families of profiles, including but not limited to still picture profiles.

# Patent rights declaration(s)

Sony Group Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “VVC operation range extensions (Draft 4)”, JVET-W2005, F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, G. J. Sullivan, Y.-K. Wang

[2] “AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-V0121, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang

[3] “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-V0054, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang

[4] “AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, JVET-W-0093, A. Browne, S. Keating, K. Sharman

[5] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu