| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0143-v04 |
| --- | --- |

| Title: | AHG10: VTM Encoder Changes for ALF Usage with Subpicture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Zare Alireza Aminlou Antti Hallapuro Miska M. Hannuksela | Tel: Email: | alireza.zare@nokia.com alireza.aminlou@nokia.com antti.hallapuro@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document proposes several VTM software changes which enable the use of ALF in applications where extraction and merging of subpictures occur, e.g., in subpicture-based viewport adaptive streaming (VAS). Currently, ALF should be disabled in such applications when encoding using VTM software. The proposed changes include: 1) making the maximum number of ALF APSs per picture configurable, 2) allowing both luma and chroma in case of having one ALF APS, and 3) introducing an offset value to the ALF APS identifiers. This offset along with limiting the maximum number of ALF APSs per picture allows controlling the allocation of ALF APS identifiers in sub-streams which may be merged into a single stream. Moreover, the contribution proposes a VTM encoder configuration that enables a constant joint chroma coding residual (JCCR) sign, since JCCR sign is a picture-level parameter and hence needs to be the same in all encoded sub-streams which may be merged into a single stream. Furthermore, the contribution proposes several changes to the VTM subpicture merger tool to improve the performance of merging operation. Finally, the contribution illustrates the benefit of enabling ALF in a subpicture-base VAS use case.

# Introduction

VVC ALF comprises luma ALF, chroma ALF and cross-component ALF (CC-ALF). The filter coefficients are either pre-defined and fixed in both encoder and decoder or adaptively signaled on a picture basis using adaptation parameter set (APS). The ALF APS filters are decided at the encoding time. The number of ALF APSs per picture is limited to 8 by the current version of VVC. An ALF APS persists for one or more complete picture(s) until the next ALF APS having the same identifier.

The ALF usage is limited in applications that employ subpicture extraction from multiple source bitstreams and merging those subpictures into a destination bitstream, due to the limitations above. For example, in subpicture-based viewport-adaptive streaming (VAS) of 360° video, several bitstreams are encoded, each with different quality or resolution and with a relatively high number of subpictures (e.g., 8 to. 96). A player determines which subpictures are streamed and merged into a bitstream to be decoded. Merging the subpictures coded using independent and non-coordinated encoders into a single bitstream may cause having ALF APSs with the same identifiers but with different content. This results in an ALF APS signalling conflict and leads to a VVC non-confirming bitstream. Therefore, in applications using subpicture extraction and merging, the ALF APS signaling of different sub-bitstreams needs to be managed at the encoding or merging stage.

The naïve solution is to disable ALF (i.e., both fixed ALF and ALF APS) at the cost of substantial coding efficiency loss. The other solution is to keep the pre-defined fixed ALF enabled, while the ALF APS usage is disabled at the cost of discarding the benefit of parameter adaptation by ALF APS. The more efficient solution in terms of coding efficiency is to enable ALF APS filters, in addition to fixed ALF, in a restricted manner such that any aggregation of sub-bitstreams results in a confirming bitstream without any ALF APS identifiers conflict. For example, in Figure 1 a subpicture partitioning of CMP content (3×3 grid in each CMP face) is shown at two low- and high-quality versions. For each version, a set of 4 ALF APSs having a unique range is allocated, such that high-quality version allocates ALF APSs of range {0 .. 3} and low-quality version allocates ALF APSs of range {4 .. 7}.

| Figure 1. a) CMP subpicture partitioning (High-quality version), 4 ALF APSs are allocated in the range of {0 .. 3} | b) CMP subpicture partitioning (Low-quality version), 4 ALF APSs are allocated in the range of {4 .. 7} |
| --- | --- |

As another example, each version of a picture may be divided into several subpicture groups which each allocate a unique range of ALF APSs across different versions of subpicture groups. In Figure 2, each picture version is divided into 4 subpicture group (resulting in 8 subpicture groups in total in both versions), for which an ALF APS with unique identifier is allocated.

| Figure 2. a) CMP subpicture partitioning (High-quality version), black dashed lines show ALF APS subpicture group boundaries | b) CMP subpicture partitioning (Low-quality version), black dashed lines show ALF APS subpicture group boundaries |
| --- | --- |

This contribution proposes several fixes and changes to VTM encoder software to enable managing ALF APSs and allow aggregation of different sub-bitstreams coded separately into a single VVC confirming bitstream without raising any conflicts in ALF APS identifiers.

# VTM encoder modifications

## Configurable maximum number of ALF APSs per picture

The maximum number of ALF APS allowed in a picture is defined as an encoder configuration parameter. The defined configuration parameter may take a value in the range of {0, …, 8}.

## Defining an offset value to the ALF APS identifiers

To mange the allocation of ALF APS identifiers to sub-streams which are merged into a single bitstream at the switching points, an offset value to ALF APS identifier is defined. The allocation performs in a such manner that a unique range of ALF APS identifiers is assigned to each sub-stream.

## Allowing both luma and chroma in a single ALF APS

In the current version of VTM encoder, one ALF APS is reserved for chroma. Hence, when the number of ALF APSs is limited to one, that ALF APS is assigned to chroma and luma has no ALF APS. The VTM encoder is fixed to allow both luma and chroma in a single ALF APS.

## Constant joint chroma coding residual (JCCR) sign flag

An encoder configuration parameter is defined to enable constant/fixed sign for joint chroma coding residual flag.

## VTM encoder code changes

Configurable maximum number of ALF APSs per picture:

Under the NOKIA_ALF_APS_CHANGES macro, there are 8 changes to define two encoder configuration parameters MaxNumALFAPS and ConstantJointCbCrSignFlag, related to the sections 2.1 and 2.4 above, respectively. The new encoder parameters are defined as below.

#======== ALF APS ====================== MaxNumALFAPS : 1 # The maximum number of ALF APSs, up to 8 APSs ConstantJointCbCrSignFlag : 0 # 0: positive and negative sign, 1: Constant positive sign

In several places, the already defined macro ALF_CTB_MAX_NUM_APS was replaced with the newly defined encoder configuration parameter using m_encCfg->getMaxNumALFAPS().

- In EncAdaptiveLoopFilter.cpp, in ALFProcess function

// derive filter (chroma) #if NOKIA_ALF_APS_CHANGES if (!(m_encCfg->getMaxNumALFAPS() == 0) && isChromaEnabled(cs.pcv->chrFormat)) // Find ALF parameters for chroma if ALF APS is enabled #else if (isChromaEnabled(cs.pcv->chrFormat)) #endif

- In EncAdaptiveLoopFilter.cpp, in alfEncoderCtb function

#if NOKIA_ALF_APS_CHANGES if ((m_encCfg->getMaxNumALFAPS()==0) && ((numTemporalAps + useNewFilter) > 0)) { continue; } #endif

- In EncAdaptiveLoopFilter.cpp, in alfEncoderCtb function

//chroma #if NOKIA_ALF_APS_CHANGES if (!(m_encCfg->getMaxNumALFAPS() == 0) && isChromaEnabled(cs.pcv->chrFormat)) // Find ALF parameters for chroma if ALF APS is enabled. #else if (isChromaEnabled(cs.pcv->chrFormat)) #endif { m_alfParamTemp = alfParamNewFiltersBest;

Defining an offset value to the ALF APS identifiers:

- In VLCWriter.cpp, in codeAPS function

#if NOKIA_ALF_APS_ID_OFFSET if (pcAPS->getAPSType() == ALF_APS) { WRITE_CODE((pcAPS->getAPSId() + NOKIA_ALF_APS_ID_OFFSET ) % 8, 5, "adaptation_parameter_set_id"); } else #endif WRITE_CODE(pcAPS->getAPSId(), 5, "adaptation_parameter_set_id");

- In VLCWriter.cpp, in codeSliceHeader function

for (int i = 0; i < pcSlice->getNumAlfApsIdsLuma(); i++) { #if NOKIA_ALF_APS_ID_OFFSET WRITE_CODE((apsId[i] + NOKIA_ALF_APS_ID_OFFSET ) % 8, 3, "sh_alf_aps_id_luma[i]"); #else WRITE_CODE(apsId[i], 3, "sh_alf_aps_id_luma[i]"); #endif

- In VLCWriter.cpp, in codeSliceHeader function

if (alfChromaIdc) { #if NOKIA_ALF_APS_ID_OFFSET WRITE_CODE((pcSlice->getAlfApsIdChroma() + NOKIA_ALF_APS_ID_OFFSET ) % 8, 3, "sh_alf_aps_id_chroma"); #else WRITE_CODE(pcSlice->getAlfApsIdChroma(), 3, "sh_alf_aps_id_chroma"); #endif } if (pcSlice->getSPS()->getCCALFEnabledFlag())

- In VLCWriter.cpp, in codeSliceHeader function

if (filterParam.ccAlfFilterEnabled[COMPONENT_Cb - 1]) { // write CC ALF Cb APS ID #if NOKIA_ALF_APS_ID_OFFSET WRITE_CODE((pcSlice->getCcAlfCbApsId() + NOKIA_ALF_APS_ID_OFFSET ) % 8, 3, "sh_alf_cc_cb_aps_id"); #else WRITE_CODE(pcSlice->getCcAlfCbApsId(), 3, "sh_alf_cc_cb_aps_id"); #endif } // Cr WRITE_FLAG(filterParam.ccAlfFilterEnabled[COMPONENT_Cr - 1] ? 1 : 0, "sh_alf_cc_cr_enabled_flag"); if (filterParam.ccAlfFilterEnabled[COMPONENT_Cr - 1]) { // write CC ALF Cr APS ID #if NOKIA_ALF_APS_ID_OFFSET WRITE_CODE((pcSlice->getCcAlfCrApsId() + NOKIA_ALF_APS_ID_OFFSET ) % 8, 3, "sh_alf_cc_cr_aps_id"); #else WRITE_CODE(pcSlice->getCcAlfCrApsId(), 3, "sh_alf_cc_cr_aps_id"); #endif

Allowing both luma and chroma in a single ALF APS:

- In EncAdaptiveLoopFilter.cpp, in alfEncoderCtb

#if NOKIA_ALLOW_BOTH_LUMA_AND_CHROMA_IN_ONE_ALF_APS #if NOKIA_ALF_APS_CHANGES if (numTemporalAps + useNewFilter > std::min(ALF_CTB_MAX_NUM_APS - 1, m_encCfg->getMaxNumALFAPS())) #else if (numTemporalAps + useNewFilter > std::min(7, ALF_CTB_MAX_NUM_APS)) #endif #else if (numTemporalAps + useNewFilter >= ALF_CTB_MAX_NUM_APS) #endif { continue; }

- In EncAdaptiveLoopFilter.cpp, in alfEncoderCtb

#if NOKIA_ALLOW_BOTH_LUMA_AND_CHROMA_IN_ONE_ALF_APS #if NOKIA_ALF_APS_CHANGES int counter = m_encCfg->getMaxNumALFAPS(); #else int counter = ALF_CTB_MAX_NUM_APS; #endif while ((newApsIdChroma < 0) && ((counter--))) #else while (newApsIdChroma < 0) #endif { curId--; if (curId < 0) { #if NOKIA_ALF_APS_CHANGES curId = m_encCfg->getMaxNumALFAPS() - 1; #else curId = ALF_CTB_MAX_NUM_APS - 1; #endif

Allowing constant joint chroma coding residual (JCCR) sign flag, when it is enabled from encoder configuration:

- In EncGOP.cpp, in compressGOP function

#if NOKIA_ALF_APS_CHANGES if (m_pcCfg->getConstantJointCbCrSignFlag()) pcPic->cs->picHeader->setJointCbCrSignFlag(true); else m_pcSliceEncoder->setJointCbCrModes(*pcPic->cs, Position(0, 0), pcPic->cs->area.lumaSize()); #else m_pcSliceEncoder->setJointCbCrModes(*pcPic->cs, Position(0, 0), pcPic->cs->area.lumaSize()); #endif

New macros defined in TypeDef.h:

#define NOKIA_ALF_APS_CHANGES 1 #define NOKIA_ALF_APS_ID_OFFSET 0 #define NOKIA_ALLOW_BOTH_LUMA_AND_CHROMA_IN_ONE_ALF_APS 1

# Merger modification

Currently the merger tool does not support input streams containing subpictures by checking if subpicture information is present in the input sub-streams. In case of using the bitstream extractor to extract subpictures from a picture containing multiple subpictures, the extracted streams contain subpicture information even if they contain a single subpicture. The merger tool was complemented to facilitate merging of input sub-streams in which subpicture information exists while there is one subpicture within a picture.

In addition, the merger was improved to avoid writing of redundant ALF APSs with the same identifier and content in the same picture, to achieve a better rate-distortion performance.

Currently, the merger tool only checks to prevent a non-confirming case where two APSs with the same identifier having different content.

Merger code changes:

- In SubpicMergeApp.cpp, in generateMergedStreamSPSes function, allowing merging of input streams containing 1 subpicture within a picture.

#if NOKIA_MERGER_FIXES CHECK(subpic.psManager.getSPS(spsId)->getSubPicInfoPresentFlag() && subpic.psManager.getSPS(spsId)->getNumSubPics() > 1, "Input streams containing subpictures not supported"); #else CHECK(subpic.psManager.getSPS(spsId)->getSubPicInfoPresentFlag(), "Input streams containing subpictures not supported") #endif

- In SubpicMergeApp.cpp, in generateMergedPic function, to avoid writing of redundant APSs with the same identifier and content.

#if NOKIA_MERGER_FIXES std::vector<int> usedApsIds; #endif for (auto &subpic : *m_subpics) { auto apsIdIt = subpic.apsIds.begin(); for (auto &nalu : subpic.nalus) { if (nalu.m_nalUnitType == NAL_UNIT_PREFIX_APS) { #if NOKIA_MERGER_FIXES if (std::find(usedApsIds.begin(), usedApsIds.end(), apsIdIt->first) == usedApsIds.end()) { #endif OutputNALUnit naluOut(NAL_UNIT_PREFIX_APS, nalu.m_nuhLayerId, nalu.m_temporalId); hlsWriter.setBitstream( &naluOut.m_Bitstream ); hlsWriter.codeAPS(subpic.psManager.getAPS(apsIdIt->first, apsIdIt->second)); accessUnit.push_back(new NALUnitEBSP(naluOut)); #if NOKIA_MERGER_FIXES usedApsIds.push_back(apsIdIt->first); } #endif apsIdIt++; } } }

New macros defined in TypeDef.h:

#define NOKIA_MERGER_FIXES 1

# Bitstream Extractor modification

Currently in the PPS rewriting process in the bitstream extractor tool, the scaling window parameters are set regardless of whether the scaling window is enabled or not in the input streams. A condition was defined to allow the scaling window parameters to be defined only if the scaling window is enabled in the input streams.

Extractor code changes:

- In BitstreamExtractorApp.cpp, in function xRewritePPS, where prior to setting the scaling window.

#if NOKIA_EXTRACTOR_FIXES if (sourcePPS.getScalingWindow().getWindowEnabledFlag()) { #endif int subWidthC = SPS::getWinUnitX(sourceSPS.getChromaFormatIdc()); int subHeightC = SPS::getWinUnitY(sourceSPS.getChromaFormatIdc());

New macros defined in TypeDef.h:

#define NOKIA_EXTRACTOR_FIXES 1

# Results

The BD-rate results of VTM encoder with varying values for the maximum number of ALF APSs per picture are reported in the tables below. The reference method is the case where ALF tool including both fixed ALF and ALF APS is deactivated (i.e., ALF and CCALF encoder configuration parameters are set to zero). The experiments were conducted using both AI and RA configurations of JVET common test conditions for SDR video [1]. The encoding was performed using VVC reference software (VTM) version 14.0 [3], in which the proposed changes and fixes were integrated.

The BD-rate results of AI configuration are shown in Table 1 to Table 5, for the cases of maximum number of ALF APSs per picture equals to 0, 1, 2, 7, and 8, respectively.

Table 1. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.53% | 0.28% | 0.28% | 107% | 106% |
| Class A2 | -1.90% | 0.15% | 0.15% | 104% | 110% |
| Class B | -1.62% | 0.11% | 0.11% | 105% | 105% |
| Class C | -1.54% | 0.06% | 0.06% | 108% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.64% | 0.14% | 0.14% | 106% | 105% |
| Class D | -1.22% | 0.06% | 0.06% | 110% | 100% |
| Class F | -1.08% | 0.09% | 0.09% | 105% | 103% |

Table 2. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -8.15% | -10.71% | 56% | 82% |
| Class A2 | -2.37% | -11.41% | -15.69% | 55% | 83% |
| Class B | -1.86% | -15.64% | -12.99% | 55% | 83% |
| Class C | -1.84% | -6.67% | -7.04% | 55% | 82% |
| Class E |  |  |  |  |  |
| Overall | -1.93% | -10.90% | -11.49% | 55% | 83% |
| Class D | -1.11% | -5.43% | -4.62% | 56% | 72% |
| Class F | -2.49% | -8.74% | -9.12% | 61% | 82% |

Table 3. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -8.15% | -10.71% | 57% | 82% |
| Class A2 | -2.37% | -11.41% | -15.69% | 55% | 83% |
| Class B | -1.86% | -15.64% | -12.99% | 55% | 82% |
| Class C | -1.84% | -6.67% | -7.04% | 55% | 80% |
| Class E |  |  |  |  |  |
| Overall | -1.93% | -10.90% | -11.49% | 55% | 82% |
| Class D | -1.11% | -5.43% | -4.62% | 56% | 71% |
| Class F | -2.49% | -8.74% | -9.12% | 61% | 81% |

Table 4. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 7

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 7) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -8.15% | -10.71% | 56% | 82% |
| Class A2 | -2.37% | -11.41% | -15.69% | 55% | 83% |
| Class B | -1.86% | -15.64% | -12.99% | 55% | 83% |
| Class C | -1.84% | -6.67% | -7.04% | 55% | 81% |
| Class E |  |  |  |  |  |
| Overall | -1.93% | -10.90% | -11.49% | 55% | 82% |
| Class D | -1.11% | -5.43% | -4.62% | 56% | 73% |
| Class F | -2.49% | -8.74% | -9.12% | 61% | 82% |

Table 5. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 8

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 8) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -8.15% | -10.71% | 56% | 81% |
| Class A2 | -2.37% | -11.41% | -15.69% | 55% | 83% |
| Class B | -1.86% | -15.64% | -12.99% | 55% | 82% |
| Class C | -1.84% | -6.67% | -7.04% | 55% | 80% |
| Class E |  |  |  |  |  |
| Overall | -1.93% | -10.90% | -11.49% | 55% | 82% |
| Class D | -1.11% | -5.43% | -4.62% | 56% | 72% |
| Class F | -2.49% | -8.74% | -9.12% | 60% | 82% |

Table 6. All Intra overall results

|  | All Intra (Overall) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| # ALF APS = 0 | -1.64% | 0.14% | 0.14% | 106% | 105% |
| # ALF APS = 1 | -1.93% | -10.90% | -11.49% | 55% | 83% |
| # ALF APS = 2 | -1.93% | -10.90% | -11.49% | 55% | 82% |
| # ALF APS = 7 | -1.93% | -10.90% | -11.49% | 55% | 82% |
| # ALF APS = 8 | -1.93% | -10.90% | -11.49% | 55% | 82% |

The 17-frame BD-rate results of RA configuration are shown in Table 7 to Table 11, for the cases of maximum number of ALF APSs per picture equals to 0, 1, 2, 7, and 8, respectively. The full-frame results will be updated later.

Table 7. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 0 (17-frame results)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.53% | 0.26% | 0.56% | 106% | 108% |
| Class A2 | -2.55% | 0.34% | 0.31% | 107% | 104% |
| Class B | -1.49% | 0.20% | 0.44% | 111% | 102% |
| Class C | -1.00% | 0.55% | 0.08% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.58% | 0.33% | 0.34% | 110% | 103% |
| Class D | -0.32% | 0.34% | 0.07% | 113% | 103% |
| Class F | -0.85% | -0.30% | -1.36% | 112% | 99% |

Table 8. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 1  (17-frame results)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.22% | -3.80% | -8.47% | 105% | 115% |
| Class A2 | -3.52% | -11.95% | -16.71% | 103% | 114% |
| Class B | -2.88% | -16.64% | -11.76% | 110% | 112% |
| Class C | -1.89% | -5.83% | -6.45% | 113% | 97% |
| Class E |  |  |  |  |  |
| Overall | -2.61% | -10.25% | -10.67% | 108% | 109% |
| Class D | -2.68% | -3.44% | -2.10% | 114% | 109% |
| Class F | -1.87% | -5.85% | -5.43% | 112% | 110% |

Table 9. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 2  (17-frame results)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.23% | -3.77% | -8.51% | 105% | 118% |
| Class A2 | -3.60% | -12.07% | -16.79% | 104% | 114% |
| Class B | -2.99% | -16.59% | -11.84% | 110% | 110% |
| Class C | -1.94% | -5.98% | -6.51% | 113% | 98% |
| Class E |  |  |  |  |  |
| Overall | -2.68% | -10.29% | -10.74% | 109% | 109% |
| Class D | -2.77% | -3.72% | -2.43% | 115% | 102% |
| Class F | -1.88% | -5.79% | -5.37% | 112% | 106% |

Table 10. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 7  (17-frame results)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 7) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.25% | -3.78% | -8.52% | 105% | 126% |
| Class A2 | -3.65% | -12.02% | -16.89% | 104% | 120% |
| Class B | -3.06% | -16.75% | -12.03% | 110% | 117% |
| Class C | -2.00% | -6.07% | -6.65% | 113% | 105% |
| Class E |  |  |  |  |  |
| Overall | -2.73% | -10.36% | -10.86% | 109% | 116% |
| Class D | -2.84% | -3.87% | -2.60% | 117% | 109% |
| Class F | -1.95% | -5.96% | -5.64% | 112% | 109% |

Table 11. Anchor: ALF off, Test: maximum number of ALF APSs per picture is 8  (17-frame results)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Anchor (No ALF) Vs. Test (# ALF APS = 8) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.26% | -3.73% | -8.51% | 105% | 113% |
| Class A2 | -3.66% | -12.02% | -16.89% | 104% | 122% |
| Class B | -3.06% | -16.78% | -12.02% | 111% | 104% |
| Class C | -1.98% | -6.04% | -6.58% | 113% | 103% |
| Class E |  |  |  |  |  |
| Overall | -2.73% | -10.35% | -10.84% | 109% | 109% |
| Class D | -2.84% | -3.87% | -2.60% | 116% | 108% |
| Class F | -1.95% | -5.96% | -5.64% | 112% | 99% |

Table 12. 17-frame Random Access overall results

|  | Random Access (Overall) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| # ALF APS = 0 | -1.58% | 0.33% | 0.34% | 110% | 103% |
| # ALF APS = 1 | -2.61% | -10.25% | -10.67% | 108% | 109% |
| # ALF APS = 2 | -2.68% | -10.29% | -10.74% | 109% | 109% |
| # ALF APS = 7 | -2.73% | -10.36% | -10.86% | 109% | 116% |
| # ALF APS = 8 | -2.73% | -10.35% | -10.84% | 109% | 109% |

# 360-degree VAS use case

In this section the benefit of enabling ALF in a subpicture-base VAS use case is discussed. We study the performance of several methods for allocation of ALF APSs across subpictures which each is coded at two quality levels [2]. The allocation designs guarantee that any combination of high- and low-quality subpictures, which form a 360° viewing orientation, can be merged into a single VVC confirming bitstream, while no coding coordination with respect to sharing of ALF parameters among subpictures is required. The proposed ALF-APS allocation methods exhibit different trade-off preferences between the coding efficiency and encoding parallelization granularity of 360° content.

The experiments were conducted with 10 video clips from the JVET test content for 360° video coding, which are originally in equirectangular projection (ERP) format. Prior to encoding, the source ERP sequences were converted to cubemap projection (CMP) format with a face size of 1536×1536 using JVET 360Lib-10.0 software. Each CMP face was partitioned into a uniform subpicture grid of 3×3, resulting in 54 subpictures per picture. The encoding was performed using VVC reference software (VTM) version 12.0 [3]. The simulations were conducted using Random Access configuration of JVET common test condition for 360° video coding  with 33 frames from each sequence. The decoding refresh type was set to instantaneous decoding refresh (IDR) picture with period of 32. For comparing the performance of different ALF-APS allocation methods, the reference method is the case where ALF tool including both pre-defined fixed ALF and ALF APS is deactivated (i.e., ALF and CCALF encoder configuration parameters are set to zero). In all test scenarios including the reference method, the joint chroma coding residual (JCCR) sign flag was set to a constant sign of positive, and luma mapping with chroma scaling (LMCS) and Adaptive maximum binary tree size (AMaxBT) tools were disabled. Setting the JCCR sign flag to positive required a modification of VTM, while the other two tools were switched off from VTM encoder configuration.

We analyze the performance of high- and low-quality content separately, without rendering the actual viewing orientation for different viewing directions. The high-quality and low-quality subpictures are coded using quantization parameter (QP) set of 22, 25, 27, 30 and 29, 32, 34, 37, respectively. The YUV BD-rate performance is reported using a weighted average with weights of 6, 1, and 1 for Y, U, and V components, respectively. For the low-quality streams, only the YUV BD-rate result is reported since component-wise performance is nicely aligned with that of high-quality version.

## ALF APS disabled

In this method ALF is enabled, while the ALF APS usage is disabled. Hence, only the pre-defined fixed ALF is allowed. The VTM encoder was modified to deactivate the usage of ALF APS.

## Picture-based ALF-APS allocation

In a picture level ALF-APS allocation, the number of ALF APSs are divided between low- and high-quality content such that each version has its own unique range of ALF APS identifiers. For example in this study, for each quality version 4 ALF APSs out of 8 are allocated, where high-quality ALF APS identifiers range is [0..3] and that of low-quality content is [4..7].

## Subpicture-group-base ALF-APS allocation

In this method, the subpictures within a picture are divided into several ALF-APS subpicture groups, for each of which a certain number of unique ALF APS identifiers is assigned. Figure 3 shows the subpicture partitioning considered in this study. The figure shows a 360° content in cubemap projection (CMP) format, where the solid black lines are the CMP boundaries, solid white lines are the individual subpictures boundaries, and black dashed lines are the ALF-APS subpicture-group boundaries. Each picture is divided into 4 ALF-APS subpicture groups as depicted in Figure 3, where for each of subpicture groups only 1 ALF APS with a unique identifier in the range of [0..7] is assigned.

Figure 3. CMP subpicture partitioning and ALF-APS subpicture groups. Solid black line: CMP boundaries, Solid white line: subpicture boundaries, and black dashed line: ALF-APS subpicture group boundaries.

## Results

The BD-rate performance of three methods described above are illustrated in Table 13 to Table 15, respectively. The three studied methods present different trade-off preferences between coding efficiency and parallelization granularity. When ALF APS is disabled, a high parallelization granularity of subpicture level can be achieved; however, its coding efficiency is low compared to the other two methods. The picture-based allocation method provides a slightly higher coding efficiency, with a lower parallelization granularity of picture level. The subpicture-group-based allocation technique represents similar coding efficiency with that of picture-based allocation, while a finer parallelization granularity of subpicture group can be achieved.

In Table 13, the reason for observing loss in chroma components is because the fixed ALF filters are applied only on luma component and enabling the fixed ALF comes with overall bitrate increase without having any impact on the quality of chroma components.

Table 13. BD-rate performance of ALF APS disabled case (fixed ALF enabled, and ALF APS disabled)

| Test clips | Y (HQ) | U (HQ) | V (HQ) | YUV (HQ) | YUV (LQ) |
| --- | --- | --- | --- | --- | --- |
| Balboa | -2.3% | 0.4% | 0.0% | -1.7% | -0.9% |
| BranCastle | -1.5% | 0.1% | -0.4% | -1.1% | -1.3% |
| Broadway | -2.8% | 0.0% | 0.4% | -2.0% | -1.3% |
| Landing | -1.4% | -0.1% | 0.3% | -1.0% | -0.9% |
| Harbor | -1.7% | 0.3% | 0.3% | -1.2% | -1.2% |
| SkateboardInLot | -1.4% | 0.5% | 0.7% | -0.9% | -0.4% |
| Trolley | -1.5% | 0.1% | 0.1% | -1.1% | -1.4% |
| KiteFlite | -1.6% | 0.3% | 0.2% | -1.2% | -1.0% |
| ChairliftRide | -0.9% | 0.3% | 0.3% | -0.6% | -0.3% |
| Gaslamp | -2.0% | 0.3% | 0.4% | -1.4% | -1.6% |
| Overall | -1.7% | 0.2% | 0.2% | -1.2% | -1.0% |

Table 14. BD-rate performance of picture-based ALF-APS allocation (fixed ALF enabled, and 4-ALF-APS allocation per picture)

| Test clips | Y (HQ) | U (HQ) | V (HQ) | YUV (HQ) | YUV (LQ) |
| --- | --- | --- | --- | --- | --- |
| Balboa | -3.2% | -2.7% | -2.4% | -3.0% | -2.1% |
| BranCastle | -2.9% | -6.0% | -7.9% | -3.9% | -3.1% |
| Broadway | -3.8% | -4.4% | -6.3% | -4.1% | -3.0% |
| Landing | -2.8% | -4.9% | -4.6% | -3.3% | -2.9% |
| Harbor | -2.1% | -3.4% | -3.4% | -2.4% | -2.2% |
| SkateboardInLot | -2.2% | -5.4% | -3.9% | -2.8% | -1.9% |
| Trolley | -2.0% | -2.8% | -1.7% | -2.0% | -2.6% |
| KiteFlite | -2.2% | -4.2% | -4.2% | -2.7% | -2.5% |
| ChairliftRide | -1.6% | -3.5% | -2.5% | -2.0% | -1.5% |
| Gaslamp | -2.6% | -4.2% | -4.7% | -3.1% | -3.3% |
| Overall | -2.5% | -4.1% | -4.2% | -2.9% | -2.5% |

Table 15. BD-rate performance of subpicture-group-base ALF-APS allocation (fixed ALF enabled, and 1-ALF-APS allocation per subpicture group)

| Test clips | Y (HQ) | U (HQ) | V (HQ) | YUV (HQ) | YUV (LQ) |
| --- | --- | --- | --- | --- | --- |
| Balboa | -3.0% | -2.3% | -2.9% | -2.9% | -1.9% |
| BranCastle | -2.8% | -5.9% | -8.0% | -3.8% | -2.9% |
| Broadway | -3.6% | -4.6% | -6.3% | -4.0% | -2.7% |
| Landing | -2.7% | -4.8% | -5.0% | -3.2% | -2.7% |
| Harbor | -2.1% | -3.2% | -4.0% | -2.5% | -2.0% |
| SkateboardInLot | -2.0% | -4.6% | -2.8% | -2.4% | -1.4% |
| Trolley | -1.9% | -3.0% | -1.7% | -2.0% | -2.5% |
| KiteFlite | -2.2% | -4.4% | -4.4% | -2.7% | -2.5% |
| ChairliftRide | -1.3% | -3.7% | -3.0% | -1.8% | -1.3% |
| Gaslamp | -2.5% | -3.8% | -4.7% | -2.9% | -2.9% |
| Overall | -2.4% | -4.0% | -4.3% | -2.8% | -2.3% |

Table 16. Overall results, Anchor: both fixed ALF and ALF APS deactivated

# Conclusions

It is suggested to adopt the proposed software changes into VTM reference software.

# References

JVET-T2010, VTM common test conditions and software reference configurations for SDR video

Zare, A. Aminlou, and M. M. Hannuksela, “VVC Adaptive Loop Filter Optimization for Subpicture-based Viewport-adaptive Streaming,” IEEE MMSP 2021 conference, Oct. 2021, Tampere, Finland.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).