| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0147 |
| --- | --- |

| Title: | EE2-related: intra mode derivation based on TIMD for GPM inter/intra |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Seung-hwan Kim Jaehyun Lim (LGE) 19, Yangjae-daero 11-gil, Seocho-gu, Seoul, Republic of Korea Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: Tel: Email: | +02 10 8505 0793 hm.jang@lge.com +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | LG Electronics Inc. KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes TIMD based intra mode derivation and its application without signaling to indicate which intra mode is used on geometry intra-region for the geometry partitioning mode with inter and intra(GPM inter/intra) which was proposed by JVET-W0110[1]. It also restricts that intra with intra combination is applied. With the proposed method, GPM inter/intra do not need pre-defined IPM candidates which is adaptively selected along with GPM angle and reuse conventional TIMD method. The suggested GPM with inter/intra method show further coding performance against EE2-3.2 a/b test with similar encoding complexity.

Simulation result of the proposed method over ECM-2.0(Tool-off test) is summarized as follow.

RA : TBD / TBD/ TBD

LDB : TBD/ TBD / TBD

LDP : TBD / TBD / TBD

In v1, only intermediate results are attached in the simulation results section.

Introduction

The geometry partitioning mode with inter and intra(GPM inter/intra) was proposed in JVET-W0110 with further compression performance for GPM. In this contribution, for each GPM-separated region whether applying intra or inter mode is determined with a flag. if the intra prediction mode is applied for a GPM region from the flag, it is need to signal index to derive intra mode among the pre-defined intra mode list adaptively selected based on GPM partitioning information.

Proposal

The proposed method includes two aspects against EE2-3.2 test as below.

- One intra prediction mode is derived based on conventional TIMD method instead of pred-defined IPM list and the derived TIMD mode is applied without syntax signaling.

- Restrict allowing intra with intra combination on GPM Block.

Simulation Results

The proposed method was implemented on the top of ECM-2.0 [2], which are defined as the anchor software in EE2 for tool-off test [3] under CTC[4].

|  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | TBD | TBD | TBD | TBD | TBD |  |  |  |  |  |  |  |  |  |  |
| A2 | TBD | TBD | TBD | TBD | TBD |  |  |  |  |  |  |  |  |  |  |
| B | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| C | -0.08% | -0.14% | -0.07% | 102% | 102% | -0.18% | -0.67% | -0.45% | 102% | 101% | -0.78% | -2.29% | -1.46% | 106% | 98% |
| E |  |  |  |  |  | 0.02% | -0.49% | -1.01% | 103% | 102% | -0.11% | -1.23% | -0.88% | 106% | 99% |
| Overall | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| D | 0.01% | -0.29% | 0.00% | 102% | 101% | -0.02% | -0.23% | -0.98% | 102% | 101% | -0.33% | -1.53% | -1.46% | 106% | 101% |
| F | -0.01% | -0.18% | -0.17% | 102% | 101% | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Figure 1. Simulation result over ECM-2.0 (tool-off)

Conclusion

It is proposed modified GPM intra/inter blending mode with TIMD based intra mode derivation. It is observed further compression benefit over EE2-3.2 test with negligible encoder/decoder complexity changes. Therefore It is suggested to consider adopting the proposed method in the next ECM software.

Reference

- Y. Kidani, H. Kato, and K. Kawamura, “AHG12: GPM with inter and intra prediction,” JVET-W0110, by teleconference, 7–16 July 2021.

- ECM-2.0: https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/tree/ECM-2.0

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-W2024, by teleconference, 7–16 July 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

Patent rights declaration(s)

LG Electronics Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

_______________________________