| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0150-v1 |
| --- | --- |

| Title: | AHG12: Enhanced sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Ning Yan, C.-W. Kuo, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Email: | {xiaoyuxiu, yiwenchen, yanning03, cheweikuo, jhuhong-jheng chenwei06, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, two enhancements are proposed to further improve the performance of the sign prediction tool in ECM-2.0, including 1) extension of the sign prediction to LFNST mode; 2) adaptive selection of transform coefficient positions for the sign prediction by maximizing their impacts on reconstructed border samples. Compared to the ECM-2.0 anchors, simulation results reportedly show that the proposed enhancements provide average {Y, U, V}BD-rate changes of {-0.31%, -0.31%, -0.41%} and {-0.xx%, -0.xx%, -0.xx%} for AI and RA configurations, respectively. The corresponding enc/dec run-time are 107%/107% for AI and 101%/102% for RA.

# Introduction

In ECM-2.0 [1], sign prediction tool is applied to improve the coding efficiency of the signs of non-zero transform coefficients. The technique aims at estimating the signs of the coefficients in one transform block (TB) from its neighboring reconstructed samples and codes the difference between the estimated signs and true signs. Specifically, when it is applied to one TB, at most M (which is set to 8) non-zero coefficients in the raster-scan order of the top-left coding group (CG) of the TB are selected for the sign prediction. Both encoder and decoder need to generate the reconstructed samples (also know as hypothesis) at the top and left borders of the TB based on different sign combinations of the selected coefficients. The best sign combination, which minimizes a cost measurement of the spatial discontinuity at the TB’s top and left borders, is used as the sign predictors.

Additionally, to avoid conducting multiple inverse transforms, the existing sign prediction in the ECM-2.0 applies one template-based hypothesis generation scheme, where each template is a set of reconstructed border samples of one TB. The templates are pre-calculated at both encoder and decoder, based on the inverse transform of some unit coefficient matrices, each of which is generated by setting one specific coefficient in the matrix to one while all the others are kept as zeros.

At last, in the ECM-2.0, the sign prediction is only enabled to the TBs where only primary transforms, including DCT-2 and MTS transform kernels, are applied. For the TBs that apply low-frequency non-separable transform (LFNST), the sign prediction is always skipped.

# Proposal

In this contribution, the following two enhancements are proposed to further improve the coding efficiency of the sign prediction tool in the ECM-2.0:

- It is proposed to extend the sign prediction to the LFNST mode. Additionally, to achieve a better gain/complexity trade-off, a maximum of 4 (i.e., M = 4) coefficients in the top-left CG are allowed to be predicted for one LFNST TB.

- Instead of always selecting the first M coefficients in raster-scan order, it is proposed to choose the non-zero transform coefficients in the top-left CG for the sign prediction, based on their impacts on the quality of the reconstructed border samples of one TB. Specifically, to select the transform coefficients, the following cost function is applied which measures the L1-norm energy of the variations caused by one transform coefficient on the reconstructed border samples:

cost=Ci,j∙k=0N-1Ti,jk

where Ci,j represents the transform coefficient at coordinate i,j in the TB; Ti,j represents the template that corresponds to the transform coefficient, which consists of N reconstructed samples at the top and left border of the TB. Based on the above cost function, the encoder/decoder will select the signs of the coefficients with the M largest costs to be predicted.

# Simulation results

The proposed methods were implemented on top of the reference software ECM-2.0 [2] and tested using the common test conditions as defined in [3].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.35% | -0.47% | 109% | 108% |
| Class A2 | -0.29% | -0.27% | -0.36% | 106% | 107% |
| Class B | -0.26% | -0.25% | -0.33% | 105% | 108% |
| Class C | -0.25% | -0.28% | -0.37% | 107% | 108% |
| Class E | -0.50% | -0.46% | -0.59% | 108% | 103% |
| Overall | -0.31% | -0.31% | -0.41% | 107% | 107% |
| Class D | -0.18% | -0.05% | -0.20% | 104% | 108% |
| Class F | -0.20% | -0.31% | -0.18% | 110% | 108% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.34% | -0.24% | 101% | 104% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.23% | -0.09% | -0.06% | 101% | 102% |
| Class C | -0.17% | -0.27% | -0.12% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.11% | -0.40% | -0.10% | 99% | 98% |
| Class F | -0.12% | -0.27% | -0.13% | 101% | 102% |

# References

M. Coban, F. Le Léannec, J. Ström, “Algorithm description of Enhanced Compression Model 2 (ECM2)”, JVET-W2025, July, 2021, Teleconference.

ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July, 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).