| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0156-v1 |
| --- | --- |

| Title: | Non-EE2: Fix for histogram of gradients derivation in DIMD mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao Vadim Seregin Marta Karczewicz | Tel: Email: | kemicao@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a fix for deriving an index for the diagonal intra mode direction while generating histogram of gradients in DIMD mode. It was implemented on top of ECM-2.0 software, it reportedly provides Y, U, V-BD rate reduction with encoder and decoder runtime as follows:

AI: {} runtime: {}

RA: {} runtime: {}

# Introduction

Derivation of Histogram of Gradients (HoG) feature is applied in Decoder Intra Mode Derivation (DIMD). HoG is a vector of length 67 with each element denoting the magnitude of corresponding direction and HoG indicates a tangent of the intra prediction angle. For a current coding unit (CU), the HoG is computed using left, top, and top-left reconstructed neighbors.

For each sample, the gradient of its horizontal direction, GHor, and its vertical direction, GVer, are utilized to compute tan⁡(θ), which can determine angle θ. Angle θ denotes the direction of the gradient for this sample as shown in Fig.1.

Figure 1 Gradient for a sample point

In ECM, tanθ=GVer/GHor, and then tanθ is modified by multiplying 65536. After that, it’s compared with the values in a table, angTable={0, 2048, 4096, 6144, 8192, 12288, 16384, 20480, 24576, 28672, 32768, 36864, 40960, 47104, 53248, 59392, 65536} of length 17.

The index of the value in the table which is closest to the modified tanθ will be picked as the direction of the gradient.

In the implementation, this is done by looping through the table to derive index idx of the intra direction:

For ( int i = 0; i < 17; i++ )

{

if ( tan(θ) < angTable[i] )

{

idx = tan(θ) - angTable[i - 1] < angTable[i] - tan(θ) ? i-1 : i;

break;

}

}

When θ=45, the tanθ is equal 65536 and the condition in the loop is not true for the largest value, so a wrong idx is derived.

The same problem was identified in JVET-X0139.

# Proposed fix

The condition check inside the loop is changed as highlighted:

For ( int i = 1; i < 17; i++ )

{

if ( tan(θ) <= angTable[i] )

{

idx = tan(θ) - angTable[i - 1] < angTable[i] - tan(θ) ? i-1 : i;

break;

}

}

After the proposed fix, the index 16 of θ=45 is correctly derived.

# Results

The proposed method is implemented on top of ECM-2.0. Test results are summarized in the below table.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class C | -0.02% | -0.04% | 0.01% | 100% | 99% |
| Class E | -0.01% | -0.07% | 0.10% | 102% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.03% | -0.02% | 101% | 104% |
| Class F | -0.06% | -0.15% | -0.18% | 101% | 101% |
| Class TGM | -0.01% | -0.03% | -0.02% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.13% | 0.17% | 102% | 101% |
| Class C | -0.02% | 0.22% | 0.07% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.45% | 0.01% | 100% | 100% |
| Class F | -0.02% | -0.11% | -0.14% | 100% | 99% |
| Class TGM | -0.04% | -0.06% | -0.04% | 99% | 99% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).