| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0159-v1 |
| --- | --- |

| Title: | Cross-check report of JVET-X0129: AHG8: Independent Rice Parameter Derivation for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-X0129 proposal on modification of Rice parameters derivation for HBD/HBR coding. Proponents of the JVET-X0129 propose to remove cross-TB RRC history propagation and utilize first Rice decoded coefficient to be used in the Rice parameter derivation template. Implementation and simulation results have been cross-checked. Implementation and simulation results (for completed QP points) are found matching to those presented by the proponent.

# Introduction

This document reports results of the cross-check for JVET-X0129 Independent Rice Parameter Derivation for high bit depth and high bit rate extensions.  The following work was conducted:

- Cross-check of simulation results for LowQP and lossless.

- Cross-check of software implementation.

# Cross-check results

In terms of coding performance, results, reported by proponent were replicated during the cross-check.

Tables 1 present simulation results HBD/HBR CTC in Low QP coding configuration.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ422 | 0.07% | 0.03% | 0.03% | 0.07% | 0.04% | 0.03% | 100% | 102% |
| Overall | 0.09% | 0.04% | 0.05% | 0.09% | 0.04% | 0.04% | 99% | 101% |
|  | 0.08% | 0.04% | 0.04% | 0.08% | 0.04% | 0.04% | 100% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14 |  |  |  |  |  |
| PQ444 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ422 | 0.00% | -0.02% | 0.03% | 0.00% | -0.02% | 0.02% | 102% | 102% |
| Overall | 0.03% | 0.03% | 0.01% | 0.02% | 0.02% | 0.01% | 101% | 100% |
|  | 0.01% | 0.00% | 0.02% | 0.01% | 0.00% | 0.02% | 102% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14 |  |  |  |  |  |
| PQ444 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ422 | 0.02% | 0.02% | 0.02% | 0.02% | 0.02% | 0.02% | 99% | 100% |
| Overall | 0.03% | 0.04% | 0.01% | 0.03% | 0.04% | 0.01% | 101% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.09% | 0.04% | 0.04% | 100% | 98% |
| HLG422 | 0.09% | 0.04% | 0.03% | 100% | 99% |
| Overall | 0.09% | 0.04% | 0.03% | 100% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | 100% | 101% |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.21% | 0.16% | 0.16% | 98% | 96% |
| SVT12 | 0.09% | 0.03% | 0.03% | 98% | 98% |
| Overall | 0.15% | 0.10% | 0.10% | 98% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | 100.00% | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| SVT12 | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Overall | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |

Tables 3 present simulation results HBD/HBR CTC in lossless coding configuration.

| PQ | All Intra |  |  |  | Low delay B |  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |  | ratio |  | bit-rate savings |  | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  |  | VTM-14.0 | proposed |  |  | VTM-14.0 | proposed |  |
| PQ444 |  |  |  |  |  |  |  |  |  |  |  |
| PQ422 |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time |  |  |  |  |  |  |  |  |  |  |  |
| HLG | All Intra |  |  |  | Low delay B |  |  |  | Random Access |  |  |
|  | ratio |  |  | bit-rate savings | ratio |  |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  |  | VTM-14.0 | proposed |  |  | VTM-14.0 | proposed |  |
| HLG444 |  |  |  |  |  |  |  |  |  |  |  |
| HLG422 |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time |  |  |  |  |  |  |  |  |  |  |  |

| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  | VTM-14.0 | proposed |  |
| SVT16 |  |  |  |  |  |  |  |  |  |
| SVT12 |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |

# References

[1] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu.