| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0171 (v1) |
| --- | --- |

| Title: | Crosscheck of JVET-X0063 (AHG10: Deblocking filter setting for VTM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu 8910 University Center Lane San Diego, CA 92122, USA | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the cross-check results for AHG10: Deblocking filter setting for VTM as reported in JVET-X0063. The cross-check results match the coding performance reported by the proponent.

# Results

The source code for JVET-X0063 was provided by the proponent. The software is built based on VTM-14.0. The crosscheck results are illustrated as following tables. The detailed results could be seen in the attached excel files.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -0.76% | 0.26% | 100% | 99% |
| Class A2 | -0.77% | -0.83% | -0.72% | 100% | 100% |
| Class B | -0.89% | -0.66% | -0.58% | 100% | 100% |
| Class C | -0.56% | -0.99% | -1.01% | 100% | 99% |
| Class E | -0.56% | -0.15% | 0.15% | 100% | 100% |
| Overall | -0.74% | -0.69% | -0.44% | 100% | 100% |
| Class D | -0.48% | -0.73% | -0.71% | 100% | 101% |
| Class F | -0.39% | -1.06% | -1.49% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.19% | -0.01% | 100% | 101% |
| Class A2 | -0.42% | -0.27% | -0.36% | 100% | 100% |
| Class B | -0.39% | -0.67% | -0.54% | 100% | 100% |
| Class C | -0.23% | -1.00% | -0.94% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.58% | -0.50% | 100% | 100% |
| Class D | 0.03% | -0.91% | -0.62% | 100% | 100% |
| Class F | -0.19% | -1.37% | -1.74% | 100% | 100% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-X0063.