| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0181_v2 |
| --- | --- |

| Title: | Crosscheck of JVET-X0091 (Non-EE2: On TMVP improvement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-X0091. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-X0091 [1] is proposed to modify the derivation process of TMVP merge candidate. There are two methods in the proposal. In the method 1, the reference picture of the TMVP merge candidate is selected from any one of reference pictures in the reference picture list instead of being fixed to the picture with index 0. The selected reference picture is the one whose scaling factor is the closet to 1. In the method 2, when constructing the merge candidate list of non-subblock merge modes, the TMVP is conditionally added to the list according to the motion information of neighboring spatial blocks. Moreover, after the list is constructed, the TMVP is refined using the template matching method.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The coding results of method 1 and 1+2 under the ECM CTC are summarized in Table 1-2.

Table 1: Performance of method 1 over ECM-2.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | -0.04% | 95% | 97% |
| Class A2 | -0.03% | -0.09% | 0.00% | 96% | 98% |
| Class B | -0.03% | 0.02% | -0.01% | 93% | 96% |
| Class C | -0.09% | -0.03% | 0.04% | 92% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.01% | 0.00% | 94% | 96% |
| Class D | -0.09% | -0.05% | 0.12% | 92% | 93% |
| Class F | 0.00% | 0.07% | -0.09% | 93% | 93% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.20% | -0.42% | 94% | 97% |
| Class C | -0.71% | -1.06% | -0.97% | 92% | 94% |
| Class E | 0.17% | -0.82% | -0.24% | 92% | 95% |
| Overall | -0.28% | -0.64% | -0.56% | 93% | 95% |
| Class D | -0.75% | -1.08% | -1.69% | 91% | 91% |
| Class F | -0.36% | -0.67% | -0.75% | 92% | 93% |

Table 2: Performance of method 1+2 over ECM-2.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.09% | -0.15% | 94% | 98% |
| Class A2 | -0.11% | -0.12% | -0.11% | 95% | 99% |
| Class B | -0.13% | -0.11% | -0.18% | 92% | 97% |
| Class C | -0.22% | -0.15% | -0.24% | 91% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.12% | -0.18% | 93% | 97% |
| Class D | -0.20% | -0.36% | -0.03% | 91% | 95% |
| Class F | -0.08% | -0.11% | -0.12% | 92% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.39% | -0.41% | 95% | 97% |
| Class C | -0.70% | -0.89% | -0.57% | 92% | 94% |
| Class E | -0.01% | 0.10% | -0.54% | 92% | 96% |
| Overall | -0.34% | -0.44% | -0.50% | 93% | 96% |
| Class D | -0.79% | -1.02% | -1.05% | 92% | 92% |
| Class F | -0.50% | -0.58% | -0.38% | 93% | 95% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

R.-L. Liao, J. Chen, Y. Ye, X. Li, “Non-EE2: On TMVP improvement”, JVET-X0091, October 2021.