| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0186-v1 |
| --- | --- |

| Title: | Subjective Quality Assessment of VVC and HEVC for 8K Video Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Charles Bonnineau, Wassim Hamidouche Naty Sidaty, Jean-François Travers, Olivier Déforges | Email: | charles.bonnineau@tdf.fr |

_____________________________

# Abstract

This contribution provides a comparative subjective quality evaluation between the VTM-11 reference software (VVC) [1] and the HM-16.20 reference software (HEVC) [2] for 8K resolution videos. In addition, we evaluate the perceived quality improvement offered by 8K over UHD 4K resolution. The compression performance of both VVC and HEVC standards has been conducted in random access (RA) coding configuration. This test was performed on six video scenes with various spatial and temporal characteristics collected from two different sources: the Japanese organization ITE and Fraunhofer HHI [3]. Objective measurements using PSNR, MS-SSIM [4], and VMAF [5] metrics are provided. Subjectively, VVC offers an average of around 41% of bitrate reduction over HEVC for the same visual quality. In addition, a significant visual difference between uncompressed 4K and 8K has been noticed for most tested scenes.

# Subjective test methodology

As mention above, six video scenes were selected for this subjective test. An example frame of the used video sequences is given in Figure 1. Based on these six uncompressed (raw) selected 8K video sequences (scenes), ten processed video sequences (PVSs) are generated per scene:

- one 8K (7680x4320) hidden reference uncompressed

video.

- one 4K (4320x2160) uncompressed video. In that case, the source signal is first downscaled to 4K and then rescaled to 8K by using the Lanczos3 [6] filter provided by ffmpeg2 for both operations.

- 8K video encoded at four bitrates with HEVC.

- 8K video encoded at four bitrates with VVC

In total, 60 video sequences are evaluated in this study.

| - BodeMuseum | - Festival2 | - JapaneseMaple |
| --- | --- | --- |
| - LayeredKimono | - OberbaumSpree | - SteelPlant |

# Figure 1:  Snapshots of the used 8K video sequences

For each scene, the test points are obtained using different fixed QP values. To cover a wide range of visual quality, we determined the highest bitrate value considering the transparency, i.e., the bitrate for which degradation starts to appear, as the highest bitrate point for each sequence. Also, the bitrates were carefully selected so that each bitrate Ri is approximately half of the next bitrate Ri+1 and each VVC bitrate RiVVC is equal to the corresponding HEVC bitrate RiHEVC for i∈{1,2,3,4}.

The subjective study has been conducted at IRT b-com in a room complying with the ITU-R BT.500-13 Recommendation [7] (see Figure 2). A SONY 85” KD-85ZG 8K display and a Zaxel’s zaxtar 5 8K player has been used to visualize the video sequences. The observers are at a distance of 0.8H (see Figure 3). Nineteen non-expert observers aged from 22 to 53 years have taken part in this experiment. All participants have been screened for normal visual acuity and color blindness using the Ishihara and Snellen vision tests.

Figure 2:  Test laboratory conditions

Figure 3:  Visualisation workflow

In this study, we used the method described in the ITU-R Rec BT500-14 [8], called Double Stimulus Continuous Quality Scale (DSCQS), to collect the video quality scores from participants. This testing method requires a prior pseudo-random sequencing of the testing videos, as the observer has no interactivity with the player. This method presents the test videos by pairs ("video A" and "video B") separated with annotated mid-greys. The presented pairs contain the implicit 8K uncompressed reference and one random PVS over all the ten configurations, i.e., the same scene encoded with HEVC or VVC at four bitrates or the uncompressed sequence in 4K or 8K resolution. Each video pair is presented twice

Also, to prevent visual fatigue, the test is divided into three sessions of 20 minutes each. Before each experiment, participants receive clear explanations about the evaluation procedures.

After the first "video A/video B" pair presentation, the participant could report his opinion about the perceived video quality on two vertical lines with the corresponding sequence index for both "video A" and "video B". For this testing method, the vertical rating lines are divided into five segments of the same height and scaled from the lower to the higher quality with the labels Bad, Poor, Fair, Good, and Excellent. After each video pair visualization, participants can vote by annotating both videos along the continuous quality scale. The scores are then collected by converting the annotations into a value between 0 and 100.

At the end of the subjective test sessions, the results for each scene are assessed by the DMOS, corresponding to the average of the difference between the hidden reference and the corresponding PVS scores computed by:

xa= 1ni=1nxi,a ,

where n is the total number of valid participants, xa is the DMOS value of the tested configuration a, a∈{Rjm, 4K, 8K (ref)} for j∈{1,2} and m∈{VVC,HEVC}, and xi,a is the differential score computed as:

xi,a=100-yi, ref-yi, a ,

with the pair yi, ref-yi, a representing the scores attributed by the participant i, i∈{1,…,n}, to respectively the hidden reference (8K) and the tested configuration a, i.e.  both videos of a given test point.

To ensure that the vote distributions are normal, the bias reduction technique described in the ITU-T P.913 Recommendation [9] has been applied. Thus, from each resulting DMOS xa, the associated confidence intervals at 95% (xa-ca, xa+ca) can be computed as follows:

ca=1.96san ,

where sa is the standard deviation of the tested configuration a computed as:

sa=i=1nxi,a-xa2(n-1)

with xi,a and xa corresponding to the differential score of the observer i, i∈{1,…,n}, and the DMOS score of the tested configuration a, respectively.

Objective measurements

# In this experiment, objective quality metrics, including PSNR, MS-SSIM [4], and VMAF [5], are used to measure the distortion between the 8K reconstructed signal and the source video. PSNR is assessed on the luma component only.

# The RD-curves are depicted in Figure 4 and 5. It can be noted that the bitrates selected for transparency lead to quite different PSNR values depending on the sequence. In contrast, for more perceptually correlated objective metrics like MS-SSIM or VMAF, the predicted quality converges to the maximum value for all 8K sequences. We can note that the bitrate selected for transparency varies from 11Mbps to 180Mbps, depending on the test sequence.

Figure 4:  Objective quality comparison using PSNR

Figure 5:  Objective quality comparison using MS-SSIM and VMAF

We also use the Bjontegaard-Delta computation method described in [10] to quantify the average gain in bitrate and visual quality offered by the VTM over the HM codec. The results are summarized in Table 1. In average, the VTM codec enables around 31%, 26% and 35% of bitrate saving over the HM codec, regarding PSNR, MS-SSIM and VMAF, respectively.

Table 1:  BD-BR scores based on objective assessment of the VTM-11 codec compared to the anchor HM-16.20.

| # Sequence | # BD-BR (PSNR) | # BD-BR (MS-SSIM) | # BD-BR (VMAF) |
| --- | --- | --- | --- |
| # LayeredKimono | # -29.77% | # -21.05% | # -33.30% |
| # BodeMuseum | # -32.75% | # -25.05% | # -34.70% |
| # OberbaumSpree | # -32.07% | # -27.00% | # -33.41% |
| # Festival2 | # -36.40% | # -33.36% | # -28.24% |
| # JapaneseMaple | # -28.33% | # -23.37% | # -30.86% |
| # SteelPlant | # -28.20% | # -24.40% | # -27.57% |
| # Average | # -31.27% | # -25.7% | # -35.30% |

Subjective measurements

The RD curves based on the collected DMOS scores are depicted in Figure 6 for all 8K sequences. These curves also display the scores obtained for the 8K hidden reference videos and the 4K sequences, with their associated 95% confidence interval represented by transparent areas.

Figure 6:  DMOS-based comparison with associated 95% confidence interval

# The results demonstrate that the perceived quality between uncompressed 8K and 4K formats depends on the scene content. Thus, for the sequences JapaneseMaple, SteelPlant, BodeMuseum, and LayeredKimono, the visual quality between both resolutions is significantly different. For sequences with non-significant visual difference between 8K and 4K resolutions (Festival2 and OberbaumSpree), the motion in the scene can explain the 8K definition loss at 60fps. Indeed, the global motion in Festival2 video sequence prevents from perceiving the details. For the OberbaumSpree motion blur appears on the scene due to a continuous horizontal camera traveling. It shows that higher framerates, e.g., 100/120fps, must be considered to fully benefit from the 8K resolution.

# Finally, we applied the BD-BR method to the DMOS scores. All the results are reported in Table 2. These results demonstrate that VVC offers a compression gain over HEVC for the same perceived quality from 32.66% to 54.30% with an average of 41.65% over the whole 8K test dataset.

Table 2:  BD-BR scores based on DMOS of the VTM-11 codec compared to the anchor HM-16.20.

| # Sequence | # BD-BR (DMOS) |
| --- | --- |
| # LayeredKimono | # -43.82% |
| # BodeMuseum | # -40.20% |
| # OberbaumSpree | # -54.30% |
| # Festival2 | # -32.66% |
| # JapaneseMaple | # -44.81% |
| # SteelPlant | # -34.11% |
| # Average | # -41.65% |

Conclusion

In this contribution an objective and subjective quality assessments of two video codecs including the HEVC standard through its HM-16.20 implementation and the VVC standard using the VTM-11 have been done. In terms of BD-BR, the VTM codec enables bitrate savings in the range of 25.7% to 35.30% in average for 8K video sequences depending on the considered objective metric. The subjective scores analysis showed that VVC brings quality improvement compared to the HEVC standard for different video sequences, coded at various bitrates.

Three main observations can be drawn from these subjective evaluations:

- VVC outperforms HEVC for 8K video with around 41% bitrate saving in average for a similar visual quality.

- The required bandwidth to convey 8K video at high quality is content-dependent and may vary in the range of 11 Mbps to 180 Mbps with VVC.

- Quality enhancement brought by 8K resolution over 4K is content-dependent: high quality acquisition sensors are essential, and high frame rate (>60fps) is important for contents with high motion.

References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “Jvet common test conditions and software reference configurations for sdr video,” March 2019.

[2] F. Bossen, “Common test conditions and software reference configurations,” May 2012.

[3] B. Bross, H. Kirchhoffer, C. Bartnik, and M. Palkow, “Document JVETQ0791: Multiformat berlin test sequences.” 13-17 January 2020.

[4] Z. Wang, E. P. Simoncelli, and A. C. Bovik, “Multiscale structural similarity for image quality assessment,” in The Thrity-Seventh Asilomar Conference on Signals, Systems & Computers, 2003, vol. 2. Ieee, 2003, pp. 1398–1402.

[5] A. A. Zhi Li, “Toward a Practical Perceptual Video Quality Metric,” in Netflix TechBlog, June 2016.

[6] C. E. Duchon, “Lanczos filtering in one and two dimensions,” Journal of applied meteorology, vol. 18, no. 8, pp. 1016–1022, 1979.

[7] ITU-R, “Recommendation BT.500-13: Methodologies for the Subjective Assessment of the Quality of Television Images.”

[8] ITU-R, “Recommendation BT.500-14: Methodologies for the Subjective Assessment of the Quality of Television Images.”

[9] ITU-R, “Recommendation BT.913: Methods for the Subjective Assessment of Video for Quality, Audio and Audiovisual Quality of Internet Video and Distribution Quality Television in any Environment.”

[10] G. Bjøntegaard, “Document VCEG-M33 ITU-T Q6/16: Calculation of Average PSNR Differences Between RD- Curves,” April 2001.