| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X2006-v1 |
| --- | --- |

| Title: | Additional SEI messages for VSEI (Draft 5) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Gary J. Sullivan Ye-Kui Wang | Email: | jill.boyce@intel.com garysull@microsoft.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the annotated regions SEI message, the alpha channel information SEI message, the depth representation information SEI message, the multiview acquisition information SEI message, the scalability dimension information SEI message, the extended dependent random access point (DRAP) indication SEI message, the display orientation SEI message, the colour transform information SEI message, and the multiview view position SEI message. The draft text also includes text changes for some technical corrections and editorial improvements.

Draft 5 incorporated items:

- JVET-X0059 AHG2/AHG9: Comments on the 2nd edition draft text for VSEI

Draft 4 incorporated items:

- JVET-W0077 AHG9: Comments on multiview-related SEI messages in VSEI

- JVET-W0078 AHG9: Multiview view position SEI message

- JVET-W0080 AHG9: Some errata and clarification items for Additional SEI messages for VSEI

- JVET-W0083 AHG9: Bug fixes for some SEI messages in the VSEI amendment

- DoCR items not covered by any of the above

Draft 3 incorporated items:

- Addition of the display orientation SEI message (JVET-V0061)

- Addition of the colour transform information SEI message (JVET-V0108)

- Changes on the scalability dimension information SEI message (JVET-V0063)

- Changes on the MAI, DRI, and ACI SEI messages and their interactions with the SDI SEI message (JVET-V0064)

- Changes on the DRAP and EDRAP indication SEI messages (JVET-V0065)

Draft 2 incorporated items:

- Addition of the alpha channel information SEI message, the depth representation information SEI message, the multiview acquisition information SEI message, and the scalability dimension information SEI message (JVET-U0082)

- Addition of the extended DRAP indication SEI message (JVET-U0084)

- Some minor editorial corrections and improvements (JVET-U0086)

- Fix for ticket #1412: change the coding of ffi_display_elemental_periods_minus1 from u(4) to u(8)

# Changes to the specification text:

# Replace 2.3 with the following:

  - Additional references

–	ISO 11664-1 (in force), Colorimetry – Part 1: CIE standard colorimetric observers.

–	ISO/IEC 11578:1996, Information technology – Open Systems Interconnection – Remote Procedure Call (RPC).

–	Recommendation ITU-T T.35:2000, Procedure for the allocation of ITU-T defined codes for non standard facilities.

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

–	IETF RFC 5646 (in force), Tags for Identifying Languages.

# In clause 3, add the following definition:

alpha blending: A process not specified by this Specification, in which an auxiliary coded picture is used in combination with a primary coded picture and with other data not specified by this Specification in the display process. In an alpha blending process, the samples of an auxiliary coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding luma samples of the primary coded picture.

# In clause 4, add the following abbreviation:

ACI	Alpha Channel Information

CTI	Colour Transform Information

DRI	Depth Representation Information

EDRAP	Extended Dependent Random Access Point

MAI	Multiview Acquisition Information

SDI	Scalability Dimension Information

# In clause 6.1, make the following changes:

  - 6.1 General

...

Technical specifications that reference this Specification for carrying VUI parameters syntax structure shall specify a container to carry the data of the VUI parameters syntax structure and to identify the length in bits of the VUI parameters syntax structure, e.g., the vui_payload( ) syntax structure specified in Rec. ITU-T H.266 | ISO/IEC 23090-3. The design of the container needs to should provide the ability to detect the number of bits in the vui_parameters( ) syntax structure and to allow the number of bits to be increased in future versions of this Specification, thus enabling this Specification to provide extensibility by directly appending additional syntax elements to the end of the vui_parameters( ) syntax structure in future versions of this Specification. The syntax of the container of the vui_parameters( ) syntax structure is outside the scope of this Specification.

Technical specifications that reference this Specification for carrying SEI messages shall specify a way to carry the payload syntax of each specified SEI message, to identify which SEI message is conveyed, and to identify the length in bits of the SEI message syntax structure, e.g., the sei_payload( ) syntax structure specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 and Rec. ITU-T H.265 | ISO/IEC 23008-2. The design of the container needs to should provide the ability to detect the number of bits in an SEI message and to allow the number of bits to be increased in future versions of this Specification, thus enabling this Specification to provide extensibility by directly appending additional syntax elements to the end of the SEI message syntax structure in future versions of this Specification. The syntax of the container of the SEI messages as well as the method of identifying which SEI message is outside the scope of this Specification.

...

# In clause 7.3, add the following NOTE immediately after the 1st paragraph and adjust the NOTE indices for the existing NOTEs accordingly:

NOTE 1	The interpretation of several syntax elements of the VUI parameters are specified by reference to coding-independent code points specified in Rec. ITU-T H.273 | ISO/IEC 23091-2. Further information about the usage of such code points is found in ITU-T H-Suppl. 19 | ISO/IEC TR 23091-4.

# Replace 6.3 with the following:

  - 6.3 Specification of syntax functions and descriptors

The functions presented in this clause are used in the syntactical description. These functions are expressed in terms of the value of the VUI parameters syntax or an SEI message syntax data pointer that indicates the position of the next bit to be read by the decoding process from the syntax structure.

read_bits( n ) reads the next n bits from the syntax structure and advances the data pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the data pointer.

The following descriptors specify the parsing process of each syntax element:

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

–	f(n): fixed-pattern bit string using n bits written (from left to right) with the left bit first. The parsing process for this descriptor is specified by the return value of the function read_bits( n ).

–	i(n): signed integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a two's complement integer representation with most significant bit written first.

–	se(v): signed integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in clause 9 with the order k equal to 0.

––	st(v): null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters as specified in ISO/IEC 10646. The parsing process is specified as follows: st(v) begins at a byte-aligned position in the bitstream and reads and returns a series of bytes from the bitstream, beginning at the current position and continuing up to but not including the next byte-aligned byte that is equal to 0x00, and advances the bitstream pointer by ( stringLength + 1 ) * 8 bit positions, where stringLength is equal to the number of bytes returned.

NOTE – The st(v) syntax descriptor is only used in this Specification when the current position in the bitstream is a byte-aligned position.

u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

–	ue(v): unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in clause 9 with the order k equal to 0.

# In clauses 8.1, replace Table 4 with the following:

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Filler payload | The PU containing the SEI message |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| User data unregistered | Unspecified |
| Film grain characteristics | Specified by the syntax of the SEI message |
| Frame packing arrangement | Specified by the syntax of the SEI message |
| Display orientation | Specified by the syntax of the SEI message |
| Referenced parameter sets | The CLVS containing the SEI message |
| Decoded picture hash | The PU containing the SEI message |
| Mastering display colour volume | The CLVS containing the SEI message |
| Colour transform information | Specified by the syntax of the SEI message |
| Content light level information | The CLVS containing the SEI message |
| DRAP indication | The PU containing picture associated with the SEI message |
| Alternative transfer characteristics | The CLVS containing the SEI message |
| Ambient viewing environment | The CLVS containing the SEI message |
| Content colour volume | Specified by the syntax of the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Generalized cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Alpha channel information | Specified by the syntax of the SEI message |
| Frame-field information | The PU containing the SEI message |
| Depth representation information | Specified by the semantics of the SEI message |
| Multiview acquisition information | The CVS containing the SEI message |
| Multiview view position | The CVS containing the SEI message |
| Annotated regions | Specified by the syntax of the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |
| Scalability dimension information | The CVS containing the SEI message |
| Extended DRAP indication | The picture associated with the SEI message |

# In clause 8.6.2, add the following NOTE immediately after the 1st paragraph and adjust the NOTE indices for the existing NOTEs accordingly:

NOTE 1	The interpretation of frame_packing_arrangement_type is in alignment with the code point specifications in Rec. ITU-T H.273 | ISO/IEC 23091-2. However, more values of frame_packing_arrangement_type are specified in Rec. ITU-T H.273 | ISO/IEC 23091-2 than are specified for use herein.

# In clause 8.6.2, make the following change:

...

fp_arrangement_cancel_flag equal to 1 indicates that the frame packing arrangement SEI message ...

...

# In clause 8.8.2, make the following change:

...

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

Prior to computing the hash, the decoded picture data are arranged into one or three strings of bytes called pictureData[ cIdx ] of lengths dataLen[ cIdx ] as follows:

...

# In clause 8.14.2, make the following change:

...

ccv_cancel_flag equal to 1 indicates that the content colour volume SEI message ...

...

# Replace the texts in clause 8.11.2 with the following:

The presence of the DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the associated IRAP picture of the DRAP picture.

The constraints indicated by the presence of the DRAP indication SEI message, which shall all apply, are as follows:

–	The DRAP picture is a trailing picture.

–	The DRAP picture has a temporal sublayer identifier equal to 0.

–	The DRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the associated IRAP picture of the DRAP picture.

–	Any picture that is in the same layer and follows the DRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the DRAP picture in decoding order or output order, with the exception of the associated IRAP picture of the DRAP picture.

# In clauses 8.16.1. change the coding of the ffi_display_elemental_periods_minus1 syntax element from u(4) to u(8).

# Add clauses 8.18 to 8.25 as follows:

  - Annotated regions SEI message

8.18.1 Annotated regions SEI message syntax

| annotated_regions( payloadSize ) { | Descriptor |
| --- | --- |
| ar_cancel_flag | u(1) |
| if( !ar_cancel_flag ) { |  |
| ar_not_optimized_for_viewing_flag | u(1) |
| ar_true_motion_flag | u(1) |
| ar_occluded_object_flag | u(1) |
| ar_partial_object_flag_present_flag | u(1) |
| ar_object_label_present_flag | u(1) |
| ar_object_confidence_info_present_flag | u(1) |
| if( ar_object_confidence_info_present_flag ) |  |
| ar_object_confidence_length_minus1 | u(4) |
| if( ar_object_label_present_flag ) { |  |
| ar_object_label_language_present_flag | u(1) |
| if( ar_object_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| ar_bit_equal_to_zero /* equal to 0 */ | f(1) |
| ar_object_label_language | st(v) |
| } |  |
| ar_num_label_updates | ue(v) |
| for( i = 0; i < ar_num_label_updates; i++ ) { |  |
| ar_label_idx[ i ] | ue(v) |
| ar_label_cancel_flag | u(1) |
| LabelAssigned[ ar_label_idx[ i ] ] = !ar_label_cancel_flag |  |
| if( !ar_label_cancel_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| ar_bit_equal_to_zero /* equal to 0 */ | f(1) |
| ar_label[ ar_label_idx[ i ] ] | st(v) |
| } |  |
| } |  |
| } |  |
| ar_num_object_updates | ue(v) |
| for( i = 0; i <= ar_num_object_updates; i++ ) { |  |
| ar_object_idx[ i ] | ue(v) |
| ar_object_cancel_flag | u(1) |
| ObjectTracked[ ar_object_idx[ i ] ] = !ar_object_cancel_flag |  |
| if( !ar_object_cancel_flag ) { |  |
| if( ar_object_label_present_flag ) { |  |
| ar_object_label_update_flag | u(1) |
| if( ar_object_label_update_flag ) |  |
| ar_object_label_idx[ ar_object_idx[ i ] ] | ue(v) |
| } |  |
| ar_bounding_box_update_flag | u(1) |
| if( ar_bounding_box_update_flag ) { |  |
| ar_bounding_box_cancel_flag | u(1) |
| ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] = !ar_bounding_box_cancel_flag |  |
| if( !ar_bounding_box_cancel_flag ) { |  |
| ar_bounding_box_top[ ar_object_idx[ i ] ] | u(16) |
| ar_bounding_box_left[ ar_object_idx[ i ] ] | u(16) |
| ar_bounding_box_width[ ar_object_idx[ i ] ] | u(16) |
| ar_bounding_box_height[ ar_object_idx[ i ] ] | u(16) |
| if( ar_partial_object_flag_present_flag ) |  |
| ar_partial_object_flag[ ar_object_idx[ i ] ] | u(1) |
| if( ar_object_confidence_info_present_flag ) |  |
| ar_object_confidence[ ar_object_idx[ i ] ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.18.2 Annotated regions SEI message semantics

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects.

Use of this SEI message requires the definition of the following variables:

- A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- A conformance cropping window left offset, ConfWinLeftOffset

- A conformance cropping window top offset, ConfWinTopOffset

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause 7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

ar_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous annotated regions SEI message that is associated with one or more layers to which the annotated regions SEI message applies. ar_cancel_flag equal to 0 indicates that annotated regions information follows.

When ar_cancel_flag equal to 1 or a new CVS of the current layer begins, the variables LabelAssigned[ i ], ObjectTracked[ i ], and ObjectBoundingBoxAvail are set equal to 0 for i in the range of 0 to 255, inclusive.

ar_not_optimized_for_viewing_flag equal to 1 indicates that the decoded pictures that the annotated regions SEI message applies to are not optimized for user viewing, but rather are optimized for some other purpose such as algorithmic object classification performance. ar_not_optimized_for_viewing_flag equal to 0 indicates that the decoded pictures that the annotated regions SEI message applies to may or may not be optimized for user viewing.

ar_true_motion_flag equal to 1 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to was selected with a goal of accurately representing object motion for objects in the annotated regions. ar_true_motion_flag equal to 0 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to may or may not be selected with a goal of accurately representing object motion for objects in the annotated regions.

ar_occluded_object_flag equal to 1 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object or a portion of an object that may not be visible or may be only partially visible within the cropped decoded picture. ar_occluded_object_flag equal to 0 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is entirely visible within the cropped decoded picture. It is a requirement of bitstream conformance that the value of ar_occluded_object_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_partial_object_flag_present_flag equal to 1 indicates that ar_partial_object_flag[ ar_object_idx[ i ] ] syntax elements are present. ar_partial_object_flag_present_flag equal to 0 indicates that ar_partial_object_flag[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_partial_object_flag_present_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_label_present_flag equal to 1 indicates that label information corresponding to objects in the annotated regions is present. ar_object_label_present_flag equal to 0 indicates that label information corresponding to the objects in the annotated regions is not present.

ar_object_confidence_info_present_flag equal to 1 indicates that ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are present. ar_object_confidence_info_present_flag equal to 0 indicates that ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_object_confidence_present_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_confidence_length_minus1 plus 1 specifies the length, in bits, of the ar_object_confidence[ ar_object_idx[ i ] ] syntax elements. It is a requirement of bitstream conformance that the value of ar_object_confidence_length_minus1 shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_label_language_present_flag equal to 1 indicates that the ar_object_label_language syntax element is present. ar_object_label_language_present_flag equal to 0 indicates that the ar_object_label_language syntax element is not present.

ar_bit_equal_to_zero shall be equal to zero.

ar_object_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the ar_object_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

ar_num_label_updates indicates the total number of labels associated with the annotated regions that are signalled. The value of ar_num_label_updates shall be in the range of 0 to 255, inclusive.

ar_label_idx[ i ] indicates the index of the signalled label. The value of ar_label_idx[ i ] shall be in the range of 0 to 255, inclusive.

ar_label_cancel_flag equal to 1 cancels the persistence scope of the ar_label_idx[ i ]-th label. ar_label_cancel_flag equal to 0 indicates that the ar_label_idx[ i ]-th label is assigned a signalled value.

LabelAssigned[ ar_label_idx[ i ] ] equal to 1 indicates that the ar_label_idx[ i ]-th label is assigned. LabelAssigned[ ar_label_idx[ i ] ] equal to 0 indicates that the ar_label_idx[ i ]-th label is not assigned.

ar_label[ ar_label_idx[ i ] ] specifies the contents of the ar_label_idx[ i ] –th label. The length of the ar_label[ ar_label_idx[ i ] ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

ar_num_object_updates indicates the number of object updates to be signalled. ar_num_object_updates shall be in the range of 0 to 255, inclusive.

ar_object_idx[ i ] is the index of the object parameters to be signalled. ar_object_idx[ i ] shall be in the range of 0 to 255, inclusive.

ar_object_cancel_flag equal to 1 cancels the persistence scope of the ar_object_idx[ i ]-th object. ar_object_cancel_flag equal to 0 indicates that parameters associated with the ar_object_idx[ i ]-th object are signalled.

ObjectTracked[ ar_object_idx[ i ] ] equal to 1 indicates that the object_idx[ i ]-th object is tracked. ObjectTracked[ ar_object_idx[ i ] ] equal to 0 indicates that the object_idx[ i ]-th object is not tracked.

ar_object_label_update_flag equal to 1 indicates that an object label is signalled. ar_object_label_update_flag equal to 0 indicates that an object label is not signalled.

ar_object_label_idx[ ar_object_idx[ i ] ] indicates the index of the label corresponding to the ar_object_idx[ i ]-th object. When ar_object_label_idx[ ar_object_idx[ i ] ] is not present, its value is inferred from a previous annotated regions SEI message in output order in the same CVS, if any. The value of ar_object_label_idx[ ar_object_idx[ i ] ] shall be in the range of 0 to 255, inclusive.

ar_bounding_box_update_flag equal to 1 indicates that object bounding box parameters are signalled.  ar_bounding_box_update_flag equal to 0 indicates that object bounding box parameters are not signalled.

ar_bounding_box_cancel_flag equal to 1 cancels the persistence scope of the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], ar_bounding_box_height[ ar_object_idx[ i ] ]. ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ]. ar_bounding_box_cancel_flag equal to 0 indicates that ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] ar_bounding_box_height[ ar_object_idx[ i ] ] ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are signalled.

ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 1 indicates that the bounding box information of the object_idx[ i ]-th object is signalled. ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 0 indicates that the bounding box information of the object_idx[ i ]-th object is not signalled.

ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the ar_object_idx[ i ]-th object in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

The value of ar_bounding_box_left[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − 1, inclusive.

The value of ar_bounding_box_top[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − 1, inclusive.

The value of ar_bounding_box_width[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − ar_bounding_box_left[ ar_object_idx[ i ] ], inclusive.

The value of ar_bounding_box_height[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − ar_bounding_box_top[ ar_object_idx[ i ] ], inclusive.

The identified object rectangle contains the luma samples with horizontal picture coordinates from SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] ) to SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] + ar_bounding_box_width[ ar_object_idx[ i ] ] ) − 1, inclusive, and vertical picture coordinates from SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] ) to SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] + ar_bounding_box_height[ ar_object_idx[ i ] ] ) − 1, inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

The values of ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] persist in output order within the CVS for each value of ar_object_idx[ i ]. When not present, the values of  ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] or ar_bounding_box_height[ ar_object_idx[ i ] ] are inferred from a previous annotated regions SEI message in output order in the CVS, if any.

ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is only partially visible within the cropped decoded picture. ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that may or may not be only partially visible within the cropped decoded picture. When not present, the value of ar_partial_object_flag[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CVS, if any.

ar_object_confidence[ ar_object_idx[ i ] ] indicates the degree of confidence associated with the ar_object_idx[ i ]-th object, in units of 2−( ar_object_confidence_length_minus1 + 1 ), such that a higher value of ar_object_confidence[ ar_object_idx[ i ] ] indicates a higher degree of confidence. The length of the ar_object_confidence[ ar_object_idx[ i ] ] syntax element is ar_object_confidence_length_minus1 + 1 bits. When not present, the value of_object_confidence[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CVS, if any.

  - Scalability dimension information SEI message

8.19.1 Scalability dimension information SEI message syntax

| scalability_dimension_info( payloadSize ) { | Descriptor |
| --- | --- |
| sdi_max_layers_minus1 | u(6) |
| sdi_multiview_info_flag | u(1) |
| sdi_auxiliary_info_flag | u(1) |
| if( sdi_multiview_info_flag ¦ ¦ sdi_auxiliary_info_flag ) { |  |
| if( sdi_multiview_info_flag ) |  |
| sdi_view_id_len_minus1 | u(4) |
| for( i = 0; i <= sdi_max_layers_minus1; i++ ) { |  |
| sdi_layer_id[ i ] | u(6) |
| if( sdi_multiview_info_flag ) |  |
| sdi_view_id_val[ i ] | u(v) |
| if( sdi_auxiliary_info_flag ) |  |
| sdi_aux_id[ i ] | u(8) |
| if( sdi_aux_id[ i ] > 0 ) { |  |
| sdi_num_associated_primary_layers_minus1[ i ] | u(6) |
| for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ ) |  |
| sdi_associated_primary_layer_idx[ i ][ j ] | u(6) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.19.2 Scalability dimension information SEI message semantics

The scalability dimension information (SDI) SEI message provides the SDI for each layer in the current CVS, i.e., the CVS containing the SDI SEI message, such as 1) when there may be multiple views, the view ID of each layer; and 2) when there may be auxiliary information (such as depth or alpha) carried by one or more layers, the auxiliary ID of each layer.

When an SDI SEI message is present in any AU of a CVS, an SDI SEI message shall be present for the first AU of the CVS. All SDI SEI messages in a CVS shall have the same content.

sdi_max_layers_minus1 plus 1 indicates the maximum number of layers in the current CVS.

sdi_multiview_info_flag equal to 1 indicates that the current CVS may have multiple views and the sdi_view_id_val[ ] syntax elements are present in the SDI SEI message. sdi_multiview_info_flag equal to 0 indicates that the current CVS does not have multiple views and the sdi_view_id_val[ ] syntax elements are not present in the SDI SEI message.

sdi_auxiliary_info_flag equal to 1 indicates that one or more layers in the current CVS may be auxiliary layers, which carry auxiliary information, and the sdi_aux_id[ ] syntax elements are present in the SDI SEI message. sdi_auxiliary_info_flag equal to 0 indicates that the current CVS does not have an auxilary layer and the sdi_aux_id[ ] syntax elements are not present in the SDI SEI message.

sdi_view_id_len_minus1 plus 1 specifies the length, in bits, of the sdi_view_id_val[ i ] syntax element.

sdi_layer_id[ i ] specifies the layer identifier of the i-th layer that may be present in the current CVS.

sdi_view_id_val[ i ] specifies the view identifier of the i-th layer in the current CVS. The length of the sdi_view_id_val[ i ] syntax element is sdi_view_id_len_minus1 + 1 bits.

The variable NumViews, specifying the number of views in the current CVS, and the list ViewId, specifying the view identifiers of the views in the current CVS, are derived as follows:

NumViews = 1
if( sdi_multiview_info_flag ) {
	ViewId[ 0 ] = sdi_view_id_val[ 0 ]
	for( i = 1; i  <=  sdi_max_layers_minus1; i++ ) {
		newViewFlag = 1
		for( j = 0; j < i; j++ )		(47)
			if( sdi_view_id_val[ i ]  = =  sdi_view_id_val[ j ] )
				newViewFlag = 0
		if( newViewFlag ) {
			ViewId[ NumViews ] = sdi_view_id_val[ i ]
			NumViews++
		}
	}
}

sdi_aux_id[ i ] equal to 0 indicates that the i-th layer in the current CVS does not contain auxiliary pictures. sdi_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in the current CVS as specified in Table 15. When sdi_auxiliary_info_flag is equal to 0, the value of sdi_aux_id[ i ] is inferred to be equal to 0.

Table 15 – Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sdi_aux_id[ i ] in the range of 128 to 159, inclusive, is specified through means other than the sdi_aux_id[ i ] value.

sdi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sdi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of sdi_aux_id[ i ] in the range of 0 to 255, inclusive.

If sdi_aux_id[ i ] is equal to 0, the i-th layer is referred to as a primary layer. Otherwise, the i-th layer is referred to as an auxiliary layer. When sdi_aux_id[ i ] is equal to 1, the i-th layer is also referred to as an alpha auxiliary layer. When sdi_aux_id[ i ] is equal to 2, the i-th layer is also referred to as a depth auxiliary layer.

sdi_num_associated_primary_layers_minus1[ i ] plus 1 specifies the number of associated primary layers of i-th layer, which is an auxiliary layer. The value of sdi_num_associated_primary_layers_minus1[ i ] shall be less than the total number of primary layers.

sdi_associated_primary_layer_idx[ i ][ j ] specifies the layer index of the j-th associated primary layer of the i-th layer, which is an auxiliary layer. The value of sdi_aux_id[ sdi_associated_primary_layer_idx[ i ][ j ] ] shall be equal to 0.

NOTE 2 – An auxiliary layer describes a property of and applies to its associated primary layers.

  - Multiview acquisition information SEI message

8.20.1 Multiview acquisition information SEI message syntax

| multiview_acquisition_info( payloadSize ) { | Descriptor |
| --- | --- |
| intrinsic_param_flag | u(1) |
| extrinsic_param_flag | u(1) |
| num_views_minus1 | ue(v) |
| if( intrinsic_param_flag ) { |  |
| intrinsic_params_equal_flag | u(1) |
| prec_focal_length | ue(v) |
| prec_principal_point | ue(v) |
| prec_skew_factor | ue(v) |
| for( i = 0; i <= intrinsic_params_equal_flag ? 0 : num_views_minus1; i++ ) { |  |
| sign_focal_length_x[ i ] | u(1) |
| exponent_focal_length_x[ i ] | u(6) |
| mantissa_focal_length_x[ i ] | u(v) |
| sign_focal_length_y[ i ] | u(1) |
| exponent_focal_length_y[ i ] | u(6) |
| mantissa_focal_length_y[ i ] | u(v) |
| sign_principal_point_x[ i ] | u(1) |
| exponent_principal_point_x[ i ] | u(6) |
| mantissa_principal_point_x[ i ] | u(v) |
| sign_principal_point_y[ i ] | u(1) |
| exponent_principal_point_y[ i ] | u(6) |
| mantissa_principal_point_y[ i ] | u(v) |
| sign_skew_factor[ i ] | u(1) |
| exponent_skew_factor[ i ] | u(6) |
| mantissa_skew_factor[ i ] | u(v) |
| } |  |
| } |  |
| if( extrinsic_param_flag ) { |  |
| prec_rotation_param | ue(v) |
| prec_translation_param | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |
| for( j = 0; j < 3; j++ ) { /* row */ |  |
| for( k = 0; k < 3; k++ ) { /* column */ |  |
| sign_r[ i ][ j ][ k ] | u(1) |
| exponent_r[ i ][ j ][ k ] | u(6) |
| mantissa_r[ i ][ j ][ k ] | u(v) |
| } |  |
| sign_t[ i ][ j ] | u(1) |
| exponent_t[ i ][ j ] | u(6) |
| mantissa_t[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

8.20.2 Multiview acquisition information SEI message semantics

The multiview acquisition information (MAI) SEI message specifies various parameters of the acquisition environment for the layers that may be present in the current CVS, i.e., the CVS containing the MAI SEI message. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

When an MAI SEI message is present in any AU of a CVS, an MAI SEI message shall be present for the first AU of the CVS. All MAI SEI messages in a CVS shall have the same content.

When a CVS does not contain an SDI SEI message, the CVS shall not contain an MAI SEI message.

When an AU contains both an SDI SEI message and an MAI SEI message, the SDI SEI message shall precede the MAI SEI message in decoding order.

Some of the views for which the MAI is included in an MAI SEI message may not be present in the current CVS.

In the semantics below, syntax elements and variables with index i refer to the syntax elements and variables that apply to the i-th view in the current CVS specified by the SDI SEI message, i.e., the view with view identifier equal to ViewId[ i ].

The extrinsic camera parameters are specified according to a right-handed coordinate system, where the upper left corner of the image is the origin, i.e., the ( 0, 0 ) coordinate, with the other corners of the image having non-negative coordinates. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(48)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ] and T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

extrinsic_param_flag equal to 1 indicates the presence of extrinsic camera parameters. extrinsic_param_flag equal to 0 indicates the absence of extrinsic camera parameters.

num_views_minus1 plus 1 specifies the number of views for which the MAI is included in the MAI SEI message. The value of num_views_minus1 shall be equal to NumViews − 1.

intrinsic_params_equal_flag equal to 1 indicates that the intrinsic camera parameters are equal for all cameras and only one set of intrinsic camera parameters is present. intrinsic_params_equal_flag equal to 0 indicates that the intrinsic camera parameters are different for each camera and that a set of intrinsic camera parameters is present for each camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_skew_factor specifies the exponent of the maximum allowable truncation error for skew factor as given by 2−prec_skew_factor. The value of prec_skew_factor shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_focal_length_x[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_focal_length_y[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction. The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_x[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction. The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_y[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_skew_factor[ i ] equal to 0 indicates that the sign of the skew factor of the i-th camera is positive.

sign_skew_factor[ i ] equal to 1 indicates that the sign is negative.

exponent_skew_factor[ i ] specifies the exponent part of the skew factor of the i-th camera. The value of exponent_skew_factor[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified skew factor.

mantissa_skew_factor[ i ] specifies the mantissa part of the skew factor of the i-th camera. The length of the mantissa_skew_factor[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_skew_factor[ i ] is equal to 0, the length is Max( 0, prec_skew_factor − 30 ).

- Otherwise (exponent_skew_factor[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_skew_factor[ i ] + prec_skew_factor − 31 ).

The intrinsic matrix A[ i ] for i-th camera is represented by

(49)

prec_rotation_param specifies the exponent of the maximum allowable truncation error for rE[ i ][ j ][ k ] (see Equation 50) as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for tE[ i ][ j ] (see Equation 51) as given by 2−prec_translation_param. The value of prec_translation_param shall be in the range of 0 to 31, inclusive.

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of ( j, k ) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of ( j, k ) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of ( j, k ) component of the rotation matrix for the i-th camera. The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

- If exponent_r[ i ] is equal to 0, the length is Max( 0, prec_rotation_param − 30 ).

- Otherwise (exponent_r[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

(50)

sign_t[ i ][ j ] equal to 0 indicates that the sign of the j-th component of the translation vector for the i-th camera is positive. sign_t[ i ][ j ] equal to 1 indicates that the sign is negative.

exponent_t[ i ][ j ] specifies the exponent part of the j-th component of the translation vector for the i-th camera. The value of exponent_t[ i ][ j ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t[ i ][ j ] specifies the mantissa part of the j-th component of the translation vector for the i-th camera. The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

- If exponent_t[ i ] is equal to 0, the length v is set equal to Max( 0, prec_translation_param − 30 ).

- Otherwise (0 < exponent_t[ i ] < 63), the length v is set equal to Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The translation vector T[ i ] for the i-th camera is represented by:

(51)

The association between the camera parameter variables and corresponding syntax elements is specified by Table 16. Each component of the intrinsic and rotation matrices and the translation vector is obtained from the variables specified in Table 16 as the variable x computed as follows:

- If e is in the range of 0 to 63, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989.

Table 16 – Association between camera parameter variables and syntax elements.

| x | s | e | n |
| --- | --- | --- | --- |
| focalLengthX[ i ] | sign_focal_length_x[ i ] | exponent_focal_length_x[ i ] | mantissa_focal_length_x[ i ] |
| focalLengthY[ i ] | sign_focal_length_y[ i ] | exponent_focal_length_y[ i ] | mantissa_focal_length_y[ i ] |
| principalPointX[ i ] | sign_principal_point_x[ i ] | exponent_principal_point_x[ i ] | mantissa_principal_point_x[ i ] |
| principalPointY[ i ] | sign_principal_point_y[ i ] | exponent_principal_point_y[ i ] | mantissa_principal_point_y[ i ] |
| skewFactor[ i ] | sign_skew_factor[ i ] | exponent_skew_factor[ i ] | mantissa_skew_factor[ i ] |
| rE[ i ][ j ][ k ] | sign_r[ i ][ j ][ k ] | exponent_r[ i ][ j ][ k ] | mantissa_r[ i ][ j ][ k ] |
| tE[ i ][ j ] | sign_t[ i ][ j ] | exponent_t[ i ][ j ] | mantissa_t[ i ][ j ] |

  - Multiview view position SEI message

    - Multiview view position SEI message syntax

| multiview_view_position( payloadSize ) { | Descriptor |
| --- | --- |
| num_views_minus1 | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |
| view_position[ i ] | ue(v) |
| } |  |

8.21.2 Multiview view position SEI message semantics

The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS.

When a multiview view position SEI message is present in any AU of a CVS, a multiview view position SEI message shall be present for the first AU of the CVS. All multiview view position SEI messages in a CVS shall have the same content.

When a CVS does not contain an SDI SEI message, the CVS shall not contain a multiview view position SEI message.

When an AU contains both an SDI SEI message and a multiview view position SEI message, the SDI SEI message shall precede the multiview view position SEI message in decoding order.

Some of the views for which the view position information is included in a multiview view position SEI message may not be present in the current CVS.

num_views_minus1 plus 1 shall be equal to NumViews derived from the SDI SEI message for the CVS.

view_position[ i ] indicates the order of the view with view identifier equal to ViewId[ i ] among all the views from left to right for the purpose of display, with the order for the left-most view being equal to 0 and the value of the order increasing by 1 for next view from left to right. The value of view_position[ i ] shall be in the range of 0 to 62, inclusive.

  - Depth representation information SEI message

8.21.1 Depth representation information SEI message syntax

| depth_representation_info( payloadSize ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( d_min_flag ¦ ¦ d_max_flag ) |  |
| disparity_ref_view_id | ue(v) |
| if( z_near_flag ) |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| depth_rep_info_element( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| depth_rep_info_element( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| } |  |
| } |  |

##### 8.21.1.1 Depth representation information element syntax

| depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

8.21.2 Depth representation information SEI message semantics

The syntax elements in the depth representation information (DRI) SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH for the purpose of processing decoded primary and auxiliary pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

Use of this SEI message requires the definition of the following variable:

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i, no picture in the CVS shall be associated with a DRI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i and a DRI SEI message, the SDI SEI message shall precede the DRI SEI message in decoding order.

When present, the DRI SEI message shall be associated with one or more layers that are indicated as depth auxiliary layers by an SDI SEI message. The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the DRI SEI message applies.

When present, the DRI SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a DRI SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table 17. In Table 17, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera. The value of depth_representation_type shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this document. The values of 4 to 15, inclusive, for depth_representation_type are reserved for future use by ITU-T | ISO/IEC. Although the value of depth_representation_type is required to be in the range of 0 to 3, inclusive, in this version of this document, decoders shall allow values of depth_representation_type in the range of 4 to 15, inclusive, to appear in the syntax. Decoders conforming to this version of this document shall ignore all data that follow a value of depth_representation_type in the range of 4 to 15, inclusive, in the depth representation information SEI message.

The variable maxVal is set equal to ( 1  <<  BitDepthY ) − 1.

Table 17 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the view identifier for which the disparity values are derived. The value of disparity_ref_view_id shall be in the range of 0 to 1023, inclusive.

NOTE 1 – The view identifier of the i-th view in the current CVS is equal to ViewId[ i ] as specified in the semantics of the SDI SEI message in subclause 8.19.2.

NOTE 2 – disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table 18 are derived from the respective variables in the s, e, n and v columns of Table 18 as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 3 – The above specification is similar to that found in IEC 60559:1989.

Table 18 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the associated primary picture of the auxiliary picture of type AUX_DEPTH.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piece-wise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity. The value of depth_nonlinear_representation_num_minus1 shall be in the range of 0 to 62, inclusive.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piece-wise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The value of depth_nonlinear_representation_model[ i ] shall be in the range of 0 to 65 535, inclusive. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 4 – When depth_representation_type is equal to 3, an auxiliary picture contains non-linearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the non-linear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-non-linear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of depth_nonlinear_representation_num_minus1. The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k  <=  depth_nonlinear_representation_num_minus1 + 1; k++ ) {
	pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev2 = depth_nonlinear_representation_model[ k + 1 ]	(52)

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for( x = Max( x1, 0 ); x  <=  Min( x2, maxVal ); x++ )
			DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

##### 8.21.2.1 Depth representation information element semantics

The syntax structure specifies the value of an element in the DRI SEI message.

The depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

  - Alpha channel information SEI message

8.22.1 Alpha channel information SEI message syntax

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| } |  |
| } |  |

8.22.2 Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value. When not present, the value of alpha_channel_incr_flag is inferred to be equal to 0. [Ed. (YK): This inference seems unnecessary or even not correct. It is not present only when alpha_channel_cancel_flag is equal to 1, in which case no information applies. If the absence of the SEI message is also considered, then its value should depend on the persistency scope of the previous SEI message of this type, and this inference can then be incorrect. The right course of action seems to be to just remove the inference rule.]

When alpha_channel_incr_flag is equal to 1, alpha_transparent_value shall not be equal to alpha_opaque_value and Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) shall have an integer value. A value of alpha_transparent_value that is equal to alpha_opaque_value indicates that the auxiliary coded picture is not intended for alpha blending purposes.

NOTE 1 – For alpha blending purposes, alpha_opaque_value may be greater than alpha_transparent_value, or it may be less than alpha_transparent_value. Interpretation sample values should be clipped to the range of alpha_opaque_value to alpha_transparent_value, inclusive.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 2 – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 3 – Alpha blending composition is normally performed with a background picture B, a foreground picture F, and a decoded auxiliary coded picture A, all of the same size. Assume for purposes of example illustration that the chroma resolution of B and F have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr.

Define the variables alphaRange, alphaFwt and alphaBwt as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(53)

alphaFwt = Abs( a − alpha_transparent_value )		(54)

alphaBwt = Abs( a − alpha_opaque_value )		(55)

Then, in alpha blending composition, samples d of the displayed picture D could be calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(56)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(57)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(58)

The samples of pictures D, F and B could also represent red, green, and blue component values (see clause 7.3). Here the Y, Cb and Cr component values are assumed. Each component, e.g., Y, is assumed for purposes of example illustration above to have the same bit depth in each of the pictures D, F and B. However, different components, e.g., Y and Cb, need not have the same bit depth in this example.

If alpha_channel_use_idc is equal to 0, the following applies:

- F would be the decoded picture obtained from the decoded luma and chroma, and A would be the decoded picture obtained from the decoded auxiliary coded picture. In this case, the indicated example alpha blending composition involves multiplying the samples of F by factors obtained from the samples of A.

- A picture format that is useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. When the foreground picture was F, the pre-multiplied-black video S is given by

sY  = ( alphaFwt * fY ) / alphaRange		(59)

sCb = ( alphaFwt * fCb ) / alphaRange		(60)

sCr = ( alphaFwt * fCr ) / alphaRange		(61)

- Pre-multiplied-black video has the characteristic that the picture S will appear correct if displayed against a black background. For a non-black background B, the composition of the displayed picture D may be calculated as

dY  = sY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(62)

dCb = sCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(63)

dCr = sCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(64)

Otherwise, when alpha_channel_use_idc is equal to 1, alpha blending composition does not involve multiplication of the samples of S by factors obtained from the samples of A, S would be the decoded picture obtained from the decoded luma and chroma, and A would again be the decoded picture obtained from the decoded auxiliary coded picture.

  - Extended DRAP indication SEI message

8.23.1 Extended DRAP indication SEI message syntax

| extended_drap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| edrap_rap_id_minus1 | u(16) |
| edrap_leading_pictures_decodable_flag | u(1) |
| edrap_reserved_zero_12bits | u(12) |
| edrap_num_ref_rap_pics_minus1 | u(3) |
| for( i = 0; i <= edrap_num_ref_rap_pics_minus1; i++ ) |  |
| edrap_ref_rap_id[ i ] | u(16) |
| } |  |

8.23.2 Extended DRAP indication SEI message semantics

The picture associated with an extended DRAP (EDRAP) indication SEI message is referred to as an EDRAP picture.

The presence of the EDRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the EDRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures, which consists of a list of IRAP or EDRAP pictures in decoding order that are within the same CLVS and identified by the edrap_ref_rap_id[ i ] syntax elements.

The constraints indicated by the presence of the EDRAP indication SEI message, which shall all apply, are as follows:

–	The EDRAP picture is a trailing picture.

–	The EDRAP picture has a temporal sublayer identifier equal to 0.

–	The EDRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the referenceablePictures.

–	Any picture that is in the same layer and follows the EDRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order or output order, with the exception of the referenceablePictures.

–	Any picture in the list referenceablePictures does not include, in the active entries of its reference picture lists, any picture that is in the same layer and is not a picture at an earlier position in the list referenceablePictures.

NOTE – Consequently, the first picture in referenceablePictures, even when it is an EDRAP picture instead of an IRAP picture, does not include any picture from the same layer in the active entries of its reference picture lists.

edrap_rap_id_minus1 plus 1 specifies the RAP picture identifier, denoted as RapPicId, of the EDRAP picture.

Each IRAP or EDRAP picture is associated with a RapPicId value. The RapPicId value for an IRAP picture is inferred to be equal to 0. The RapPicId values for any two EDRAP pictures associated with the same IRAP picture shall be different.

edrap_leading_pictures_decodable_flag equal to 1 specifies that both of the following constraints apply:

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order shall follow, in output order, any picture that is in the same layer and precedes the EDRAP picture in decoding order.

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order and precedes the EDRAP picture in output order shall not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

edrap_leading_pictures_decodable_flag equal to 0 does not impose such constraints.

edrap_reserved_zero_12bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for edrap_reserved_zero_12bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of edrap_reserved_zero_12bits.

edrap_num_ref_rap_pics_minus1 plus 1 indicates the number of IRAP or EDRAP pictures that are within the same CLVS as the EDRAP picture and may be included in the active entries of the reference picture lists of the EDRAP picture.

edrap_ref_rap_id[ i ] indicates RapPicId of the i-th RAP picture that may be included in the active entries of the reference picture lists of the EDRAP picture. The i-th RAP picture shall be either the IRAP picture associated with the current EDRAP picture or an EDRAP picture associated with the same IRAP picture as the current EDRAP picture.

  - Display orientation SEI message

8.24.1 Display orientation SEI message syntax

| display_orientation( payloadSize ) { | Descriptor |
| --- | --- |
| display_orientation_cancel_flag | u(1) |
| if( !display_orientation_cancel_flag ) { |  |
| display_orientation_persistence_flag | u(1) |
| display_orientation_transform_type | u(3) |
| display_orientation_reserved_zero_3bits | u(3) |
| } |  |

8.24.2 Display orientation SEI message semantics

When the associated picture has PicOutputFlag equal to 1, the display orientation SEI message informs the decoder of a transformation that is recommended to be applied to the cropped decoded picture prior to display.

display_orientation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous display orientation SEI message in output order. display_orientation_cancel_flag equal to 0 indicates that display orientation information follows.

display_orientation_persistence_flag specifies the persistence of the display orientation SEI message for the current layer.

display_orientation_persistence_flag equal to 0 specifies that the display orientation SEI message applies to the current decoded picture only.

display_orientation_persistence_flag equal to 1 specifies that the display orientation SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a display orientation SEI message is output that follows the current picture in output order.

display_orientation_transform_type specifies the rotation and mirroring to be applied to the picture. When display_orientation_transform_type specifies both rotation and mirroring, rotation applies before mirroring. The values of display_transform_type are specified in Table 19.

Table 19 – display_orientation_transform_type values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (anticlockwise) |
| 3 | rotation by 180 degrees (anticlockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (anticlockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (anticlockwise) |
| 6 | rotation by 270 degrees (anticlockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (anticlockwise) |

display_orientation_reserved_zero_3bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for display_orientation_reserved_zero_3bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of display__orientation_reserved_zero_3bits.

  - Colour transform information SEI message

8.25.1 Colour transform information SEI message syntax

| colour_transform_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_transform_id | ue(v) |
| colour_transform_cancel_flag | u(1) |
| if( !colour_transform_cancel_flag ) { |  |
| colour_transform_persistence_flag | u(1) |
| colour_transform_video_signal_info_present_flag | u(1) |
| if( colour_transform_video_signal_info_present_flag ) { |  |
| colour_transform_full_range_flag | u(1) |
| colour_tranform_primaries | u(8) |
| colour_transform_transfer_function | u(8) |
| colour_transform_matrix_coefficients | u(8) |
| } |  |
| colour_transform_bit_depth_minus8 | u(4) |
| colour_transform_log2_number_of_points_per_lut_minus1 | u(3) |
| colour_transform_cross_component_flag | u(1) |
| if( colour_transform _cross_component_flag ) |  |
| colour_transform_cross_comp_inferred_flag | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transf_lut[ 0 ][ i ] | u(v) |
| if( colour_transform_cross_component_flag = = 0 ¦ ¦ colour_transform_cross_comp_inferred_flag = = 0 ) { |  |
| colour_transform_lut2_present_flag | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transf_lut[ 1 ][ i ] | u(v) |
| if( colour_transform_lut2_present_flag ) |  |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transf_lut[ 2 ][ i ] | u(v) |
| } else |  |
| colour_transform_chroma_offset | u(v) |
| } |  |
| } |  |

8.25.2 Colour transform information SEI message semantics

The colour transform information (CTI) SEI message provides information to enable remapping of the reconstructed colour samples of the output pictures for purposes such as converting the output pictures to a representation that is more suitable for an alternative display. The colour transform model used in the CTI SEI message is composed of a first piece-wise linear function applied to the first colour component. Depending on the values of syntax elements colour_transform_cross_component_flag, colour_transform_cross_comp_inferred_flag, and colour_transform_lut2_present_flag, one or two additional piece-wise linear functions may be signalled for the second and third colour components.

When ChromaFormatIdc is equal to 0 (monochrome), the CTI SEI message shall not be present, although decoders shall allow such messages to be present and shall ignore any such CTI SEI messages that may be present.

colour_transform_id contains an identifying number that may be used to identify the purpose of the CTI. The value of colour_transform_id may be used (in a manner not specified in this Specification) to indicate that the input to the remapping process is the output of some conversion process that is not specified in this Specification, such as a conversion of the picture to some alternative colour representation (e.g., conversion from a YCbCr colour representation to a GBR colour representation). When more than one CTI SEI message is present with the same value of colour_transform_id, the content of these CTI SEI messages shall be the same. When CTI SEI messages are present that have more than one value of colour_transform_id, this may indicate that the remapping processes indicated by the different values of colour_transform_id are alternatives that are provided for different purposes or that a cascading of remapping processes is to be applied in a sequential order (an order that is not specified in this Specification). The value of colour_transform_id shall be in the range of 0 to 232 − 2, inclusive.

Values of colour_transform_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_transform_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all CTI SEI messages containing a value of colour_transform_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE – The colour_transform_id can be used to support different remapping processes that are suitable for different display scenarios. For example, different values of colour_transform_id may correspond to different remapped colour spaces supported by displays.

colour_transform_cancel_flag equal to 1 indicates that the CTI SEI message cancels the persistence of any previous CTI SEI message in output order that applies to the current layer. colour_transform_cancel_flag equal to 0 indicates that CTI follows.

colour_transform_persistence_flag specifies the persistence of the CTI SEI message for the current layer.

colour_transform_persistence_flag equal to 0 specifies that the CTI SEI message applies to the current decoded picture only.

colour_transform_persistence_flag equal to 1 specifies that the CTI SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a CTI SEI message is output that follows the current picture in output order.

colour_transform_video_signal_info_present_flag equal to 1 specifies that syntax elements colour_transform_full_range_flag, colour_transform_primaries, colour_transform_transfer_function and colour_transform_matrix_coefficients are present, colour_transform_video_signal_info_present_flag  equal to 0 specifies that syntax elements colour_transform_full_range_flag, colour_transform_primaries, colour_transform_transfer_function and colour_transform_matrix_coefficients are not present.

colour_transform_full_range_flag has the same semantics as specified in clause 7.3 for the vui_full_range_flag syntax element, except that colour_transform_full_range_flag identifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CLVS. When not present, the value of colour_transform_full_range_flag is inferred to be equal to the value of vui_full_range_flag.

colour_transform_primaries has the same semantics as specified in clause 7.3 for the vui_colour_primaries syntax element, except that colour_transform_primaries identifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CLVS. When not present, the value of colour_transform_primaries is inferred to be equal to the value of vui_colour_primaries.

colour_transform_transfer_function has the same semantics as specified in clause 7.3 for the vui_transfer_characteristics syntax element, except that colour_transform_transfer_function identifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CLVS. When not present, the value of colour_transform_transfer_function is inferred to be equal to the value of vui_transfer_characteristics.

colour_transform_matrix_coefficients has the same semantics as specified in clause 7.3 for the vui_matrix_coeffs syntax element, except that colour_transform_matrix_coefficients identifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CLVS. When not present, the value of colour_transform_matrix_coefficients is inferred to be equal to the value of vui_matrix_coeffs.

colour_transform_bit_depth_minus8 plus 8 specifies the bit depth of the colour components of the associated pictures for purposes of interpretation of the CTI SEI message. When any CTI SEI message is present with the value of colour_transform_bit_depth plus 8 not equal to the bit depth of the decoded colour components, the SEI message refers to the hypothetical result of a conversion operation performed to convert the decoded colour component samples to the bit depth equal to colour_transform_input_bit_depth plus 8.

The value of colour_transform_bit_depth plus 8 shall be in the range of 8 to 16, inclusive. Values of colour_transform_bit_depth from in the range of 17 to 23, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all CTI SEI messages that contain a value of colour_transform_bit_depth from in the range of 17 to 23, inclusive, and bitstreams shall not contain such values.

bitDepth is set equal to ( colour_transform_bit_depth + 8 ).

colour_transform_log2_number_of_points_per_lut_minus1 specifies the log2 of the number of pivot points in the piece-wise linear remapping functions minus 1.

log2numLutPoints is set equal to ( colour_transform_log2_number_of_points_per_lut_minus1 + 1 ).

numLutPoints is set equal to ( 1  <<  log2numLutPoints ).

colourTransformSize is set equal to ( numLutPoints + 1 ).

log2distX is set equal to ( bitDepth − log2numLutPoints ).

colour_transform_cross_component_flag equal to 1 indicates that the remapping of the second and third colour components is performed as cross-component remapping based on the first colour component. colour_transform_cross_component_flag equal to 0 indicates that intra-component remapping is applied to the second and third colour components.

maxIntraComp is set equal to ( 2 * ( 1 − colour_transform_cross_component_flag ) ).

colour_transform_cross_comp_inferred_flag equal to 1 indicates that the remapping piece-wise linear functions of the second and third colour components are inferred from the remapping piece-wise linear function of the first colour component. colour_transform_cross_comp_inferred_flag equal to 0 indicates that the remapping piece-wise linear functions of the second and third colour components are signalled. When not present, the value of colour_transform_cross_comp_inferred_flag is inferred to be equal to 0.

colour_transf_lut[ c ][ i ] specifies the piecewise linear remapping function of the colour component of index c. When colour_transf_lut[ 1 ][ i ] is present and colour_transf_lut[ 2 ][ i ] is not present, the value of colour_transf_lut[ 2 ][ i ] is inferred to be equal to colour_transf_lut[ 1 ][ i ]. The length of colour_transf_lut[ c ][ i ] is 2 + bitDepth – log2numLutPoints bits.

colour_transform_lut2_present_flag equal to 1 specifies that colour_transf_lut[ 2 ][ i ] is present in the CTI SEI message. colour_transform_lut2_present_flag equal to 0 specifies that colour_transf_lut[ 2 ][ i ] is not present in the CTI SEI message. When not present, the value of colour_transform_lut2_present_flag is inferred to be equal to 0.

colour_transform_chroma_offset specifies the CTI chroma offset. When not present, colour_transform_chroma_offset is inferred to be equal to 0. The length of colour_transform_chroma_offset is 2 + bitDepth – log2numLutPoints bits.

The remapping process of the input picture components rec[ c ], with width and height equal to picWidth[ c ] and picHeight[ c ], respectively, to the output remapped picture components map[ c ], for c=0..2, is performed as follows.

The array pivotPointX is derived as follows.

–	For j=0 ..( numLutPoints − 1 ), pivotPointX[ j ] is set equal to ( j  <<  log2distX ).

For c=0..maxIntraComp, the arrays pivotPointY[ c ] and slope[ c ] are derived as follows:

–	pivotPointY[ c ][ 0 ] is set equal to colour_transf_lut[ c ][ 0 ]

–	For j=1..( numLutPoints − 1 ), pivotPointY[ c ][ j ] is derived as follows:

pivotPointY[ c ][ j ] = pivotPointY[ c ][ j − 1 ] + colour_transf_lut[ c ][ j ]	(53)

–	For j=0..( numLutPoints − 1 ), slope[ c ][ j ] is derived as follows:

slope[ c ][ j ] = ( ( colour_transf_lut[ c ][ j + 1 ]  <<  11 ) + ( 1  <<  ( log2distX − 1 ) ) )  >>  log2distX	(54)

When colour_transform_cross_component_flag is equal to 1, the arrays ccPivotPointY[ c ] and ccSlope[ c ] are derived as follows, for c=1..2:

–	If colour_transform_cross_comp_inferred_flag is equal to 0, ccPivotPointY[ c ] is derived as follows:

–	For j=0..numLutPoints, ccPivotPointY[ c ][ j ] is set equal to ( colour_transf_lut[ c ][ j ]  <<  ( 11 − log2distX ) ).

–	Otherwise (colour_transform_cross_comp_inferred_flag is equal to 1), ccPivotPointY[ c ] is derived as follows:

–	For j=0..( numLutPoints − 1 ), tmpPivotPt[ j ] is derived as follows:

–	If colour_transf_lut[ 0 ][ j + 1 ] is equal to 0, tmpPivotPt[ j ] is set equal to ( 1  <<  11 ).

–	Otherwise, tmpPivotPt[ j ] is derived as follows:

tmpPivotPt[ j ] = ( colour_transf_lut[ 0 ][ j + 1 ] + colour_transform_chroma_offset )  <<	(55)
		( 11 – log2distX )

–	The array ccPivotPointY[ c ] is derived as follows:

–	For j=1..( numLutPoints − 1 ), ccPivotPointY[ c ][ j ] is derived as follows:

ccPivotPointY[ c ][ j ] = ( tmpPivotPt[ j ] + tmpPivotPt[ j − 1 ] + 1 ) / 2	(56)

–	ccPivotPointY[ c ][ 0 ] is set equal to tmpPivotPt[ 0 ].

–	ccPivotPointY[ c ][ numLutPoints ] is set equal to tmpPivotPt[ numLutPoints − 1 ].

–	For j=0..( numLutPoints − 1 ), the value of ccSlope[ c ][ j ] is set equal to ( ccPivotPointY[ c ][ j + 1 ] − ccPivotPointY[ c ][ j ] ).

For c=0..maxIntraComp, the intra-component remapping process of the input samples picture rec[ c ] into the remapped samples picture map[ c ] is performed as follows.

–	for i=0..picWidth[ c ] − 1, j=0..picHeight[ c ] − 1, the following applies:

idx = rec[ c ][ i ][ j ]  >>  log2distX
map[ c ][ i ][ j ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, pivotPointY[ c ][ idx ] +	(57)
		( ( slope[ c ][ idx ] * ( rec[ i ][ j ] − pivotPointX[ idx ] ) + ( 1  <<  10 ) )  >>  11 ) )

When colour_transform_cross_component_flag is equal to 1, for c=1..2, the cross-component remapping process of the input samples picture rec[ c ] into the remapped samples picture map[ c ] is performed as follows:

–	offset is set equal to ( 1  <<  ( bitDepth − 1 ) ).

–	subWc and subHc are set equal to ( picWidth[ 0 ] / picWidth[ c ] ) and ( picHeight[ 0 ] / picHeight[ c ] ), respectively.

–	For i=0..picWidth[ c ] − 1, j=0..picHeight[ c ] − 1, the following applies:

coloc = rec[ 0 ][ i * SubWc ][ j * SubHc ]
idx = coloc  >>  log2distX		(58)
scale = ccPivotPointY[ c ][ idx ] + ( ( ccSlope[ c ][ idx ] * ( coloc − pivotPointX[ idx ] ) )  >>  log2distX )
map[ c ][ i ][ j ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
		( ( offset  <<  11 ) + scale * ( rec[ c ][ i ][ j ] − offset ) + ( 1  <<  10 ) )  >>  11 )

# In Bibliography, add the following reference immediately after the reference to ITU-T H.271:

Supplement ITU-T H-Suppl. 19 | ISO/IEC TR 23091-4, Information technology — Coding-independent code points — Part 4: Usage of video signal type code points