| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X2024-v2 |
| --- | --- |

| Title: | Exploration Experiment on Enhanced Compression beyond VVC capability (EE2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | EE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Ling Li Karam Naser Jacob Ström Martin Winken Xiaoyu Xiu Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com aurali@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document describes Exploration Experiments (EEs) planned to be performed between 24td and 25th JVET meetings to evaluate enhanced compression tools beyond VVC capability.

# Introduction

EE focus is to evaluate individual coding technologies and their combinations. Contributions improving compression efficiency further is highly encouraged.

EE related discussions shall happen on JVET and JVET-CE reflectors.

EE tests should be implemented on top the ECM software, ECM-3.0 is used as an anchor in the tests.

Tests shall be performed according to the CTC described in JVET-X2017 with an additional TGM 4:2:0 optional class. TGM tests are required for testing SCC tools.

AI and RA test configurations are required for intra tool testing, while RA and LB test configurations are required for inter tool testing. LP configuration is optional. In LB and LP configurations, the sequences length is reduced to 5 seconds for all classes.

If encoder modification is included in EE tests, such encoder optimization, if applicable, introduced to the anchor should be tested.

# Timeline

T1 = 2 weeks (October 29, 2021) after JVET meeting: ECM is released

T2 = T1 + 1 week (November 5, 2021): EE description is finalized

T3 = T2 + 2 weeks (November 19, 2021): Initial software release for EE tests

T4 = JVET meeting start – 3 weeks (December 21, 2021): Software in EE branches is frozen

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1a | ABT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |  |
| 1.1b | UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |  |
| 1.1c | ABT + UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |  |
| 2 Intra prediction |  |  |  |
| 2.1a | Extended MRL candidate list | Qualcomm Keming Cao | OPPO Kazushi Sato Ofinno Vasily Rufitskiy |
| 2.1b | Extended MRL with the reduced number candidates | Qualcomm Keming Cao | Ofinno Vasily Rufitskiy |
| 3 Inter prediction |  |  |  |
| 3.1a | GPM with intra and inter prediction | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng | OPPO Kazushi Sato |
| 3.1b | 3.1a + DIMD and neighboring mode based IPM derivation + combination of GPM-intra and GPM-MMVD | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |  |
| 3.1c | 3.1b + TIMD-based IPM derivation | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |  |
| 3.1d | 3.1c + combination of GPM-intra and GPM-TM | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |  |
| 3.2 | Modified pairwise candidates | Canon Guillaume Laroche | Bytedance Lei Zhao |
| 3.3a | TMVP improvement method 1 | Alibaba Ru-Ling Liao | OPPO Zhihuang Xie |
| 3.3b | TMVP improvement method 1 + method 2 | Alibaba Ru-Ling Liao | OPPO Zhihuang Xie |
| 3.4 | MV candidate type-based ARMC | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao |  |
| 3.5a | Template matching based merge candidate list construction | Bytedance Lei Zhao | Canon Guillaume Laroche |
| 3.5b | Template matching based merge candidate list construction (Test 3.5a) + Modified pairwise candidates (Test 3.2) | Bytedance Lei Zhao Canon Guillaume Laroche |  |
| 3.6a | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao |  |
| 3.6b | MV candidate type-based ARMC (Test 3.4) + modified pairwise candidates (Test 3.2) | Qualcomm Yao-Jen Chang Canon Guillaume Laroche |  |
| 3.6c | MV candidate type-based ARMC (Test 3.4) + MV candidates from Test 3.5 | Qualcomm Yao-Jen Chang Bytedance Lei Zhao |  |
| 3.6d | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 + Modified pairwise candidates from Test 3.2 + MV candidates from Test 3.5 | Qualcomm Yao-Jen Chang Canon Guillaume Laroche Bytedance Lei Zhao Alibaba Ru-Ling Liao |  |
| 3.7 | Increased number of TM merge candidates | Qualcomm Yao-Jen Chang |  |
| 3.8 | Alternative template matching | Qualcomm Han Huang |  |
| 3.9a | TM based reordering for MMVD | Bytedance Mehdi Salehifar |  |
| 3.9b | TM based reordering for MMVD and affine MMVD | Bytedance Mehdi Salehifar |  |
| 3.9c | TM based reordering for MMVD (Test 3.9a) + MVD sign prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang |  |
| 3.9d | TM based reordering for MMVD and affine MMVD (Test 3.9b) + MVD sign prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang |  |
| 3.10 | MVD sign prediction | Qualcomm Yan Zhang | Ofinno Vasily Rufitskiy |
| 3.11 | Non-adjacent spatial neighbor for affine merge mode | Kwai Wei Chen |  |
| 3.12a | History-parameter-based affine model inheritance | Bytedance Kai Zhang |  |
| 3.12b | History-parameter-based affine model inheritance (Test 3.12a) + non-adjacent spatial neighbors for affine merge mode (Test 3.11) | Bytedance Kai Zhang Kwai Wei Chen |  |
| 3.12c | 3.12b with TM disabled | Bytedance Kai Zhang Kwai Wei Chen |  |
| 3.13 | Modifications of IBC merge/AMVP list construction | Bytedance Na Zhang | Ofinno Damian Ruiz Coll |
| 4 Transform and coefficients coding |  |  |  |
| 4.1a | QIdx based signs selection | Alibaba Jie Chen |  |
| 4.1b | Extension of sign prediction area | Alibaba Jie Chen |  |
| 4.1c | Qidx based signs selection + extension of sign prediction area | Alibaba Jie Chen |  |
| 4.2a | Sign prediction with adaptive coefficient selection | Kwai Xiaoyu Xiu | OPPO Luhang Xu |
| 4.2b | Sign prediction with adaptive coefficient selection + extension of sign prediction to LFNST | Kwai Xiaoyu Xiu | OPPO Luhang Xu |
| 4.3a | Test 4.1c + extension of sign prediction to LFNST (Test 4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu |  |
| 4.3b | Extension of sign prediction area (4.1b) + Sign prediction with adaptive coefficient selection with extension of sign prediction to LFNST (4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu |  |
| 4.4a | Adaptive intra MTS | Qualcomm Bappaditya Ray |  |
| 4.4b | Adaptive intra MTS – encoder only (non-normative) | Qualcomm Bappaditya Ray |  |
| 4.4c | Adaptive intra MTS – with parameter signalling | Qualcomm Bappaditya Ray |  |
| 4.4d | Intra MTS without adaptivity (fixed number of candidates) | Qualcomm Bappaditya Ray |  |
| 5 In-loop filtering |  |  |  |
| 5.1 | Edge-based classifier for CCSAO | Qualcomm Anand Meher Kotra Kwai Che-Wei Kuo |  |
| 5.2a | Adaptive filter shape selection for ALF | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer |  |
| 5.2b | Adaptive filter shape selection for ALF with reduced filter length | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer |  |

# Tools description

## Partitioning

### Test 1.1: Unsymmetric partitioning methods (JVET-X0068)

### Asymmetric Binary Tree (ABT) partitioning

Four new asymmetric binary tree splitting modes are added to the multi-type tree structure of VVC, to allow new splitting configurations. These added split modes are shown on Figure 1.

According to the added split modes, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction.

Figure 1. Split modes supported in the proposed QTBT+TT+ABT framework

### Unsymmetric Quad-Tree (UQT) partitioning

Four types of UQT are proposed. UQT-H1 and UQT-H2 split a CU with dimension W×H into four child-Cus with dimensions {W×H/8, W×H/2, W×H/4, W×H/8} and {W×H/8, W×H/4, W×H/2, W×H/8}, respectively. UQT-V1 and UQT-V2 split a CU with dimension W×H into four child-Cus with dimensions {W/8×H, W/2×H, W/4×H, W/8×H} and {W/8×H, W/4×H, W/2×H, W/8×H}, respectively.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.1a | ABT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |
| 1.1b | UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |
| 1.1c | ABT + UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec |

## Intra prediction

### Test 2.1: Extended MRL candidate list (JVET-X0142)

In the current ECM-2.0 software, MRL candidate list is set to be {1, 2}. In this test, extended MRL candidate list is extended to be {1, 3, 5, 7, 12}. Subset of {1, 3, 5, 7, 12} as MRL candidate list will also be tested. MRL candidate list for TIMD mode is set to be {1, 3}.

Different number of candidates will also be tested and how often the farther away candidates are really used will be examined.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.1a | Extended MRL candidate list | Qualcomm Keming Cao |
| 2.1b | Extended MRL with the reduced number candidates | Qualcomm Keming Cao |

## Inter prediction

### Test 3.1: GPM with intra and inter prediction (JVET-X0166)

Geometric partitioning mode (GPM) with intra and inter is investigated. In the method, the following four modifications are introduced into the previous EE solution (JVET-W0110):

- Restrict GPM intra-intra partitioning

- Direct motion vector and intra prediction mode (IPM) storage on the GPM-blending area

Additionally, DIMD and neighboring mode based IPM derivation, combination of GPM-intra and GPM-MMVD, TIMD-based IPM derivation and combination of GPM-intra and GPM-TM will be tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.1a | GPM with inter and intra prediction | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |
| 3.1b | 3.1a + DIMD and neighboring mode based IPM derivation + combination of GPM-intra and GPM-MMVD | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |
| 3.1c | 3.1b + TIMD-based IPM derivation | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |
| 3.1d | 3.1c + combination of GPM-intra and GPM-TM | KDDI Yoshitaka Kidani LGE Hyeongmun Jang Bytedance Zhipin Deng |

### Test 3.2: Modified pairwise candidates (JVET-X0119)

In this test, the pairwise candidate is removed from the regular merge list and derived during the reordering process of merge candidate list (AMRC). In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.2 | Modified pairwise candidates | Canon Guillaume Laroche |

### Test 3.3: TMVP improvement (JVET-X0091)

In the contribution, the derivation process of TMVP merge candidate is modified. In the first method, when constructing the merge candidate list of non-subblock merge modes, the TMVP is conditionally added to the list according to the motion information of neighboring spatial blocks. And, the reference picture of the TMVP merge candidate is selected from any one of reference pictures in the reference picture list according to a pre-defined rule. The selected reference picture is the one whose scaling factor is the closet to 1. Moreover, in the second method, after the reference picture of TMVP is selected, the template matching method is used to decide the scaling factor and inter prediction direction of the TMVP.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.3a | TMVP improvement method 1 | Alibaba Ru-Ling Liao |
| 3.3b | TMVP improvement method 1 + method 2 | Alibaba Ru-Ling Liao |

### Test 3.4: MV candidate type-based ARMC (JVET-X0133)

In this test, MV candidate type-based ARMC is investigated. In the method, the merge candidates in at least one of candidate types, e.g., TMVP, NA-MVP, SbTMVP, etc., are reordered based on the ARMC TM cost values. The reordered candidates are then added into the merge candidate list. Some candidate types add more merge candidates to perform the reordering and the selection. Combinations with other proposals are also tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.4 | MV candidate type-based ARMC | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao |

### Test 3.5: Template matching based merge candidate list construction (TM-MCLC) (JVET-X0087)

With TM-MCLC, adjacent MVPs, non-adjacent MVPs and HMVPs candidates are put into the merge candidate list in an ascending order of template matching costs. Moreover, non-adjacent MVPs is further extended with more spatial and non-adjacent temporal positions.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.5a | Template matching based merge candidate list construction | Bytedance Lei Zhao |
| 3.5b | Template matching based merge candidate list construction (Test 3.5a) + Modified pairwise candidates (Test 3.2) | Bytedance Lei Zhao Canon Guillaume Laroche |

### Test 3.6: Combination of the ARMC related methods

The following combinations of the ARMC related methods will be tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.6a | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 | Qualcomm Yao-Jen Chang Alibaba Ru-Ling Liao |
| 3.6b | MV candidate type-based ARMC (Test 3.4) + Modified pairwise candidates (Test 3.2) | Qualcomm Yao-Jen Chang Canon Guillaume Laroche |
| 3.6c | MV candidate type-based ARMC (Test 3.4) + MV candidates from Test 3.5 | Qualcomm Yao-Jen Chang Bytedance Lei Zhao |
| 3.6d | MV candidate type-based ARMC (Test 3.4) + TMVP improvement from Test 3.3 + Modified pairwise candidates (Test 3.2) + MV candidates from Test 3.5 | Qualcomm Yao-Jen Chang Canon Guillaume Laroche Bytedance Lei Zhao Alibaba Ru-Ling Liao |

### Test 3.7: Increased number of TM merge candidates (JVET-X0134)

In this test, the number of TM merge candidates and the TM operations are investigated to optimize the current TM merge mode.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.7 | Increased number of TM merge candidates | Qualcomm Yao-Jen Chang |

### Test 3.8: Alternative template matching (JVET-X0146)

In this test, an alternative template matching method is investigated. The template matching cost is calculated based on the prediction hypothesis of current block and the neighboring reconstructed samples. The alternative template matching method is used to design an additional merge mode which is signaled at block level.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.8 | Alternative template matching merge mode | Qualcomm Han Huang |

### Test 3.9: Template matching-based reordering for extended MMVD design (JVET-X0085)

Template matching based reordering method for extended MMVD (TMR-EMMVD) is tested with the following aspects:

- MMVD refinement positions are reordered based on template costs

- Additional positions are added along new angles.

- After the reordering process, selected positions are kept as available positions, consequently for MMVD index coding.

Additionally, the similar concept is also extended to Affine MMVD.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.9a | TM based reordering for MMVD | Bytedance Mehdi Salehifar |
| 3.9b | TM based reordering for MMVD and affine MMVD | Bytedance Mehdi Salehifar |
| 3.9c | TM based reordering for MMVD (Test 3.9a) + MVD sing prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang |
| 3.9d | TM based reordering for MMVD and affine MMVD (Test 3.9b) + MVD sign prediction (Test 3.10) | Bytedance Mehdi Salehifar Qualcomm Yan Zhang |

### Test 3.10: MVD sign prediction (JVET-X0132)

In this test, the MVD sign prediction method is tested with its application to several existing coding tools including:

- MVD sign prediction applied to translational inter AMVP mode

- MVD sign prediction applied to translational inter SMVD mode

- MVD sign prediction applied to affine inter AMVP mode

- MVD sign prediction applied to inter MMVD mode

- MVD sign prediction applied to affine MMVD mode

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.10 | MVD sign prediction | Qualcomm Yan Zhang |

### Test 3.11: Non-adjacent spatial neighbor for affine merge mode (JVET-X0151)

Non-adjacent spatial neighbor for affine merge mode (NSAM) is tested improving the efficiency of affine merge mode. With the NSAM, the motion information of non-adjacent spatial neighbors is utilized to derive additional inherited and constructed affine merge candidates. The derived candidates are then inserted into the existing candidate list of affine merge mode.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.11 | Non-adjacent spatial neighbor for affine merge mode | Kwai Wei Chen |

### Test 3.12: History-parameter-based affine model inheritance (JVET-X0088)

A method of history-parameter-based affine model inheritance (HAMI) is tested. With HAMI, affine parameters of previously affine-coded block are stored in a History-Parameter Table (HPT). New affine merge and AMVP candidates as well as new regular merge candidates can be constructed with affine parameters fetched from HPT and base MVs fetched from neighbouring blocks, and put into the sub-block-based merge candidate list, the affine AMVP candidate list and the regular merge candidate list, respectively. Pruning is applied to avoid redundant candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.12a | History-parameter-based affine model inheritance | Bytedance Kai Zhang |
| 3.12b | History-parameter-based affine model inheritance (Test 3.12a) + Non-adjacent spatial neighbor for affine merge mode (Test 3.11) | Bytedance Kai Zhang Kwai Wei Chen |
| 3.12c | 3.12b with TM disabled | Bytedance Kai Zhang Kwai Wei Chen |

### Test 3.13: Modifications of IBC merge/AMVP list construction (JVET-X0089)

In the test, IBC merge/AMVP list construction process is modified with three aspects:

1) Only if a block vector (BV) candidate is valid, it can be inserted into the IBC merge/AMVP candidate list;

2) More BV candidates, such as those from neighboring blocks and pairwise average candidates, can be added into the IBC merge/AMVP list;

3) ARMC-TM is extended to IBC merge list.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.13 | Modifications of IBC merge/AMVP list construction | Bytedance Na Zhang |

## Transform and coefficients coding

### Test 4.1: Sign prediction (JVET-X0120)

In ECM-2.0, if the number of signs (in the top-left 4x4 block) is larger than the allowable limit, first maxNumPredSigns signs in raster scan order are predicted. However, the raster scan order may not optimal for all TB. Usually, signs of large transform coefficient are relatively easy to predict since the impact of sign error of large transform coefficients on the reconstructed block are relatively higher.  In this test, the maximum maxNumPredSigns are selected based on the qIdx values. The qIdx of the level depends of the DQ state and can be computed as follows:

qIdx = (abs(level) << 1) - (state & 1);

In this test, performance of extending sign prediction area of a TB to maximum 32x32 block will be also tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.1a | QIdx based signs selection | Alibaba Jie Chen |
| 4.1b | Extension of sign prediction area | Alibaba Jie Chen |
| 4.1c | QIdx based signs selection + extension of sign prediction area | Alibaba Jie Chen |

### Test 4.2: Enhanced sign prediction (JVET-X0150)

Methods are tested further improving the performance of the sign prediction. Firstly, the sign prediction is extended to the LFNST mode. Secondly, transform coefficient positions are adaptively selected for the sign prediction by maximizing their impacts on reconstructed border samples.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.2a | Sign prediction with adaptive coefficient selection | Kwai Xiaoyu Xiu |
| 4.2b | Sign prediction with adaptive coefficient selection + extension of sign prediction to LFNST | Kwai Xiaoyu Xiu |

### Test 4.3: Combination of JVET-X0120 and JVET-X0150

Following variations of joint tests will be performed.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.3a | Test 4.1c + extension of sign prediction to LFNST (Test 4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu |
| 4.3b | Extension of sign prediction area (4.1b) + Sign prediction with adaptive coefficient selection with extension of sign prediction to LFNST (4.2b) | Alibaba Jie Chen Kwai Xiaoyu Xiu |

### Test 4.4: Adaptive intra MTS (JVET-X0135)

A variable number of MTS candidates for intra coded blocks depending on the position of last significant coefficient is tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.4a | Adaptive intra MTS | Qualcomm Bappaditya Ray |
| 4.4b | Adaptive intra MTS – encoder only (non-normative) | Qualcomm Bappaditya Ray |
| 4.4c | Adaptive intra MTS – with parameter signalling | Qualcomm Bappaditya Ray |
| 4.4d | Intra MTS without adaptivity (fixed number of candidates) | Qualcomm Bappaditya Ray |

## In-loop filtering

### Test 5.1: Edge-based classifier for CCSAO (JVET-X0105)

This method extends the cross-component SAO (CCSAO) with the edge-based classifier. Similar to the SAO edge classifier, for the new CCSAO edge-based classifier, the sample differences are computed and compared with a given threshold to classify the samples into different classes. The CCSAO edge-based classifier may be used to filter both the luma and chroma components.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.1 | Edge-based classifier for CCSAO | Qualcomm Anand Meher Kotra Kwai Che-Wei Kuo |

### Test 5.2: Adaptive filter shape selection for ALF (JVET-X0086)

In the current ALF design of ECM, the online-trained luma filters only have one fixed shape. In this method, a CTB-level adaptive filter shape selection (AFSS) method for ALF is tested. With AFSS, a filter shape index that is associated with luma filters is signalled for each APS. Each CTB selects the best luma filter shape from the available APSs.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.2a | Adaptive filter shape selection for ALF | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer |
| 5.2b | Adaptive filter shape selection for ALF with reduced filter length | Bytedance Wenbin Yin Qualcomm Nan Hu Alibaba Mohammed Golam Sarwer |