| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Online, 12–19 Jan 2022 | Document: JVET-Y0041-v1 |
| --- | --- |

| Title: | AhG-7 Proposed new class of gaming sequences with depth and optical flow information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | G. Martin-Cocher, M.Badawi T. Poirier, S. Puri, K. Nasser | Email: | first.last @ interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes to create a new class of content (DMV – Depth & Motion Vector) corresponding to the cloud gaming and game casting scenario. The proposed sequences include depth and optical flow information.

# Rational

Game engines provide access to various frame buffers and allow to retrieve information such as Depth and Motion Vector or optical flow. Mobile devices and other accessories such as active cameras are increasingly making use of a variety of sensors, which can be used in video conferencing, telepresence and other industrial scenarios. This information could be useful to enhance the efficiency of compression tools and should particularly be evaluated in low delay / Low Latency Controlled Complexity (LLCC) configurations.

# Creation of the sequences:

10 sequences were created using the Unity game engine to load and play several games. We captured the frame buffers using Unity Recorder and custom post-processing shaders in both the built-in render pipeline and the High-Definition Render Pipeline. We captured the color buffer into an RGBA16 Signed Float texture as well as the depth and optical flow in a second similar texture (with the linear depth info encoded into the R channel, and the [X,Y] coordinates of the motion vectors into the [G,B] channels). The textures were recorded at 60FPS in 1080p and 4K resolutions and written into uncompressed EXR 16 bit images.  Two YUVs sequences were then generated. The depth is stored in the Y channel and converted to 14 bits to avoid high bit-depth coding, vertical and horizontal motion vectors are stored at quarter pixel precision in U/V channels for the corresponding texture frame.

Figure 1 /3D_game _kit_Level-1 sequence Frame #10 top left YUV texture,

top right, depth information (Y), bottom left vertical motion (V), bottom right horizontal motion (U)

# Proposed sequences

All sequences are available on that link:

https://www.interdigital.com/data_sets/gaming-sequences

### 3D_game_kit Sequences description:

Two 10 seconds HD sequences representing two different levels in the 3D_game_kit and one 10 seconds 4K sequence were generated from the Unity Asset 3D game kit available at https://assetstore.unity.com/packages/templates/tutorials/3d-game-kit-115747.

The sequence represents a FPS type of game, where a character is exploring an alien planet.

Figure 2: 3D -game kit level-1 screenshot			 Figure 3: 3D_game_kit_level-2 screenshot

Figure 4: 3D_game_kit_4K

### Darktree Sequence description:

Darktree is a typical first-person shooter (FPS) type of game in a realistic environment generated from the Unity Asset available at:

https://assetstore.unity.com/packages/templates/systems/darktree-fps-v1-4-142383

Figure 5 : DarkTree screenshot

### Fontainebleau Sequence description:

Two 10 seconds HD sequences representing two different modes (cinematic and first person view) in the Fontainebleau demo and one 10 seconds 4K sequence were generated from the Unity Asset available at https://github.com/Unity-Technologies/FontainebleauDemo

The sequences represent the Fontainebleau forest under different lighting conditions.

Figure 6: Fontainebleau-Cinematic 				Figure 7: Fonctainebleau_FPV

Figure 8: Fontainebleau_4K

### Racing Sequence description:

This car racing 10 second-long 4K sequence is generated from a Unity Project from Pablo Labs. A video of the project is available at: https://www.youtube.com/watch?v=19vUeWcoD6I and the assets at https://www.mediafire.com/file/fvv2dpakkih4mum/Assets.zip/file.

The sequence contains textual overlays.

Figure 9 : Racing screenshot

### Sequence characteristics:

| Sequence name | Resolution | Input bit-depth | Frame rate | frame count | Md5 sum |
| --- | --- | --- | --- | --- | --- |
| 3D_game_kit_Level-1_1920x1080_60p_420_10bits 3D_game_kit_Level-1_DepthMotion_1920x1080_60p_444_14bits | 1920x1080 1920x1080 | 10 14 | 60 60 | 600 600 | 760ade9249e50f3cd2e5c607121dd767 1dbb5c8904c0b931c49a15264e6920c9 |
| 3D_game_kit_Level-2_1920x1080_60p_420_10_bits 3D_game_kit_Level-2_DepthMotion_1920x1080_60p_444_16_bits | 1920x1080 1920x1080 | 10 .14 | 60 60 | 600 600 | d05fa25c96e62cfcbf2283f852947ad9 b6a55026dd246dd24af9b4af2d147e8b |
| Darktree_1920x1080_60p_420_10_bits Darktree_DepthMotion_1920x1080_60p_444_14_bits | 1920x1080 1920x1080 | 10 .14 | 60 60 | 600 600 | 8ebadc06ad943c2f4f0f2fa4906db87a 797c38b7a619df011055d7a352a0d290 |
| Fontainebleau_Cinematic_1920x1080_60p_420_10_bits Fontainebleau_Cinematic_DepthMotion_1920x1080_60p_444_14_bits | 1920x1080 1920x1080 | 10 .14 | 60 60 | 600 600 | 612a335f6fc672e04ca06f116b411e5f 226c083acd571528a03a445d51545df3 |
| Fontainebleau_FPV_1920x1080_60p_420_10_bits Fontainebleau_FPV_DepthMotion_1920x1080_60p_444_14_bits | 1920x1080 1920x1080 | 10 .14 | 60 60 | 600 600 | 0672ea5eb4f7826549130a024e5bb0d4 bbd94d368eb229a8f1f88b6f507a9bbe |
| 3D_game_kit_4K_3840x2160_60p_420_10bits 3D_game_kit_4K_DepthMotion_3840x2160_60p_444_14bits | 3840x2160 3840x2160 | 10 14 | 60 60 | 600 600 | 7f667d0f6575d817c555353ab4386ecd 873df23f5e38b9391b94c5fee7cf5db0 |
| Racing _3840x2160_60p_420_10bits Racing _DepthMotion_3840x2160_60p_444_14bits | 3840x2160 3840x2160 | 10 14 | 60 60 | 600 600 | 7f6272b01b77945634fb3de21f6e850a f71313a9c3becfe6410b1ec04941b5b9 |
| Fontainebleau_Cinematic_3840x2160_60p_420_10_bits Fontainebleau_Cinematic_DepthMotion_3840x2160_60p_444_14_bits | 3840x2160 3840x2160 | 10 14 | 60 60 | 600 600 | fa0f48ea1855a0ed74aea3a183387910 3d61f5a2a85c2881e101dd0251d3e5a5 |

# Copyright

This video sequences may only be used for the purpose of developing, testing and promulgating technology standards developed by a standardization group (“Purpose”), under InterDigital ("InterDigital") owned or controlled copyrights, by individual(s) or organization(s) that participates, contributes or is part of such standardization groups ("User(s)").

- The 3D_game_kit sequences contain the 3D-game-kit Unity assets created by Unity Technologies available at https://assetstore.unity.com/packages/templates/tutorials/3d-game-kit-115747.

- The racing sequence contains the “Unity Racing Game” Unity assets created by Pablo labs available at https://www.mediafire.com/file/fvv2dpakkih4mum/Assets.zip/file .

- The Fontainebleau sequences contain the FontainebleauDemo Unity assets created by Unity Technologies available at https://github.com/Unity-Technologies/FontainebleauDemo.

- The DarkTree sequence contains the DarkTree DPS v1.4 Unity assets created by DarkTreeDevelopment available at  https://assetstore.unity.com/packages/templates/systems/darktree-fps-v1-4-142383.

All these sequences are licensed by their respective owners and it is the User responsability to comply with the license terms that are provided by such owner. InterDigital claims no ownership nor liability on these assets.

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of InterDigital or its affiliates is granted.

Any other use is strictly prohibited. This video cannot be distributed (freely or not) or otherwise disposed of or made available (via internet or otherwise), broadcasted, disclosed to third parties (except to other User(s) who agreed with the  present terms and conditions, the "Secondary Users"), used for providing any services to third parties or for any commercial use, nor copied (except as technically necessary for the Purpose), modified, adapted, translated, exchanged, publicly performed (even for research, demonstration or educational purposes), integrated, without limitation. This video and all intellectual property rights, titles and interests therein, are and remain the exclusive property of InterDigital or its affiliates or, regarding the Unity assets, of its legitimate owner.

NO WARRANTY: This video is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purposes, or any warranty arising by statute or by law, or non-infringement of any third parties's rights. In no event will InterDigital be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if InterDigital has been advised of the possibility of such damages.

InterDigital may terminate this license it at any time for any reason upon notice to User(s). In such situation, the relevant User(s) shall inform any Secondary User of such termination and make sure that such Secondary User shall cease to use the video.

This authorization is governed by the laws of France without regard to conflict of laws. It shall be exclusively submitted to the first instance civil court of Paris, France in the event of a dispute not settled amicably.

BY USING THIS VIDEO USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO THESE TERMS AND CONDITIONS, IT SHALL NOT USE THIS VIDEO. DO NOT REMOVE OR MODIFY INTERDIGITAL OR ITS AFFILIATES' NAMES, SIGNS, LOGOS, NOTICES.

# Some results

Table 1: ECM-3.0 vs VTM-15.0 for the proposed sequences

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class DMV HD | -8.86% | -26.20% | -23.98% | 321% | 375% |
| Class DMV 4K | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class DMV HD | -9.08% | -26.95% | -24.42% | 289% | 340% |
| Class DMV 4K | -10.91% | -25.64% | -25.44% | 277% | 361% |
| Overall | -9.77% | -26.46% | -24.80% | #DIV/0! | #DIV/0! |

# Proposal

Adopt a new class (DMV) of content with texture, depth and optical flow information, select sequences from the one proposed and call for additional sequences as needed.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.