| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0066-v1 |
| --- | --- |

| Title: | Update on Ali266, the optimized VVC encoder implementation by Alibaba |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Dong, S. Fang, Z. Huang, J. Liu, S. Xu, R. Yang, L. Yu, J. Chen, R.-L. Liao, Y. Ye | Tel: Email: | yan.ye@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports updated information on runtime and coding efficiency of Ali266, an optimized software VVC encoder developed by Alibaba. Ali266 was first presented in JVET-W0127 [1], it supported two presets, with its fast preset aimed at real-time encoding applications. In JVET-W0127, Ali266 was able to achieve encoding speed of 37.7 fps for 720p sequences.

In JVET-X0104 [2], we reported further improvement in Ali266’s encoding speed at its fast preset. Under the configuration of fast preset with 8-bit internal bit depth, we reported that a 1.9x encoding speedup had been achieved compared with that in JVET-W0127 for video sequences at 1920x1080 or 1080x1920 resolutions. The average encoding speeds achieved 36.3 fps for classes 1920x1080 / 1080x1920 and 63.4 fps for classes 1280x720 / 720x1080. Based on such data, we concluded that Ali266 was a VVC encoder capable of real-time encoding for 1080p sequences.

In this document, further updates to Ali266 are reported. First, Ali266 was enhanced with 4 predefined quality/speed presets, slow, medium, fast, and faster, to meet different application needs. Furthermore, we have not only achieved 10-bit real time encoding for 1080p video-on-demand (VOD) content with the “faster” preset, but also improved the coding efficiency. Our experiments demonstrate that Ali266 achieves BD rates for {YUV-PSNR, YUV-SSIM} of {-42.01%, -42.90%} over the widely used open-source HEVC software encoder x265 at the medium preset, compared to the previously reported results of {-38.10%, -41.10%} in JVET-X0104 over the same anchor.

Introduction

In JVET-W0127 and JVET-X0104, the Ali266 encoder was reported to respectively achieve 8-bit 720p and 1080p real-time encoding on a mainstream PC. At this meeting, the Ali266 encoder is further accelerated to achieve 10-bit 1080p real-time encoding. To accelerate VVC’s adoption by a wider range of video applications, we have taken more efforts to improve Ali266’s encoding speed and coding efficiency.

The main differences from JVET-X0104 include:

- More fast encoding algorithms suitable for VVC encoder were employed.

- More extensive SIMD instructions were used to reduce the required clock cycles, especially for the 10-bit internal bit depth.

- Reduced memory consumption and improved memory access efficiency.

- Better support of parallel encoding to reduce CPU idle time.

- More predefined quality/speed presets to meet different application needs.

Simulation results

Real-time encoding performance of Ali266

The encoding performance of Ali266 at the faster preset is aimed at achieving real-time VVC encoding and maintaining large coding efficiency over HEVC real-time software encoder implementation. For this purpose, we selected the widely used open source HEVC software encoder x265 at the medium preset as the anchor. For content we use a set of 20 sequences at 720p and 1080p resolutions in both landscape and portrait modes. These represent real-world VOD content in our cloud computing business.

The weighted combined YUV-PSNR and YUV-SSIM of the Y, U, and V components using a weighting factor of 6 for the luma component and 1 for each chroma component are used in the BD-rate calculation. The negative sign in table represents the average bitrate savings while maintaining the same YUV-PSNR or YUV-SSIM for the reconstructed video. With 10-bit internal bit depth, the PSNR and SSIM BD-rates of Ali266 faster preset over x265 at medium preset are shown in Table 1. As can be seen, compared to x265 at the medium preset, Ali266 at the faster preset achieves BD-rate savings of 42.01% and 42.90% for PSNR and SSIM based measurement, respectively. In terms of encoding speed, Ali266 achieves average encoding speeds of 34.30 fps and 30.62 fps for classes 1920x1080 and 1080x1920, and 61.74 fps and 58.01 fps for classes 1280x720 and 720x1280, respectively. Especially, the average 10-bit encoding speed reaches 32.46 fps for 1080p sequences. Therefore, it is our conclusion that software VVC encoder is capable of real-time 10-bit encoding for 1080p sequences, and is ready for real-world commercial applications. Ali266’s coding efficiency and encoding speed over x265 at the medium preset and the same test set as previously reported in JVET-W0127 and in JVET-X0104 are also provided in Table 1 for comparison, and show the progress of Ali266 as a faster and more efficient VVC encoder implementation. It should be pointed out that for Ali266 and x265, both of them performed 8-bit encoding in JVET-W0127 and JVET-X0104, but this time both of them performed 10-bit encoding.

Table 1: Encoding performance of the Ali266 encoder @ faster preset

| Class | RA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JVET-W0127 （Internal bit-depth = 8） |  |  | JVET-X0104 （Internal bit-depth = 8） |  |  | JVET-Y0066 （Internal bit-depth = 10） |  |  |
|  | YUV-PSNR | YUV-SSIM | Speed (fps) | YUV-PSNR | YUV-SSIM | Speed (fps) | YUV-PSNR | YUV-SSIM | Speed (fps) |
| 1920x1080 | -34.99% | -39.59% | 19.99 | -40.58% | -43.00% | 39.13 | -43.90% | -44.84% | 34.30 |
| 1080x1920 | -36.58% | -37.06% | 18.61 | -36.86% | -40.69% | 33.46 | -40.52% | -42.39% | 30.62 |
| 1280x720 | -35.12% | -31.82% | 38.77 | -40.27% | -39.22% | 64.52 | -43.11% | -40.08% | 61.74 |
| 720x1280 | -34.73% | -40.04% | 36.66 | -34.68% | -41.50% | 62.23 | -40.52% | -44.30% | 58.01 |
| Average | -35.35% | -37.13% | - | -38.10% | -41.10% | - | -42.01% | -42.90% | - |

Ali266’s Quality/Speed Presets in comparison with x265

Through different algorithm combinations, Ali266 provides 4 predefined quality/speed presets to meet different application needs: slow, medium, fast, and faster. The encoding speed and performance trade-off points of different quality/speed presets of Ali266 in comparison with different presets of x265 are shown in Figure 1.

Figure 1: Performance of Ali266 compared with x265

For the horizontal axis in Figure 1, the encoding speed is expressed by the ratio of encoding frame rate relative to that of HM-16.23. For the vertical axis, the encoding performance is expressed by the BD-rate over HM-16.23, where lower number represents higher compression performance. Both PSNR-based BD-rate and SSIM-based BD-rate are plotted in Figure 1. The PSNR and SSIM numbers are collected using the same encoding run (i.e., not separately optimized for either quality metric). Different trade-off points of x265 are also plotted in Figure 1 for comparison. The same set of internal test content is used.

As can be seen in Figure 1, Ali266 can provide about 40% bitrate savings over HM-16.23 at the “slow” preset in terms of PSNR, or over 50% bitrate savings over HM-16.23 in terms of SSIM. The speed of Ali266’s slow, medium, and fast presets are close to the speed of x265’s veryslow, slower, and slow presets, respectively. Through the comparison of these three sets of presets, it is evident that Ali266 provides large bitrate savings (of 50-60%) over x265 encodings with similar encoding speeds.

At present, the speed of Ali266’s faster preset is about 70% of that of x265’s medium preset. The faster preset is still being optimized to improve its encoding speed at a minimum coding efficiency loss, while more algorithms are being implemented and optimized to improve the coding efficiency.

Conclusion

This contribution presents the progress of Alibaba’s optimized VVC encoder implementation, Ali266. With Ali266’s faster preset, more than 42% of bitrate savings in terms of both YUV-PSNR and YUV-SSIM are achieved over x265 at the medium preset; with the average 10-bit encoding speed of 1080p sequences achieving 32.46 fps, we claim that 10-bit real-time encoding of HD content is accomplished. In addition to long-term efforts to improve encoding speed and performance, we are also committed to applying Ali266 to Alibaba’s various video businesses, starting with our collaborative effort with the Youku team as reported in JVET-Y0122 [4].

References

- S. Fang el al., “Ali266: an optimized VVC software encoder implementation”, JVET-W0127, Joint Video Experts Team (JVET), July 2021.

- X. Dong el al., “Update on the progress of the optimized VVC encoder implementation, Ali266”, JVET-X0104, Joint Video Experts Team (JVET), Oct 2021.

- F. Bossen el al., “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Joint Video Experts Team (JVET), October 2020.

- Y. Jia el al., “Ali266 @ Youku: trial deployment of VVC for video streaming”, JVET-Y0122, Joint Video Experts Team (JVET), January 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).