| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0080-v2 |
| --- | --- |

| Title: | EE1-1.1-related: alternative filter designs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Xiaozhong Xu Shan Liu | Email: | liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, two neural network based in-loop filters are proposed as the alternative filter designs for EE1.1. Multiple models and single model are trained for the proposed filter 1 and filter 2, respectively. In addition to the reconstruction image, other side information is fed into the network, such as prediction image, partition image. In this contribution, these two filters are both evaluated on EE1 anchor. The proposed filter 1 can reportedly achieve {10.63%, 26.10%, 25.78%}, {9.43%, 21.97%, 20.54%} and {7.53%, 20.45%, 20.14%} BD-rate savings with RA, LB and AI configurations, respectively. The proposed filter 2 can reportedly achieve {8.16%, 19.21%, 20.47%}, {x.xx%, x.xx%, x.xx%} and {6.51%, 15.47%, 16.94%} BD-rate savings with RA, LB and AI configurations, respectively.

Introduction

Based on the network in JVET-W0113[], more side information is utilized to accommodate different coding situations. Even if the base QP is given, it seems that the filtering of B slices is more complicated than the I slice, because of multiple slice level QPs used, the dependency among the reference frames and so on. In response to this problem, adaptive model selection is used in filter 1, which is further refined based on JVET-X0054 []. Alternatively, slice QPs are fed into the network in order to distinguish different situations. In addition, to further decrease the model storage, the number of models is decreased to only one model for the filter 2.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF.

Network architecture

Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] and the partition image (par_yuv) [] may be fed into the network. Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block. For the filter 1, it needs to be explained that the partition image is only fed for the I slice, but not for the B slice.

Filter 1: the networks of the luma component and chroma components are shown in Fig. 1 and Fig. 2, respectively. In the network of the luma component, the related chroma information is fed, and the related luma information is also fed in the network of the chroma component. Certainly, the related chroma inputs should firstly be up-sampled to the luma size.

Filter 2: the network is shown in Fig. 3, which is similar with the network architecture in the EE1-1.1.1 []. However, the partition image is not used as an input.

Compared with JVET-W0113 [1], the ResBlock is also modified as shown in Fig. 4. With the same spirit in JVET-W0113, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. In addition, the convolutional block attention module (CBAM) is inserted, and the convolution layer in the skip-layer connection is removed.

Fig. 1: The luma network architecture for the filter 1

Fig. 2: The chroma network architecture for the filter 1

Fig. 3: The network architecture for the filter 2

Fig. 4: The Resblock architecture

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Filter 1 | Filter 2 |
| Mandatory | GPU Type | Tesla V100 SXM2 32GB |  |
|  | Framework: | Torch v1.4.0 |  |
|  | Number of GPUs per Task | 1 |  |
|  | Epoch: | ~1000 |  |
|  | Batch size: | 64 |  |
|  | Loss function: | L1, L2 | L1 |
|  | Training time: | ~200h |  |
|  | Training data information: | DIV2K, TVD, BVI-DVC |  |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | additional QP17 with RA configuration |  |
| Optional |  |  |  |
|  | Number of iterations | 1200 |  |
|  | Patch size | Luma: 144x144, Chroma: 72x72 | 144x144 |
|  | Learning rate: | 1e-4 |  |
|  | Optimizer: | ADAM |  |
|  | Preprocessing: | flipped, rotated |  |
|  | Mini-batch selection process: | random cropped |  |
|  | Other information: |  |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Filter 1 | Filter 2 |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | Torch v1.4.0 |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | ~2M | 1.92M |
|  | Total Number of Parameters (All Models) | 42.78M | 1.92M |
|  | Parameter Precision (Bits) | 32 |  |
|  | Memory Parameter (MB) | ~8MB / model, 22 models in all | ~8MB / model, 1 model in all |
|  | Multiplay Accumulate (MAC)/pixel | 795K | 616K |
| Optional |  |  |  |
|  | Total Conv. Layers | 267 | 261 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | Luma: 144x144, Chroma: 72x72 | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Implementation

- For I slices, Deblock and SAO are both disabled, and the proposed filter is placed before ALF.

- For B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and SAO.

- For the filter 1, multiple results are inferred by three models for the B slice, and these models are indexed by {qp, qp-5, qp-10}, where the qp indicates the base QP.

- The proposed filter can be turned on/off at the CTU level and slice level.

- For the filter 2, a scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is same with the used scaling process in [].

Results

In the experiments, the anchor and test conditions follow the description in [][]. The corresponding results of the filter 1 are provided in Table 3-Table 5. The corresponding results of the filter 2 are provided in Table 6-Table 8.

Filter 1

Table 3. Performance of the proposed method (RA, filter 1)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -12.92% | -10.27% | -19.46% | -22.28% | 257% | 79343% |
| Class A2 | -14.85% | -11.18% | -24.46% | -27.27% | 246% | 78401% |
| Class B | -14.83% | -10.18% | -30.86% | -26.66% | 255% | 76232% |
| Class C | -14.85% | -11.04% | -26.36% | -26.18% | 204% | 68815% |
| Class E |  |  |  |  |  |  |
| Overall | -14.46% | -10.63% | -26.10% | -25.78% | 239% | 75196% |
| Class D | -16.32% | -12.62% | -27.24% | -27.60% | 197% | 64882% |
| Class F | -6.31% | -4.00% | -14.01% | -12.43% | 348% | 16956% |

Table 4. Performance of the proposed method (LB, filter 1)

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.18% | -9.50% | -11.41% | -21.02% | 252% | 79258% |
| Class A2 | -12.78% | -9.69% | -19.37% | -24.69% | 246% | 71483% |
| Class B | -11.88% | -8.90% | -22.07% | -19.57% | 242% | 67152% |
| Class C | -13.26% | -10.42% | -21.85% | -21.75% | 199% | 63688% |
| Class E | -11.37% | -9.00% | -18.90% | -18.06% | 457% | 40603% |
| Overall | -12.39% | -9.43% | -21.97% | -20.54% | 266% | 58179% |
| Class D | -14.74% | -12.26% | -21.73% | -22.59% | 191% | 57895% |
| Class F | -6.46% | -4.33% | -14.32% | -11.39% | 351% | 21262% |

Table 5. Performance of the proposed method (AI, filter 1)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.61% | -6.68% | -17.43% | -19.37% | 214% | 52715% |
| Class A2 | -10.53% | -6.76% | -21.69% | -22.01% | 165% | 45545% |
| Class B | -10.44% | -6.77% | -23.06% | -19.86% | 154% | 43055% |
| Class C | -10.44% | -7.91% | -17.06% | -19.01% | 130% | 30400% |
| Class E | -12.87% | -9.91% | -22.41% | -21.03% | 163% | 48542% |
| Overall | -10.72% | -7.53% | -20.45% | -20.14% | 160% | 42446% |
| Class D | -10.46% | -7.88% | -17.36% | -19.04% | 125% | 29433% |
| Class F | -5.25% | -3.59% | -10.42% | -10.02% | 136% | 15768% |

Filter 2

Table 6. Performance of the proposed method (RA, filter 2)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.15% | -9.42% | -13.73% | -18.99% | 127% | 39763% |
| Class A2 | -12.01% | -8.79% | -20.40% | -22.93% | 125% | 38773% |
| Class B | -10.97% | -7.78% | -20.83% | -20.27% | 127% | 40793% |
| Class C | -10.46% | -7.21% | -20.41% | -19.99% | 117% | 35365% |
| Class E |  |  |  |  |  |  |
| Overall | -11.08% | -8.16% | -19.21% | -20.47% | 124% | 38674% |
| Class D | -11.08% | -8.47% | -18.57% | -19.27% | 115% | 33077% |
| Class F | -4.91% | -2.77% | -11.68% | -10.97% | 145% | 19063% |

Table 7. Performance of the proposed method (LB, filter 2)

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -10.63% | -7.10% | -21.05% | -21.39% | 114% | 32902% |
| Class E | -9.78% | -7.84% | -15.33% | -15.84% | 161% | 39962% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.95% | -8.24% | -18.16% | -19.99% | 112% | 30429% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 8. Performance of the proposed method (AI, filter 2)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.21% | -6.41% | -12.38% | -14.89% | 158% | 27552% |
| Class A2 | -8.93% | -5.89% | -17.25% | -18.83% | 131% | 22086% |
| Class B | -8.65% | -5.90% | -16.46% | -17.28% | 125% | 20549% |
| Class C | -8.55% | -6.22% | -14.87% | -16.21% | 114% | 14894% |
| Class E | -10.66% | -8.63% | -15.93% | -17.51% | 129% | 23614% |
| Overall | -8.93% | -6.51% | -15.47% | -16.94% | 129% | 20808% |
| Class D | -8.33% | -6.29% | -13.00% | -15.96% | 111% | 14219% |
| Class F | -5.49% | -3.67% | -10.92% | -10.99% | 109% | 18668% |

Conclusions

The alternative neural network based in-loop filters for EE1.1 are presented in this contribution. Good coding performance can be achieved by applying the proposed methods.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[] L. Wang, X. Xu, S. Liu. AHG11: neural network based in-loop filter with adaptive model selection. JVET-X0054, 24th Meeting, by teleconference, 6–15 October 2021.

[] H. Zhu, X. Xu, S. Liu, “Deep learning based in-loop filter with QP and residual distribution information,” Doc. AVS-M5654, Aug. 2020.

[] W. Lin, X. He, X. Han, D. Liu, J. See, J. Zou, H. Xiong, F. Wu, “Partition-Aware Adaptive Switching Neural Networks for Post-Processing in HEVC,” IEEE Transactions on Multimedia, vol. 22, no. 11, pp. 2749–2763, Nov. 2020.

[] L. Wang, X. Xu, S. Liu. EE1-1.1: neural network based in-loop filter with constrained storage and low complexity. JVET-Y0078, 25th Meeting, by teleconference, 12–21 January 2022.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016, 24th Meeting, by teleconference, 6–15 October 2021.