| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: online, 12–19 Jan 2022 | Document: JVET-Y0082-v1 |
| --- | --- |

| Title: | EE1 test 3.1: intra prediction using neural networks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T.Dumas, F.Galpin, P.Bordes, E. François | Email: | thierry.dumas@interdigital.com franck.galpin@interdigital.com philippe.bordes@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents the results of the integration into ECM-3.1 of the single additional neural network-based intra prediction mode described in the EE1-related contribution JVET-X0118 [1]. The average BD-rate reduction with respect to ECM-3.1 is summarized as

- RA:  -0.73%  -0.13%  -0.23%

- AI:  -1.78%  -1.30%  -1.15%

# Introduction

A single additional neural network-based intra prediction has been studied in AHG11 for several meeting cycles. Especially, during the JVET meeting held on October 2021, JVET-X0118 [1] described a low-complexity version of this neural network-based intra prediction mode inside VTM-11-NNVC. The average BD-rate reduction of VTM-11-NNVC including the low-complexity neural network-based intra prediction mode, the neural networks featuring 16-bit signed integer parameters and intermediate representations, with respect to VTM-11-NNVC are -2.87% -2.32% -2.37% and -1.38% -0.55% -0.82% in AI and RA configurations respectively. These figures were cross-checked within the EE1 for the JVET meeting on October 2021. The (worst-case) complexity of the low-complexity neural network-based intra prediction mode can be evaluated at 16797 MAC/pixel.

This contribution recalls the results of the integration of the proposed algorithm into VTM-11-NNVC and reports the outcome of the integration of the same algorithm into ECM-3.1.

# Test 3.1.1: composition of the neural network-based mode

The single additional neural network-based intra prediction mode inserted into VTM-11-NNVC is identical to the one specified in Section 2 of JVET-X0118 [1].

# Test 3.1.2: composition of the neural network-based mode

The single additional neural network-based intra prediction mode put into ECM-3.1 is identical to one described in Section 2 of JVET-X0118 [1], excluding four minor changes, see Sections 3.1, 3.2, 3.3, and 3.4.

## Removing the neural network predicting the blocks of size 32x32

The neural network predicting the blocks of size 32x32 is removed from the neural network-based intra prediction mode. Consequently, the neural network-based intra prediction mode now contains 7 neural networks, each predicting blocks of a different size in S=4×4, 8×4, 16×4, 32×4, 8×8, 16×8, 16×16. In particular, f16, 16 . , θ16,16 denotes the neural network predicting 16×16 blocks, its parameters being gathered in θ16,16. A given block of size 32×32 is now predicted via f16, 16 . , θ16,16 via the following steps. The extended set of decoded reference samples, called context, surrounding a given 32×32 block is downsampled horizontally and vertically by 2 before being fed into f16, 16 . , θ16,16, and the neural network prediction is interpolated horizontally and vertically by 2. The same goes for a 64×64 block, but the horizontal and vertical downsampling factors are 4.

## Prediction of the blocks of size 128x128

A given block of size 128×128 is now predicted via f16, 16 . , θ16,16 via the steps described in Section 2.1, using 8 as horizontal and vertical downsampling factors.

## Retraining

The neural networks of the neural network-based intra prediction mode are retrained, in the same conditions as in JVET-X0118.

## Fully-connected neural network predicting the blocks of size 16×16

f16, 16 . , θ16,16 now features a fully-connected architecture, identical to that of the other neural networks in the neural network-based intra prediction mode. The “pruning” presented in JVET-X0118 [1] applies to θ16,16.

# Information on the neural networks in Test 3.1.2

| Test 3.1.2: network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 1.9.0 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 100 |
|  | Training time | 8 hours |
|  | Training data information | The set of images used to create the training sets combines frames extracted from the BVI-DVC dataset, the TVD dataset, and the RGB images in the CLIC2020 training set for low-rate image compression, converted into YCbCr. |
|  | Training configurations for generating compressed training data | VTM-8.0; QP {22, 27, 32, 37} |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context. |
|  | Learning rate | 1e-4 |
|  | Optimizer | ADAM |
|  | Loss function | Mean-squared error. A l1-norm weight decay term is added to the loss function for training the neural network. |
|  | Preprocessing | See JVET-X0118 |

| Test 3.1.2: network information in the inference stage |  |
| --- | --- |
| Parameter number | Weights at 0 not counted 4×4 block: 130049 parameters. 8×4 block: 140501 parameters. 16×4 block: 161926 parameters. 32×4 block: 204764 parameters. 8×8 block: 171983 parameters. 16×8 block: 216977 parameters. 16×16 block: 371061 parameters. |
| Parameter precision | 16-bit signed integer. |
| MAC/pixel | 4×4:8128 MAC/pixel. 8×4 block: 4389 MAC/pixel. 16×4 block: 2529 MAC/pixel. 32×4 block: 1599 MAC/pixel. 8×8 block: 2686 MAC/pixel. 16×8 block: 1694 MAC/pixel. 16×16 block: 1449 MAC/pixel. |
| Total conv. layers |  |
| Total FC layers | See Table 1, Table 4, and Table 7 in [2]. |
| Mem.T (MB) | 1.48 MB |
| GPU Type | No GPU |
| CPU Type | Xeon 2.4GHz |
| Framework | SADL (C++ with a single thread) |

# Results

The proposed algorithm has been tested on top of both VTM-11-NNVC and ECM-3.1, the used neural networks having 16-bit signed integer parameters and intermediate representations.

## Test 3.1.1: integration of the neural network-based mode into VTM-11-NNVC

### AI configuration

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -3.76% | -2.87% | -3.21% | -4.02% | -2.38% | -3.02% | 207% | 1802% | 3% |
| Class A2 | -2.09% | -1.91% | -2.24% | -2.21% | -1.38% | -1.74% | 222% | 1767% | 2% |
| Class B | -2.49% | -1.97% | -2.10% | -2.32% | -1.60% | -1.38% | 221% | 1735% | 2% |
| Class C | -2.53% | -1.86% | -1.95% | -2.59% | -1.12% | -1.03% | 218% | 1790% | 2% |
| Class E | -3.88% | -3.39% | -2.67% | -4.10% | -3.18% | -1.20% | 222% | 2023% | 3% |
| Overall | -2.87% | -2.32% | -2.37% | -2.94% | -1.85% | -1.60% | 218% | 1809% | 2% |
| Class D | -2.47% | -2.05% | -2.08% | -2.44% | -1.53% | -1.22% | 211% | 1749% | 2% |
| Class F | -1.82% | -1.19% | -1.18% | -1.90% | -0.83% | -0.60% | 137% | 993% | 2% |
| Class H |  |  |  |  |  |  |  |  |  |

### RA configuration

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 NNVC |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -2.02% | -0.78% | -1.47% | -1.95% | -0.60% | -1.39% | 110% | 327% | 1% |
| Class A2 | -1.00% | -0.36% | -0.73% | -1.00% | 0.40% | -0.18% | 102% | 208% | 1% |
| Class B | -1.35% | -0.43% | -0.59% | -1.22% | -0.04% | 0.48% | 108% | 264% | 1% |
| Class C | -1.24% | -0.69% | -0.70% | -1.22% | -0.25% | 0.25% | 112% | 341% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.38% | -0.55% | -0.82% | -1.32% | -0.12% | -0.09% | 108% | 281% | 1% |
| Class D | -1.11% | -0.67% | -0.60% | -1.18% | 0.30% | 0.25% | 108% | 283% | 1% |
| Class F | -0.93% | -0.36% | -0.53% | -0.76% | -0.28% | 0.28% | 106% | 191% | 1% |
| Class H |  |  |  |  |  |  |  |  |  |

## Test 3.1.2: integration of the neural network-based mode into ECM-3.1

NOTE: the rate-distortion results below have been generated using 4 QPs, as in the CTCs for ECM-3.1 in AHG12. The simulations have been re-run using 5 QPs, as in the CTCs in AHG11. The rate-distortion results will be updated in the next version of this contribution.

### AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.02% | -1.22% | -1.32% | 172% | 521% |
| Class A2 | -1.39% | -1.06% | -1.05% | 181% | 636% |
| Class B | -1.59% | -1.09% | -1.03% | 188% | 700% |
| Class C | -1.63% | -1.24% | -1.16% | 188% | 847% |
| Class E | -2.47% | -2.03% | -1.28% | 190% | 819% |
| Overall | -1.78% | -1.30% | -1.15% | 184% | 702% |
| Class D | -1.64% | -1.22% | -1.39% | 187% | 931% |
| Class F | -0.93% | -0.35% | -0.56% | 158% | 379% |
| Class TGM |  |  |  |  |  |

### RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | -0.16% | -0.45% | 106% | 140% |
| Class A2 | -0.55% | 0.09% | -0.22% | 109% | 120% |
| Class B | -0.71% | -0.06% | -0.11% | 112% | 137% |
| Class C | -0.75% | -0.37% | -0.21% | 117% | 150% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.13% | -0.23% | 111% | 137% |
| Class D | -0.73% | -0.34% | -0.91% | 121% | 166% |
| Class F | -0.45% | 0.03% | -0.10% | 113% | 141% |
| Class TGM |  |  |  |  |  |

# Conclusion

The integration of the single additional neural network-based intra prediction mode into ECM-3.1 provides average BD-rate reduction of -1.78% -1.30% -1.15% in AI configuration. It yields average BD-rate reduction of -0.73% -0.13% -0.23% in RA configuration. The encT/decT times in RA configuration are 111% / 137% in average. The (worst-case) complexity of the single additional neural network-based intra prediction mode is estimated at 8128 MAC/pixel.

# References

- T. Dumas, F.Galpin, P. Bordes, F. Le Léannec, “EE1-3.1: BD-rate gains vs complexity of NN-based intra prediction”, 24th JVET meeting, 6 - 15 October 2021, JVET-X0118.

- T. Dumas, F.Galpin, P. Bordes, F. Le Léannec, “AHG11: neural network-based intra prediction with transform selection in VVC”, 20th meeting, 7 - 16 October 2020, JVET-T0073.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).