| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0087-v2 |
| --- | --- |

| Title: | AHG11: An Improved CNN-based Super Resolution Method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Dong Jiang, Jucai Lin Cheng Fang, Xue Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based super-resolution used as upsampling filter for Y component. Before entering the network, the luma block is downsampled by RPR and then encoded. The proposed method is developed from the previous contribution JVET-X0097. In this contribution, the residual block has more convolutional layers and the input adds side information in this network.

Compared with VTM-11.0-NNVC, the experimental results show 9.16% BD-rate gain and 0.52% BD-rate loss on average for luma, under AI and RA configuration, respectively.

Introduction

A super-resolution method based on CNN is used to improve the compression efficiency in JVET-X0097 [1]. To further improve the coding performance, the number of convolutional layers in the residual block is increased. In addition, some side information is used as input.

Proposed method

Details of the model architecture, training and inference process are described below.

Architecture

The architecture of the proposed CNN model is shown in Fig.1. The network contains 16 advanced residual blocks(ARB), several convolutional layers, a concatenate layer, a shuffle layer and a shortcut connection from conv1 to conv2.

The parameters of each convolution layer are [cin, k, k, cout], where cin, k and cout represent the number of input channels, the size of convolution kernel and the number of output channels respectively.

The reconstruction, prediction and quantization parameter are denoted as YLR-Rec, YLR-Pred and QP, which are used as the input of the model. QP is expanded to a matrix of the same shape as YLR-Rec before entering the concatenate layer. The output of the model is the super-resolution reconstruction YSR.

In addition, the architecture of the designed ARB is shown in Fig.2, where ReLU, GAP and Softmax are activation function, global average pooling layer and normalization function respectively. Compared with JVET-X0097, an additional convolutional layer is added. The channel attention extraction module is circled by a dotted line in Fig.2.

Architecture of the proposed CNN model

Architecture of the designed ARB

Training and Inference

The training stage use BVI_DVC 4K dataset [2]. The data is downsampled by RPRx2 and then compressed by VTM.

Other details of the training and inference stage are as shown in Table 1 and Table 2, respectively.

Network Information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce GTX 2080 Ti |
|  | Framework: | PyTorch v1.3.0 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | ~2000 |
|  | Batch size: | 4Kx16 |
|  | Loss function: | L1 |
|  | Training time: | ~200h |
|  | Training data information: | BVI_DVC 4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations | ~200,000 |
|  | Patch size | 128x128 |
|  | Learning rate: | 1.00E-04 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | randomly cropped, and rotation |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Network Information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | No GPU, Intel(R) Xeon(R) Gold 6242R CPU @ 3.10GHz |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 2036097/model |
|  | Total Number of Parameters (All Models) | 4072194 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 15.53418732 |
|  | Multiply Accumulate (kMAC/pixel) | 508 |
| Optional |  |  |
|  | Total Conv. Layers | 84 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 4Kx1 |
|  | Patch size | 512x512 |
|  | Changes to network configuration or weights required to generate rate points | (e.g. ) |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | padding |
|  | Other information: |  |

Experimental results

The proposed CNN-based upsampling filter is tested on top of VTM-11.0-NNVC[3] with the common test conditions[4].

For the anchor and the test, the QP sets are both {22, 27, 32, 37, 42}.

Table 3 and Table 4 shows the test results for AI and RA configurations, respectively. Under AI configuration, the proposed method can bring about 9.16% BD-rate gain on average for luma. Under RA configuration, the proposed method can bring about 0.52% BD-rate loss on average for luma.

Results of the proposed method under AI configuration

|  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -10.82% | 1.32% | 7.95% | 148% | 33% |
|  | FoodMarket4 | -5.94% | 5.32% | 8.23% | 137% | 33% |
|  | Campfire | -12.66% | 100.00% | 32.38% | 78% | 30% |
| Class A2 | CatRobot | -8.69% | 21.72% | 28.89% | 92% | 34% |
|  | DaylightRoad2 | -2.50% | -0.07% | 2.55% | 87% | 31% |
|  | ParkRunning3 | -14.34% | 125.44% | 48.30% | 64% | 33% |
| Overall |  | -9.16% | 42.29% | 21.38% | 101% | 32% |

Results of the proposed method under RA configuration

|  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -4.51% | -1.99% | 7.67% | 81% | 31% |
|  | FoodMarket4 | -3.11% | 3.27% | 4.44% | 91% | 30% |
|  | Campfire | -4.56% | 97.41% | 27.98% | 55% | 33% |
| Class A2 | CatRobot | 6.31% | 36.81% | 52.42% | 74% | 30% |
|  | DaylightRoad2 | 16.37% | 6.39% | 12.32% | 73% | 31% |
|  | ParkRunning3 | -7.38% | 265.83% | 87.07% | 52% | 29% |
| Overall |  | 0.52% | 67.95% | 31.98% | 71% | 31% |

Conclusions

This contribution add more side information as input to the network and increase the number of convolutional layers in the residual block. Compared with JVET-X0097, this contribution brings about 2.8% gain under AI configuration.

References

- AHG11: A CNN-based Super Resolution Method, JVET-X0097, S. Peng, C. Fang, X. Zhang, D. Jiang, J. Lin.

- https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn , Di Ma, Fan Zhang, and David R. Bull.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).