| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0091-v1 |
| --- | --- |

| Title: | Non-EE2: MVP refinement for regular AMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang No.168 Weiwo Rd., Chang'an, Dongguan, Guangdong | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com jinrong.zhang@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a method of MVP refinement for regular AMVP mode, in which before using template-based searching to refine the MVP, two changes have been made. First, two out of five candidates are chosen from the candidate list by comparing the template matching costs. Second for each of the two selected candidates, template-based searching process is operated with a search pattern indicated by MMVD to derive an initial refined MVP with minimum template matching cost. Then the template-based searching process in ECM is conducted to the initial refined MVP. On top of ECM-3.1, the simulation results of the proposed method are reported in Test 1.

To reduce the complexity, two more tests with simplified methods were conducted and the performance are shown as Test 2 and Test 3 as below:

Under CTC

Test 1: RA

Test 2: RA

Test 3: RA

Non-CTC (without using per-class configuration provided in the cfg/per-class folder for random access condition)

Test 2: RA: -0.12%, -0.22%, -0.26%, 10x% 10x%

Introduction

In ECM regular AMVP mode, an MVP candidate is determined based on template matching error to select the one which reaches the minimum difference between the current block template and the reference block template, and then TM is performed only for this particular MVP candidate for MV refinement.

Proposed method

After the AMVP list construction, an MVP is derived as the following steps:

- Select two candidates with two smallest TM costs from AMVP list.

- Extend the two candidates by a search pattern where the search positions are indicated by MMVD and derive TM costs for these extended MVPs. The search pattern is illustrated in Fig. 1, where 8 directions and 9 positions along each direction are defined in Table 1.

Table 1 - The offset distance

| Distance (1/4-pel) | 1 | 2 | 4 | 8 | 12 | 16 | 24 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1. Search pattern

- Operate TM for the MVP with the minimum TM cost in the same manner in ECM.

To reduce the complexity, the simplification is proposed as follows and the performance is shown in Test 2:

- Only if the motion vector resolution of current CU is 1/4-pel or 1-pel, the proposed method is applied.

- In step 2, set each of the two selected candidates (candMv0 and candMv1) as starting point respectively, and operate template-based searching at the search positions indicated by Distance = {1, 2, 4, 8} along 8 directions to derive the corresponding TM costs for each search position. If thus far the minimum TM cost for the searched points is larger than either of the TM costs of candMv0 or candMv1, the template-based searching process for the remaining Distance = {12, 16, 24, 32, 64} along 8 directions is terminated. Otherwise, if the searched point with the minimum TM cost is starting with candMv0 (or candMv1) as the center, then only the remaining search positions around candMv0 (or candMv1) are searched.

Results

The proposed method is implemented on top of ECM-3.1 [1]. The simulation results are tested under ECM-3.1 test condition [2] with random access configurations.

Except class C and D, timings of the remaining classes are not accurate.

Test 1

Table 2 - Simulation results of the proposed method.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.12% | -0.18% | -0.20% | 111% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.07% | 0.06% | -0.17% | 110% | 104% |
| Class F | -0.09% | -0.21% | -0.15% | 105% | 100% |

Test 2 (Simplified method 1)

Table 3 - Simulation results of the proposed method.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.23% | -0.06% |  |  |
| Class C | -0.11% | -0.11% | -0.16% | 108% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.09% | -0.08% | 108% | 103% |
| Class F | -0.06% | -0.01% | -0.15% | 103% | 100% |

Test 3 (If NoBackwardPredFlag equals to 1, Test 2 is applied)

Table 4 - Simulation results of the proposed method.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.14% | -0.07% |  |  |
| Class C | -0.10% | -0.12% | -0.13% | 107% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.20% | -0.06% | 107% | 103% |
| Class F | -0.07% | -0.09% | -0.16% | 103% | 100% |

The below simulation results are tested without using per-class configuration provided in the cfg/pre-class folder for random access condition. The test duration is 10 seconds for low delay condition.

Test 2 (Simplified method 1)

Table 5 - Simulation results of the proposed method.

|  | RA |  |  |  |  | LD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.30% | -0.40% |  |  |  |  |  |  |  |
| Class A2 | -0.13% | -0.19% | -0.24% |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.21% | -0.24% |  |  |  |  |  |  |  |
| Class C | -0.11% | -0.21% | -0.19% | 107% | 102% | -0.20% | -0.19% | -0.42% | 124% | 103% |
| Class E |  |  |  |  |  | -0.27% | -0.07% | -0.15% |  |  |
| Overall | -0.12% | -0.22% | -0.26% |  |  |  |  |  |  |  |
| Class D | -0.09% | -0.04% | -0.01% | 107% | 101% | -0.12% | -0.16% | 0.16% | 124% | 103% |
| Class F | -0.06% | -0.01% | -0.15% | 103% | 100% |  |  |  |  |  |

The test2 result of ParkRuning3 for QP 22 is copied from anchor.

Conclusion

In this contribution, an MVP refinement scheme for regular AMVP mode is proposed. The test results show that it can further improve the coding efficiency of ECM-3.1. It’s suggested to study this technique in next EE2 meeitng.

References

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-3.1

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).