| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0105-v5 |
| --- | --- |

| Title: | AHG10: An improved VVC rate control scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guangjie Ren Jianghao Jia Jiwei Wang Zhenzhong Chen Wuhan University Zizheng Liu Tencent | Email: | rgj@whu.edu.cn jiajh2021@whu.edu.cn wangjiwei@whu.edu.cn zzchen@whu.edu.cn zizhengliu@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution presents some improvements based on the current R-lambda rate control algorithm. With the proposed algorithm, when using the anchor bit rate of VTM 14.0 as the target, there are -0.65%/-0.65%/-0.51% and -1.38%/-1.59%/-1.63% for Y/U/V coding efficiency improvements in low delay B and random access configuration. The proposed method includes three parts. One is the CTU-level bit allocation for skip and non-skip CTU. Another modification is to set smooth window size based on the GOP size and IntraPeriod. Besides, the quality dependency factor (QDF) based rate control was extended to low frame rate as well in this contribution.

Introduction

The R-lambda rate control algorithm for HEVC, JCT-VC K0103 [1] and M0036 [2], were also integrated into the current VTM software with some improvements JVET K0390 [3], JVET-M0600[4], JVET-T0062[5]. In this contribution, some improvements were further implemented based on VTM 14.0. The proposed method includes three parts. One is the CTU-level bit allocation for skip and non-skip CTU. Another modification is to set smooth window size based on the GOP size and IntraPeriod. Besides, the quality dependency factor (QDF) following JVET-M0600[4] was extended to low frame rate as well in this contribution.

Solution

The modifications in this contribution are listed.

- CTU-level bit allocation

In the current CTU-level bit allocation, the initial bit weight for each CTU is determined by frame-level lambda and content related parameters alpha and beta:

Ωi=λframeαi1βi#1

λframe=α*bppβ#2

The default R-D model cannot accurately describe the characteristics of the skip block. The bit allocation of skip CTU and non-skip CTU are carried out separately to solve this problem. First, the bits of skip CTU is calculated by:

bitsskip=scale*bpp×Pskip#3

where scale is empirical set as 0.4, Pskip is the number of pixels of skip CTU. And the bit allocation problem of non-skip CTUs is to solve the constrained optimization problem as:

mini=1Nnon-skipDi   s.t. i=1iNnon-skipRi≤Tframe-Tskip#4

This problem can be converted to an unconstrained optimization problem by introducing a Lagrange multiplier:

min J=i=1Nnon-skipDi+λnon-skip(i=1iNnon-skipRi-(Tframe-Tskip))  #4

where λnon-skip is solved by the Newton method to achieve optimal bit allocation for non-skip CTUs:

bitsRi=Tframe-Tskip*Ωiij=1Nnon-skipΩji

Ωi=λnon-skipαi1βi#5

The max iteration of the Newton method is set as 20, and the early terminal condition is:
|λnon-skipi+1-λnon-skipi|<0.001#6

- Change the setting of Smooth Window

In the current GOP level bit allocation, SW is the size of a smooth window used to make the bit rate change smoother. SW will influence the GOP-level and frame-level target bits:

TAvgPic=RPicAvg+RPicAvg⋅Ncoded-RcodedSW#7

TGOP=TAvgPic⋅NGOP#8

Due to the influence of the Intra frame bits, TAvgPic is usually calculated as a negative value, causing GOP-level target bits to be clipped to 200. When the size of SW is set as 40 in HM, the GOP Size of LD and RA are 4 and 8, respectively. In the current software, the GOP size of LD and RA have been extended to 32 and 8. The fixed SW is not suitable for current configuration and future extension.

To solve this problem, the GOP size and IntraPeriod (IP) are taken into consideration when setting SW:

SW=α*GOPmax32, IP+β#9

where α is set as 20, and β is set as 60.

- Extension of the QDF to low frame rate

In current software, the judgment condition for using the QDF based rate control is:

if (encRCSeq->getAdaptiveBits() > 0 && encRCSeq->getLastLambda() > 0.1)

m_lastLambda = encRCSeq->getLastLambda()

where “m_lastLambda” will never be updated under the configuration of “GOP Size=IntraPeriod” for the low frame rate sequences.

This condition was changed to extend the QDF factor to low frame rate sequences.

m_encRCSeq->getAdaptiveBits() > 0 && m_listRCPictures.size() >= m_encRCSeq->getGOPSize()

Results

The experiments were based on VTM 14.0 and were carried out on HPC cluster in Linux platform. Compared with the VTM 14.0 rate control method, the proposed full strategy can achieve -0.65%/-0.65%/-0.51%(Y/U/V) and -1.38%/-1.59%/-1.63%(Y/U/V) coding efficiency improvement in low delay B and random access configuration as shown in Table 1. It should be noted that there are some fluctuations in the encoding time under the RA configuration. This is because the default rate control test and the proposed strategy were performed in the different states of the server. As is known to all, the complexity of rate control takes a small part in entire encoding process. And the encoding time in LDB configuration is negligible. The following tests are conducted according to the common test condition [6]:

Test 1: Proposed full strategy vs. RC in VTM 14.0

Test 2: Proposed CTU-level bit allocation vs. RC in VTM 14.0

Test 3: Proposed SW setting vs. RC in VTM 14.0

Test 4: Proposed CTU-level bit allocation & SW setting vs. RC in VTM 14.0

Test 5: Proposed extension of the QDF vs. RC in VTM 14.0

Test 6: Proposed SW setting & extension of the QDF vs. RC in VTM 14.0

Table 1 Proposed full strategy vs. RC in VTM 14.0

|  | Over VTM 14.0 with Rate Control |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  | Random Access Main10 (GOP size =32) |  |  |  |  |  |  |  | Random Access Main10 (GOP size =16) |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | Y |  | U |  | V |  | EncT |  | Y |  | U |  | V |  | EncT |  |  |  |
| Class A1 |  |  |  |  |  |  |  |  |  | -3.81% |  | -2.86% |  | -3.89% |  | 143% |  |  |  |  |  |  |  |  |  |  |  |
| Class A1 |  |  |  |  |  |  |  |  |  | -1.07% |  | -2.52% |  | -2.19% |  | 139% |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.29% |  | -0.56% |  | -0.45% |  | 101% |  | -1.02% |  | -1.45% |  | -1.22% |  | 157% |  | -0.52% |  | -1.36% |  | -1.25% |  | 94% |  |  |
| Class C |  | -0.25% |  | -0.35% |  | -0.43% |  | 101% |  | -0.23% |  | -0.13% |  | -0.03% |  | 111% |  | 0.16% |  | -0.25% |  | -0.02% |  | 88% |  |  |
| Class D |  | -1.78% |  | -1.21% |  | -0.73% |  | 102% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.65% | -0.65% | -0.51% | 101% | -1.38% | -1.59% | -1.63% | 137% |  |  |  |  |
| Class E |  | -0.38% |  | -0.03% |  | -0.08% |  | 100% |  | -0.29% |  | -0.05% |  | -0.33% |  | 113% |  | -0.17% |  | 0.34% |  | 0.46% |  | 88% |  |  |
| Class F |  | -2.44% |  | -1.33% |  | -2.33% |  | 101% |  | 0.79% |  | -1.74% |  | -2.09% |  | 125% |  | -0.57% |  | -0.23% |  | -0.02% |  | 74% |  |  |

In Table 2 and Table 3, the performance of proposed CTU-level bit allocation and SW setting are shown.

Table 2 Proposed CTU-level bit allocation vs. RC in VTM 14.0

|  | Over VTM 14.0 with Rate Control |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  | Random Access Main10 (GOP size =32) |  |  |  | Random Access Main10 (GOP size =16) |  |  |  |  |
|  | Y | U | V | EncT | Y | U | V | EncT | Y | U | V | EncT |  |
| Class A1 |  |  |  |  | 0.20% | 0.70% | -0.83% | 143% |  |  |  |  |  |
| Class A1 |  |  |  |  | 0.03% | -1.63% | -1.37% | 144% |  |  |  |  |
| Class B | -0.08% | -0.43% | -0.38% | 100% | -0.13% | -0.88% | -0.67% | 167% | -0.15% | -1.28% | -1.17% | 88% |
| Class C | -0.03% | -0.46% | -0.34% | 100% | -0.02% | -0.61% | -0.47% | 129% | 0.11% | -0.28% | -0.20% | 89% |
| Class D | -0.39% | -0.95% | -0.18% | 101% |  |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.57% | -0.31% | 101% | 0.00% | -0.64% | -0.79% | 147% |  |  |  |  |
| Class E | -0.24% | 0.01% | -0.24% | 100% | -0.07% | -0.33% | -0.73% | 129% | 0.00% | 0.03% | 0.40% | 89% |
| Class F | -2.40% | -2.34% | -2.79% | 100% | -0.10% | -0.21% | -0.22% | 142% | 0.40% | -0.03% | 0.13% | 74% |

In Table 2 and Table 3, the performance of proposed CTU-level bit allocation and SW setting are shown.

Table 2 Proposed CTU-level bit allocation vs. RC in VTM 14.0

Table 3 Proposed SW Setting vs. RC in VTM 14.0

|  | Over VTM 14.0 with Rate Control |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  | Random Access Main10 (GOP size =32) |  |  |  | Random Access Main10 (GOP size =16) |  |  |  |  |
|  | Y | U | V | EncT | Y | U | V | EncT | Y | U | V | EncT |  |
| Class A1 |  |  |  |  | -1.53% | -1.18% | -1.23% | 124% |  |  |  |  |  |
| Class A1 |  |  |  |  | -0.90% | -0.89% | -0.66% | 123% |  |  |  |  |
| Class B | -0.21% | -0.03% | 0.16% | 101% | -0.99% | -0.44% | -0.40% | 112% | -0.48% | -0.64% | -0.58% | 92% |
| Class C | -0.13% | -0.08% | -0.09% | 101% | -0.49% | -0.86% | -0.62% | 98% | 0.00% | 0.21% | 0.24% | 89% |
| Class D | -1.49% | 0.12% | -0.54% | 100% |  |  |  |  |  |  |  |  |
| Overall | -0.50% | -0.01% | -0.10% | 101% | -0.95% | -0.79% | -0.68% | 112% |  |  |  |  |
| Class E | -0.37% | -0.28% | -0.52% | 100% | -0.64% | -0.40% | -0.44% | 103% | -0.24% | 0.36% | 0.15% | 89% |
| Class F | -4.14% | -3.01% | -3.49% | 100% | 3.65% | 1.26% | 1.32% | 106% | -1.97% | -1.61% | -1.62% | 87% |

Table 4 shows the results of proposed CTU-level bit allocation & SW setting.

Table 4 Proposed CTU-level bit allocation & SW Setting vs. RC in VTM 14.0

|  | Over VTM 14.0 with Rate Control |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  | Random Access Main10 (GOP size =32) |  |  |  | Random Access Main10 (GOP size =16) |  |  |  |  |
|  | Y | U | V | EncT | Y | U | V | EncT | Y | U | V | EncT |  |
| Class A1 |  |  |  |  | -1.35% | -0.47% | -2.02% | 148% |  |  |  |  |  |
| Class A1 |  |  |  |  | -1.07% | -2.52% | -2.19% | 139% |  |  |  |  |
| Class B | -0.29% | -0.56% | -0.45% | 101% | -1.02% | -1.45% | -1.22% | 157% | -0.52% | -1.36% | -1.25% | 94% |
| Class C | -0.25% | -0.35% | -0.43% | 101% | -0.47% | -1.12% | -0.90% | 127% | 0.16% | -0.25% | -0.02% | 88% |
| Class D | -1.78% | -1.21% | -0.73% | 102% |  |  |  |  |  |  |  |  |
| Overall | -0.65% | -0.65% | -0.51% | 101% | -0.95% | -1.38% | -1.49% | 143% |  |  |  |  |
| Class E | -0.38% | -0.03% | -0.08% | 100% | -0.60% | -1.06% | -1.09% | 129% | -0.17% | 0.34% | 0.46% | 88% |
| Class F | -2.44% | -1.33% | 2.33% | 101% | 5.11% | 2.22% | 2.31% | 153% | -1.73% | -1.37% | -1.19% | 76% |

Table 4 shows the results of proposed CTU-level bit allocation & SW settin

Table 5 shows the results of extension of the QDF to low frame rate. The anchor is fixed QP.

Table 5 Proposed extension of QDF vs. RC in VTM 14.0

| Sequence | GOP Size | Method | Y | U | V |
| --- | --- | --- | --- | --- | --- |
| Campfire | 32 | VTM 14.0 | 22.22% | 21.20% | 16.50% |
|  |  | QDF | 11.95% | 11.26% | 9.15% |
| RaceHorsesC | 32 | VTM 14.0 | 2.60% | 1.02% | 0.85% |
|  |  | QDF | 2.52% | 3.18% | 3.54% |
| RaceHorsesD | 32 | VTM 14.0 | 4.00% | 2.59% | 4.59% |
|  |  | QDF | 4.29% | 6.01% | 7.90% |
| SlideEditing | 32 | VTM 14.0 | 19.21% | 11.77% | 10.67% |
|  |  | QDF | 18.42% | 12.08% | 11.10% |
| SlideShow | 32 | VTM 14.0 | 33.38% | 19.00% | 23.06% |
|  |  | QDF | 29.12% | 12.91% | 14.84% |
|  | 16 | VTM 14.0 | 57.73% | 35.13% | 35.71% |
|  |  | QDF | 66.79% | 40.87% | 40.50% |

Table 6 shows the results of proposed SW setting & extension of the QDF.

Table 6 Proposed SW Setting & extension of the QDF vs. RC in VTM 14.0

|  | Over VTM 14.0 with Rate Control |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  | Random Access Main10 (GOP size =32) |  |  |  | Random Access Main10 (GOP size =16) |  |  |  |  |
|  | Y | U | V | EncT | Y | U | V | EncT | Y | U | V | EncT |  |
| Class A1 |  |  |  |  | -4.03% | -3.72% | -2.91% | 128% |  |  |  |  |  |
| Class A1 |  |  |  |  | -0.90% | -0.89% | -0.66% | 123% |  |  |  |  |
| Class B | -0.21% | -0.03% | 0.16% | 101% | -0.99% | -0.44% | -0.40% | 112% | -0.48% | -0.64% | -0.58% | 92% |
| Class C | -0.13% | -0.08% | -0.09% | 101% | -0.24% | -0.13% | 0.19% | 111% | 0.00% | 0.21% | 0.24% | 89% |
| Class D | -1.49% | 0.12% | -0.54% | 100% |  |  |  |  |  |  |  |  |
| Overall | -0.50% | -0.01% | -0.10% | 101% | -1.38% | -1.10% | -0.80% | 117% |  |  |  |  |
| Class E | -0.37% | -0.28% | -0.52% | 100% | -0.38% | 0.51% | 0.32% | 118% | -0.24% | 0.36% | 0.15% | 89% |
| Class F | -4.14% | -3.01% | -3.49% | 100% | 1.24% | -0.92% | -1.10% | 136% | -1.59% | -0.90% | -0.70% | 88% |

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC K0103, Oct. 2012.

- B. Li, H. Li, and L. Li, “Adaptive bit allocation for R-lambda model rate control in HM,” JCT-VC M0036, Apr. 2013.

- Y. Li, Z. Chen, X. Li and S. Liu, “Rate Control for VVC,” JVET-K0390, Jul. 2018.

- Z. Liu, Z. Chen, Y. Li, Y. Wu, S. Liu, “AHG10: Quality dependency factor-based rate control for VVC”, JVET-M0600, Jan. 2019.

- F. Liu, Z. Liu, Z. Chen, Y. Li, Z. Chen, “AHG10: Extension of rate control to support random access configuration with GOP size of 32”, October. 2020.

- JVET-T2010, VTM common test conditions and software reference configurations for SDR video.