| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0132-v2 |
| --- | --- |

| Title: | EE2-3.7: On the increased number of TM merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports EE2 test 3.7 results for the number of TM merge candidates increased to 7 from 4. Compared to ECM-3.1 anchor, simulation results are summarized as follows:

RA: -0.09% (Y), -0.11% (U), -0.13% (V), 102% (EncT), 101% (DecT)

LB: -0.12% (Y), -0.24% (U), -0.21% (V), 103% (EncT), 103% (DecT)

# Proposal

In EE2-3.7, the number of TM merge candidates is increased from 4 to 7 by modifying the parameter TM_MRG_MAX_NUM_CANDS in ECM-3.1 software as follows:

static const int    TM_MRG_MAX_NUM_CANDS =           7;

# Simulation results

The proposed change was tested using the common test conditions as defined in JVET-X2017. Table 1 shows the overall BD-rate performance.

Table 1. Performance of the number of TM merge candidates set as 7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.26% | -0.19% | 103% | 103% |
| Class A2 | -0.12% | -0.06% | -0.09% | 101% | 100% |
| Class B | -0.09% | -0.16% | -0.09% | 102% | 100% |
| Class C | -0.08% | 0.01% | -0.16% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.11% | -0.13% | 102% | 101% |
| Class D | -0.03% | 0.04% | 0.19% | 102% | 100% |
| Class F | -0.04% | -0.12% | -0.06% | 101% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.30% | -0.26% | 103% | 103% |
| Class C | -0.16% | -0.15% | 0.06% | 102% | 103% |
| Class E | -0.05% | -0.25% | -0.50% | 103% | 102% |
| Overall | -0.12% | -0.24% | -0.21% | 103% | 103% |
| Class D | -0.01% | 0.14% | -0.26% | 102% | 99% |
| Class F | 0.09% | 0.40% | 0.28% | 98% | 98% |

# Patent rights declaration(s)

Qualcomm Inc. does not have any current or pending patent rights relating to the technology described in this contribution.