| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0140-v1 |
| --- | --- |

| Title: | EE2-related: On the LCU boundary processing by intra-prediction tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Damian Ruiz Coll | Tel: Email: | {afilippov, vrufitskiy} @ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents a modification of the number of reference lines available for intra prediction of a block on the above LCU boundary. Three methods are proposed, enabling 1,3 and 5 lines, respectively. The first methods comprises two subtests. In the first one, the derivation of context for TIMD and DIMD is not modified. In the second subtest a neighbor above block is not available if it is outside the current LCU. The proposed modification of test 1.1 provides the following overall results: 0.49% (Y) / 0.35% (U) / 0.44% (V) and 0.31% (Y) / 0.29% (U) / 0.29% (V) in AI and RA configurations, respectively. The proposed modification of test 1.2 provides the following overall results: 0.48% (Y) / 0.36% (U) / 0.44% (V) and 0.30% (Y) / 0.22% (U) / 0.28% (V) in AI and RA configurations, respectively. The proposed modification of test 2 provides the following overall results: -0.05% (Y) / -0.12% (U) / -0.08% (V) and -0.01% (Y) / -0.01% (U) / 0.04% (V) in AI and RA configurations, respectively. The proposed modification of test 3 provides the following overall results: -0.06% (Y) /-0.08% (U) /-0.06% (V) and -0.01% (Y) / 0.04% (U) / 0.07% (V) in AI and RA configurations, respectively.

# Introduction

In the design of Versatile Video Coding, a constraint on the number of reference lines available above the LCU was introduced. MRL is disabled for a block that is located on the above LCU boundary.

In ECM-3.1, MRL is not allowed on the LCU boundary. However, decoder side intra mode derivation is performed using reconstructed reference samples that are far from the LCU boundary. Specifically, DIMD may utilize 3 reference lines and TIMD may utilize 5 reference lines above an LCU boundary.

# Proposed method

Several alternative methods are being proposed.

The 1st method consists in restricting DIMD and TIMD to use just 1 reference line for all the intra prediction tools above an LCU boundary. On the LCU boundary above template is set unavailable when deriving intra prediction mode by DIMD or TIMD, since a single reference line is insufficient for HoG used in DIMD or for template matching operations in TIMD. Two subtests were performed. In subtest 1.1, the derivation of context for TIMD and DIMD is not modified. In subtest 1.2, a neighbor above block is considered unavailable if it is outside the current LCU.

The 2nd method consists in enabling only 3 reference lines above an LCU boundary. In this method, when a block is on above LCU boundary (and not on the slice/tile boundary), if TIMD is enabled, TIMD above template side is set equal to 2 without consideration of a block height. Besides, in CCLM the same 2×3  down-sampling filter is applied on the above block boundary without checking whether a block is on the LCU boundary or not. Besides, MRLP is not disabled for a block on an LCU boundary.

The 3rd method utilized 5 reference lines above an LCU boundary. In CCLM the same 2×3 down-sampling filter is applied on the above block boundary without checking whether a block is on the LCU boundary or not. Besides, MRLP is not disabled for a block on an LCU boundary.

# Results

The results for the first method obtained for AI and RA configurations in accordance with [1] are presented in Tables 1-4 respectively. In Test 1.1 derivation of contexts for TIMD and DIMD is not modified. In test 1.2 context derivations is performed with consideration of whether a block on an LCU boundary or not (as described above).

Table 1. Performance of the proposed method 1.1 (AI configuration)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.27% | 0.08% | 0.27% | 100% | 96% |
| Class A2 | 0.38% | 0.37% | 0.43% | 97% | 89% |
| Class B | 0.51% | 0.38% | 0.42% | 99% | 90% |
| Class C | 0.39% | 0.32% | 0.37% | 100% | 92% |
| Class E | 0.90% | 0.59% | 0.74% | 100% | 90% |
| Overall | 0.49% | 0.35% | 0.44% | 99% | 91% |
| Class D | 0.22% | 0.27% | 0.09% | 100% | 95% |
| Class F | 0.27% | 0.35% | 0.25% | 100% | 97% |
| Class TGM | 0.12% | 0.14% | 0.11% | 100% | 98% |

Table 2. Performance of the proposed method 1.1 (RA configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | 0.22% | 0.01% | 100% | 99% |
| Class A2 | 0.26% | 0.26% | 0.19% | 100% | 100% |
| Class B | 0.36% | 0.36% | 0.34% | 100% | 99% |
| Class C | 0.31% | 0.29% | 0.53% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.31% | 0.29% | 0.29% | 100% | 99% |
| Class D | 0.13% | 0.32% | 0.16% | 100% | 100% |
| Class F | 0.19% | 0.17% | 0.18% | 100% | 100% |
| Class TGM | 0.08% | 0.00% | 0.09% | 100% | 100% |

Table 3. Performance of the proposed method 1.2 (AI configuration)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.28% | 0.10% | 0.21% | 100% | 96% |
| Class A2 | 0.38% | 0.31% | 0.41% | 100% | 93% |
| Class B | 0.51% | 0.39% | 0.45% | 100% | 91% |
| Class C | 0.39% | 0.35% | 0.35% | 100% | 89% |
| Class E | 0.89% | 0.63% | 0.77% | 101% | 89% |
| Overall | 0.48% | 0.36% | 0.44% | 100% | 91% |
| Class D | 0.22% | 0.18% | 0.08% | 102% | 94% |
| Class F | 0.28% | 0.39% | 0.41% | 101% | 96% |
| Class TGM | 0.11% | 0.16% | 0.11% | 101% | 99% |

Table 4. Performance of the proposed method 1.2 (RA configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.02% | -0.01% | 98% | 98% |
| Class A2 | 0.29% | 0.45% | 0.31% | 98% | 99% |
| Class B | 0.36% | 0.22% | 0.36% | 101% | 99% |
| Class C | 0.28% | 0.18% | 0.37% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.30% | 0.22% | 0.28% | 100% | 99% |
| Class D | 0.17% | 0.05% | 0.21% | 102% | 99% |
| Class F | 0.21% | 0.21% | 0.15% | 101% | 100% |
| Class TGM | 0.08% | 0.03% | 0.10% | 101% | 100% |

The results for method 2 obtained for AI and RA configurations in accordance with [1] are given in Table 5 and Table 6.

Table 5. Performance of the proposed method 2 (AI configuration)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | 0.01% | 101% | 100% |
| Class A2 | -0.03% | -0.09% | 0.02% | 100% | 100% |
| Class B | -0.06% | -0.20% | -0.19% | 100% | 99% |
| Class C | -0.08% | -0.06% | -0.15% | 99% | 99% |
| Class E | -0.05% | -0.19% | -0.01% | 101% | 100% |
| Overall | -0.05% | -0.12% | -0.08% | 100% | 99% |
| Class D | -0.03% | 0.07% | -0.20% | 101% | 101% |
| Class F | -0.12% | -0.16% | -0.05% | 100% | 99% |
| Class TGM | -0.10% | -0.10% | -0.11% | 100% | 100% |

Table 6. Performance of the proposed method 2 (RA configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.25% | 100% | 100% |
| Class A2 | 0.00% | 0.09% | 0.04% | 100% | 100% |
| Class B | -0.04% | -0.07% | 0.08% | 100% | 99% |
| Class C | 0.01% | 0.04% | 0.22% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.04% | 100% | 100% |
| Class D | -0.01% | 0.35% | -0.13% | 101% | 100% |
| Class F | 0.03% | -0.15% | -0.32% | 102% | 99% |
| Class TGM | 0.00% | -0.07% | 0.04% | 101% | 99% |

The results for method 3 obtained for AI and RA configurations in accordance with [1] are given in Table 7 and Table 8.

Table 7. Performance of the proposed method 3 (AI configuration)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.06% | 101% | 100% |
| Class A2 | -0.03% | -0.02% | -0.01% | 100% | 100% |
| Class B | -0.07% | -0.12% | -0.17% | 101% | 100% |
| Class C | -0.09% | -0.08% | -0.10% | 100% | 100% |
| Class E | -0.08% | -0.17% | 0.00% | 101% | 99% |
| Overall | -0.06% | -0.08% | -0.06% | 100% | 100% |
| Class D | -0.04% | 0.05% | -0.19% | 100% | 101% |
| Class F | -0.13% | 0.00% | -0.12% | 100% | 101% |
| Class TGM | -0.10% | -0.12% | -0.13% | 100% | 99% |

Table 8. Performance of the proposed method 3 (RA configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.14% | 99% | 99% |
| Class A2 | -0.02% | 0.00% | 0.06% | 99% | 100% |
| Class B | -0.03% | 0.05% | 0.11% | 100% | 99% |
| Class C | 0.00% | 0.11% | 0.17% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.07% | 100% | 100% |
| Class D | -0.07% | 0.34% | -0.13% | 101% | 100% |
| Class F | 0.01% | -0.22% | 0.05% | 101% | 99% |
| Class TGM | -0.01% | -0.14% | -0.03% | 102% | 100% |

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-X2017, October 2021.

# Conclusions

This contribution proposes to align the LCU boundary reference lines constraint for different intra prediction tools. We recommend to adopt one of the proposed methods into the ECM reference software.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).