| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0142-v1 |
| --- | --- |

| Title: | EE2-4.4: adaptive intra MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray Vadim Seregin Marta Karczewicz | Tel: Email: | bray@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 tests: 4.4.A, 4.4.B, 4.4.C and 4.4.D. Simulation results on top of ECM-3.1 are:

Test-4.4.A:

AI (Y/U/V):   -0.11%/-0.14%/-0.11%, EncT 101%, DecT 99%

RA (Y/U/V):   -0.01%/-0.01%/0.05%, EncT 97%, DecT 100%

LDB (Y/U/V):  -0.18%/0.28%/-0.38%, EncT 101%, DecT 100%

Test-4.4.B:

AI (Y/U/V):   0.57%/-0.02%/0.07%, EncT 87%, DecT 100%

RA (Y/U/V):   0.26%/0.19%/0.25%, EncT 95%, DecT 100%

LDB (Y/U/V):  -0.xx%/0.xx%/-0.xx%, EncT xx%, DecT xx%

Test-4.4.C:

AI (Y/U/V):   -0.11%/-0.13%/-0.11%, EncT 102%, DecT 100%

RA (Y/U/V):   -0.01%/-0.01%/0.06%, EncT 97%, DecT 100%

LDB (Y/U/V):  -0.xx%/0.xx%/-0.xx%, EncT xx%, DecT xx%

Test-4.4.D:

AI (Y/U/V):   -0.08%/0.01%/0.07%, EncT 109%, DecT 100%

RA (Y/U/V):   -0.01%/0.01%/0.06%, EncT 102%, DecT 100%

LDB (Y/U/V):  -0.xx%/0.xx%/-0.xx%, EncT xx%, DecT xx%

# Introduction

In this EE tests, performance of adaptive intra MTS [1] is investigated. Adaptive intra MTS is based on the idea that the number of MTS candidates (between 1, 4 and 6 MTS candidates) is switched based on sum of absolute value of transform coefficients. Four tests have been carried out as described below:

- Test 4.4.A: adaptive intra MTS.

- Test 4.4.B: only encoder modification.

- Test 4.4.C: adaptive intra MTS with parameter signaled at SPS level.

- Test 4.4.D: fixed 6 MTS candidates (without adaptivity).

# Results

The implementation for all tests is done on top of ECM-3.1, tests are performed following test conditions specified in JVET-V2017 [2]. Test results for AI, RA and LDB configurations, with respect to ECM-3.1, are summarized in the below tables.

## Test 4.4.A:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.10% | 0.01% | 99% | 98% |
| Class A2 | -0.15% | -0.13% | -0.10% | 100% | 99% |
| Class B | -0.13% | -0.15% | -0.15% | 101% | 100% |
| Class C | -0.07% | -0.15% | -0.16% | 104% | 99% |
| Class E | -0.12% | -0.14% | -0.14% | 101% | 99% |
| Overall | -0.11% | -0.14% | -0.11% | 101% | 99% |
| Class D | -0.08% | -0.01% | -0.13% | 105% | 100% |
| Class F | 0.03% | 0.14% | -0.10% | 102% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.08% | -0.08% | 98% | 100% |
| Class A2 | 0.00% | 0.02% | 0.05% | 97% | 100% |
| Class B | -0.03% | -0.03% | 0.09% | 97% | 100% |
| Class C | 0.01% | -0.08% | 0.11% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.05% | 97% | 100% |
| Class D | -0.02% | 0.38% | -0.19% | 97% | 100% |
| Class F | 0.06% | -0.02% | -0.20% | 98% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.45% | -0.01% | 101% | 100% |
| Class C | -0.23% | -0.16% | -0.50% | 100% | 99% |
| Class E | -0.22% | -0.18% | -0.84% | 100% | 100% |
| Overall | -0.18% | -0.28% | -0.38% | 101% | 100% |
| Class D | -0.01% | 0.11% | -0.30% | 98% | 98% |
| Class F | -0.03% | -0.10% | 0.35% | 102% | 99% |

## Test 4.4.B:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.46% | -0.22% | 0.10% | 88% | 101% |
| Class A2 | 0.57% | 0.00% | 0.10% | 90% | 102% |
| Class B | 0.54% | 0.00% | -0.07% | 86% | 100% |
| Class C | 0.59% | -0.04% | 0.14% | 85% | 99% |
| Class E | 0.68% | 0.13% | 0.13% | 89% | 98% |
| Overall | 0.57% | -0.02% | 0.07% | 87% | 100% |
| Class D | 0.71% | 0.11% | -0.10% | 84% | 98% |
| Class F | 0.36% | 0.11% | 0.12% | 97% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.06% | 0.22% | 95% | 100% |
| Class A2 | 0.19% | 0.21% | 0.07% | 94% | 99% |
| Class B | 0.31% | 0.16% | 0.29% | 95% | 100% |
| Class C | 0.27% | 0.32% | 0.34% | 94% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 0.19% | 0.25% | 95% | 100% |
| Class D | 0.33% | 0.37% | 0.03% | 95% | 100% |
| Class F | 0.31% | 0.04% | -0.15% | 98% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.11% | -0.06% | 0.23% | 101% | 100% |
| Class E | -0.22% | 0.24% | -0.98% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | -0.24% | -0.88% | 99% | 99% |
| Class F | -0.12% | -0.67% | -0.63% | 102% | 100% |

## Test 4.4.C:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.10% | 0.01% | 100% | 100% |
| Class A2 | -0.14% | -0.12% | -0.10% | 103% | 102% |
| Class B | -0.12% | -0.14% | -0.14% | 102% | 100% |
| Class C | -0.06% | -0.14% | -0.15% | 104% | 100% |
| Class E | -0.11% | -0.13% | -0.13% | 101% | 100% |
| Overall | -0.11% | -0.13% | -0.11% | 102% | 100% |
| Class D | -0.05% | 0.02% | -0.11% | 106% | 100% |
| Class F | 0.03% | 0.14% | -0.09% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.08% | -0.08% | 98% | 100% |
| Class A2 | 0.00% | 0.02% | 0.05% | 97% | 100% |
| Class B | -0.03% | -0.03% | 0.09% | 97% | 100% |
| Class C | 0.01% | -0.07% | 0.11% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.06% | 97% | 100% |
| Class D | -0.01% | 0.39% | -0.18% | 98% | 100% |
| Class F | 0.06% | -0.02% | -0.20% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.23% | -0.16% | -0.49% | 102% | 101% |
| Class E | -0.22% | -0.18% | -0.84% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.11% | -0.30% | 100% | 100% |
| Class F | -0.03% | -0.10% | 0.35% | 102% | 99% |

## Test 4.4.D:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | 0.08% | 109% | 100% |
| Class A2 | -0.14% | 0.09% | 0.07% | 109% | 101% |
| Class B | -0.08% | -0.05% | 0.05% | 109% | 100% |
| Class C | -0.05% | 0.11% | 0.10% | 109% | 100% |
| Class E | -0.06% | 0.01% | 0.08% | 108% | 99% |
| Overall | -0.08% | 0.01% | 0.07% | 109% | 100% |
| Class D | -0.03% | 0.13% | 0.08% | 110% | 100% |
| Class F | 0.07% | 0.13% | 0.27% | 103% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | -0.10% | 102% | 100% |
| Class A2 | -0.03% | 0.01% | 0.05% | 102% | 100% |
| Class B | -0.02% | 0.02% | 0.01% | 101% | 100% |
| Class C | -0.01% | 0.05% | 0.26% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.06% | 102% | 100% |
| Class D | -0.02% | 0.20% | 0.12% | 102% | 100% |
| Class F | 0.05% | -0.02% | -0.21% | 104% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-3.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.12% | -0.14% | -0.01% | 102% | 100% |
| Class E | -0.23% | 0.58% | -0.21% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.10% | -0.51% | 98% | 98% |
| Class F | -0.07% | -0.55% | -1.02% | 103% | 99% |

# Reference

[1] B. Ray, V. Seregin, M. Karczewicz, “Non-EE2: Adaptive intra MTS”, JVET-X0135, Oct. 2021.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).