| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0149-v1 |
| --- | --- |

| Title: | EE2-related: Modifications of the extended MRL candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Esmael Dinan | Tel: Email: | {afilippov, vrufitskiy} @ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents modifications of the extended MRL list. Two methods are proposed. In the first method, when TIMD flag is on, selection of the MRL list is performed for left and top side individually in accordance with block width or block height, respectively. In this method, selection of the MRL list is aligned with the template sizes that are used by TIMD for short and long side lengths. In method 2, MRL lists are defined differently for small and larger blocks. The proposed method 1 for the case of 5 elements in the extended MRL list (EE2-2.1a) provides the following overall results: -0.11% (Y) / -0.11% (U) / -0.12% (V) and -0.04% (Y) / 0.04% (U) / 0.01% (V) in AI and RA configurations, respectively. The proposed method 1 for the case of 4 elements in the extended MRL list provides the following overall results: -0.09% (Y) / -0.09% (U) / -0.05% (V) and -0.03% (Y) / 0.01% (U) / 0.08% (V) in AI and RA configurations, respectively. The proposed method 2 for the case of 4 elements in the extended MRL list (EE2-2.1b) provides the following overall results: -0.08% (Y) / -0.11% (U) / -0.04% (V) and -0.03% (Y) / 0.07% (U) / 0.11% (V) in AI and RA configurations, respectively. A combination of method1 and method 2 for the case of 4 elements in the extended MRL list (EE2-2.1b) provides the following overall results: -0.10% (Y) / -0.06% (U) / -0.06% (V) and -0.04% (Y) / 0.02% (U) / 0.02% (V) in AI and RA configurations, respectively.

# Introduction

In Exploration Experiment 2 Test 2.1 MRL list was extended to keep 5, 4 or 3 reference lines in the extended MRL list. Positions of these lines are fixed and do not depend on PU size or other properties.

# Proposed method

When TIMD is applied to a block to derive a mode, reference lines may not be adjacent to a predicted block. However, when a mode is derived, intra prediction of a block uses reference as specified by an MRL index and a value specified in the MRL list.

In method 1 it is proposed to align the size of TIMD template and the values inside MRL list, so that position of the reference line in some case may coincide with the position of reference line that was used to derive this mode in TIMD. When block is of rectangular shape it may occur that template width on the left side of a block is not equal to the height of a template above the block. It is proposed to select MRL list for above and left side individually. When TIMD flag is on, MRL list for the left side is proposed to be selected as given in Tab1e 1.

Table 1. MRL list for the left side when TIMD flag is on

| Condition | width ≤8 | width >8 |
| --- | --- | --- |
| TIMD left template width (as of ECM-3.1) | 2 | 4 |
| MRL list | {0, 1, 2} | {0, 2, 4} |

When TIMD flag is on, MRL list for the above side is proposed to be selected as given in Tab1e 2.

Table 2. MRL list for the above side when TIMD flag is on

| Condition | height ≤8 | height >8 |
| --- | --- | --- |
| TIMD above template height (as of ECM-3.1) | 2 | 4 |
| MRL list | {0, 1, 2} | {0, 2, 4} |

Method 2 relates to an additional aspect of making extended MRL list dependent on a block size. It is proposed to reduce the number of extended MRL lines from 5 to 4, but to keep line 12 in the list for larger blocks. Specific MRL list selection is performed as it is shown in Table 3.

Table 3. MRL list determination based on a block size.

| Condition | block size <16×16 | Block size ≥16×16 |
| --- | --- | --- |
| MRL list | {0, 1, 3, 5, 7} | {0, 3, 5, 7, 12} |

# Results

The results for the first method obtained for AI and RA configurations in accordance with [1] are presented in Table 4 and Table 5 respectively. This test is based on EE2-2.1a and uses 5 reference lines available in the extended MRL list.

Table 4. Performance of the proposed method 1 (AI configuration), 5 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | -0.11% | 101% | 97% |
| Class A2 | -0.09% | 0.03% | -0.02% | 100% | 98% |
| Class B | -0.15% | -0.19% | -0.17% | 102% | 101% |
| Class C | -0.13% | -0.15% | -0.13% | 101% | 99% |
| Class E | -0.10% | -0.14% | -0.14% | 103% | 103% |
| Overall | -0.11% | -0.11% | -0.12% | 101% | 100% |
| Class D | -0.05% | 0.04% | -0.21% | 101% | 95% |
| Class F | -0.06% | 0.04% | -0.07% | 100% | 100% |
| Class TGM | -0.21% | -0.08% | -0.10% | 101% | 99% |

Table 5. Performance of the proposed method 1 (RA configuration), 5 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.12% | 98% | 102% |
| Class A2 | -0.06% | 0.03% | -0.01% | 97% | 101% |
| Class B | -0.06% | 0.08% | 0.10% | 101% | 100% |
| Class C | -0.03% | 0.06% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.04% | 0.01% | 99% | 101% |
| Class D | -0.05% | 0.10% | -0.16% | 102% | 101% |
| Class F | -0.05% | -0.08% | -0.05% | 101% | 100% |
| Class TGM | -0.12% | -0.07% | -0.03% | 101% | 100% |

Results for method 1 implemented on top of EE2-2.1b that uses 4 reference lines available in the extended MRL list are shown in Table 6 and Table 7.

Table 6. Performance of the proposed method 1 over ECM-3.1 anchor (AI configuration), 4 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | -0.02% | 100% | 97% |
| Class A2 | -0.07% | -0.04% | 0.00% | 99% | 98% |
| Class B | -0.12% | -0.11% | -0.10% | 102% | 102% |
| Class C | -0.11% | -0.05% | -0.06% | 101% | 101% |
| Class E | -0.09% | -0.13% | -0.04% | 103% | 103% |
| Overall | -0.09% | -0.09% | -0.05% | 101% | 100% |
| Class D | -0.03% | 0.02% | -0.12% | 102% | 100% |
| Class F | -0.10% | 0.10% | -0.04% | 101% | 100% |
| Class TGM | -0.19% | -0.08% | -0.12% | 101% | 100% |

Table 7. Performance of the proposed method 1 over ECM-3.1 anchor (RA configuration), 4 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | -0.03% | 98% | 102% |
| Class A2 | -0.05% | 0.08% | 0.02% | 97% | 101% |
| Class B | -0.04% | 0.01% | 0.09% | 97% | 98% |
| Class C | -0.03% | 0.06% | 0.20% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.01% | 0.08% | 98% | 100% |
| Class D | -0.01% | 0.21% | -0.03% | 101% | 100% |
| Class F | 0.02% | 0.16% | -0.17% | 101% | 100% |
| Class TGM | -0.12% | -0.15% | 0.00% | 101% | 100% |

Results for method 2 implemented on top of EE2-2.1b that uses 4 reference lines available in the extended MRL list are shown in Table 8 and Table 9.

Table 8. Performance of the proposed method 2 (AI configuration), 4 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.08% | 0.18% | 100% | 98% |
| Class A2 | -0.07% | 0.02% | -0.02% | 100% | 98% |
| Class B | -0.11% | -0.21% | -0.19% | 100% | 98% |
| Class C | -0.09% | -0.09% | -0.05% | 101% | 100% |
| Class E | -0.07% | -0.14% | 0.01% | 101% | 101% |
| Overall | -0.08% | -0.11% | -0.04% | 100% | 99% |
| Class D | -0.03% | 0.18% | -0.10% | 101% | 99% |
| Class F | -0.05% | 0.00% | -0.03% | 101% | 99% |
| Class TGM | -0.16% | -0.06% | -0.07% | 99% | 96% |

Table 9. Performance of the proposed method 2 (RA configuration), 4 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.07% | -0.19% | 97% | 102% |
| Class A2 | -0.05% | 0.05% | 0.08% | 97% | 101% |
| Class B | -0.04% | 0.18% | 0.19% | 100% | 100% |
| Class C | 0.01% | -0.04% | 0.24% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.07% | 0.11% | 99% | 101% |
| Class D | 0.03% | 0.12% | 0.03% | 100% | 100% |
| Class F | -0.02% | 0.21% | -0.04% | 100% | 100% |
| Class TGM | -0.11% | -0.08% | -0.07% | 101% | 100% |

The results for a combination of method 1 and method 2 obtained for AI and RA configurations in accordance with [1] are given in Table 10 and Table 11. This combination is implemented on top of EE-2.1b that uses 4 additional reference lines.

Table 10. Performance of the proposed combination of method 1 and method 2 (AI configuration), 4 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | 0.01% | 99% | 97% |
| Class A2 | -0.08% | -0.06% | -0.08% | 100% | 98% |
| Class B | -0.14% | -0.11% | -0.12% | 100% | 101% |
| Class C | -0.11% | 0.02% | -0.01% | 100% | 100% |
| Class E | -0.09% | -0.13% | -0.10% | 100% | 101% |
| Overall | -0.10% | -0.06% | -0.06% | 100% | 100% |
| Class D | -0.02% | -0.18% | -0.21% | 100% | 101% |
| Class F | -0.10% | 0.09% | -0.21% | 100% | 101% |
| Class TGM | -0.20% | -0.07% | -0.10% | 99% | 101% |

Table 11. Performance of the proposed combination of method 1 and method 2  (RA configuration), 4 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.12% | 97% | 102% |
| Class A2 | -0.08% | -0.01% | -0.14% | 97% | 102% |
| Class B | -0.04% | -0.01% | 0.00% | 101% | 101% |
| Class C | -0.03% | 0.09% | 0.28% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | 0.02% | 99% | 101% |
| Class D | -0.01% | 0.25% | 0.01% | 101% | 101% |
| Class F | -0.02% | -0.10% | 0.04% | 102% | 101% |
| Class TGM | -0.08% | -0.09% | -0.06% | 102% | 100% |

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-X2017, October 2021.

# Conclusions

This contribution presents modifications of the extended MRL list studied in EE2-2.1. We recommend to adopt them into the ECM reference software.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).