| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0162-r1 |
| --- | --- |

| Title: | AHG 7: Gradual Decoding Refresh for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
|  | Tangi Poirier Gaëlle Martin-Cocher |  | Tangi.Poirier@InterDigital.com Gaelle.Martin-Cocher@InterDigital.com |
| Source: | Nokia, InterDigital |  |  |

_____________________________

# Abstract

This contribution presents some preliminary results of GDR implementation for ECM 3.0 [1]. GDR has been implemented in VTM [2], where constraints are imposed on coding tools to avoid possible leaks. Similar GDR implementation was ported into ECM 3.0. This GDR implementation has been tested with turning off ECM new coding tools, and no leaks have been observed.

Simulations were conducted under LDP configuration specified by LLCC AhG [3]. The results show an overall performance loss against the anchor by x.xx%.

# Introduction

Enhanced Compression Model (ECM) [1] is currently under coordinated exploration study by the Joint Video Exploration Team (JVET) of ITU-T VCEG and ISO/IEC MPEG as potential enhanced video coding technology beyond the capabilities of VVC. One of functionalities that VVC supports is Gradual Decoding Refresh (GDR). Many of coding tools in VVC however may cause leaks for GDR. VTM includes a GDR implementation, where constraints are imposed on those coding tools to prevent leaks. Similar GDR implementation was ported into ECM 3.0. This GDR implementation has been tested with the following macros disabled, BASE_ENCODER, BASE_NORMATIVE, TOOLS, and no leaks have been observed. However, if ECM new coding tools are enabled, leaks will occur. Additional constraints on those ECM new coding tools are required to avoid leaks.

.

# Experimental Results

Simulations were conducted over JVET test sequences. GDR tests are performed using low-delay P configuration specified by LLCC AhG [2] with the following setting,

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every (frame rate + 4) pictures,

- GDR period (distance between two consecutive GDR pictures) is set to (frame rate + 4),

- GOP interval (distance between GDR picture and associated recovery point picture) is set to (frame rate),

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

For comparison purpose, anchor bitstreams are encoded using low-delay P configuration with the following setting,

- The first picture is coded as an IDR picture,

- A CRA picture is inserted every (frame rate +4) pictures,

- CRA period (distance between two consecutive CRA pictures) is set to (frame rate + 4),

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

The performance results are shown in Table 1 in terms of BD rate. The overall performance loss of GDR against the anchor is about x.xx%. Note that a loss for GDR should be expected because CUs in clean area are not allowed to use any coding information from dirty areas.

Table 1. Simulation Results of proposed GDR

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Anchor (ECM 3.0) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 2.17% | 7.64% | 8.17% | 99% | 104% |
| Class E | 7.53% | 16.79% | 16.28% | 100% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.97% | 9.10% | 8.92% | 95% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

GDR implementation was ported into ECM 3.0. Simulations were conducted under low-delay P configuration specified by LLCC AhG [2]. The implementation was tested for leaks, and without enabling the new ECM’s coding tools, no leaks are observed. The preliminary results show a loss of x.xx% against the anchor. Further work is needed to enable ECM coding tools for GDR implementation.

# References

- M. Coban, F. L. Léannec, and J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM 3)”, JVET-X2025, Oct. 2021.

- S. Hong, L. Wang, K. Panusopone, “GDR Software”, JVET-U0097, Jan. 2021.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser (Interdigital), J. Xu (Bytedance), D. Nicholson (Ektacom), M. Sychev (Huawei), L. Wang (Nokia), S. Liu, W. Yang (Tencent), J.M. Tiesse (VITEC), M. Karczewicz (Qualcomm), “AHG-7, LLCC Scenarios and baseline configurations”, JVTE -Y0043, Jan. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).