| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0203-v1 |
| --- | --- |

| Title: | EE2-related: a combination of the extended MRL candidate list (JVET-Y0149) modifications with the extended LCU boundary processing area (JVET-Y0140) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Damian Ruiz Coll, Esmael Dinan | Tel: Email: | {afilippov, vrufitskiy} @ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents results of a combined modification of extended MRL candidate list (proposed in JVET-Y0149) and the extension of LCU boundary processing area (JVET-Y0140). The proposed combination provides the following overall results for Test 1: -0.17% (Y) / -0.17% (U) / -0.11% (V) and x.xx% (Y) / x.xx% (U) / x.xx% (V) in AI and RA configurations, respectively. If to combine just the extension of LCU boundary processing area by 5 reference lines and EE2-2.1a (Test 2), the following overall results are reported: -0.13% (Y) / -0.13% (U) / -0.13% (V) and x.xx% (Y) / x.xx% (U) / x.xx% (V) in AI and RA configurations, respectively. If to perform Test 1 with 3 reference lines in LCU boundary constraints (Test 3) the following overall results are reported:  -0.15% (Y) / -0.15% (U) / -0.15% (V) and x.xx% (Y) / x.xx% (U) / x.xx% (V) in AI and RA configurations, respectively. If to combine just the extension of LCU boundary processing area by 3 reference lines and EE2-2.1a (Test 4), the following overall results are reported: -0.10% (Y) / -0.14% (U) / -0.11% (V) and x.xx% (Y) / x.xx% (U) / x.xx% (V) in AI and RA configurations, respectively.

# Introduction

In Exploration Experiment 2 Test 2.1 MRL list was extended to keep 5, 4 or 3 reference lines available in the extended MRL list. The purpose of this contribution is to test modifications proposed for modification of MRL list extension when LCU boundary processing area is extended

# Proposed method

The tested combination comprises method 1 of [3] and one of the methods of [2], either 2 or 3.

In method 1 it is proposed to align the size of TIMD template and the values inside MRL list, so that position of the reference line in some case may coincide with the position of reference line that was used to derive this mode in TIMD. When block is of rectangular shape it may occur that template width on the left side of a block is not equal to the height of a template above the block. It is proposed to select MRL list for above and left side individually. When TIMD flag is on, MRL list for the left side is proposed to be selected as given in Tab1e 1.

Table 1. MRL list for the left side when TIMD flag is on

| Condition | width ≤8 | width >8 |
| --- | --- | --- |
| TIMD left template width (as of ECM-3.1) | 2 | 4 |
| MRL list | {0, 1, 2} | {0, 2, 4} |

When TIMD flag is on, MRL list for the above side is proposed to be selected as given in Tab1e 2.

Table 2. MRL list for the above side when TIMD flag is on

| Condition | height ≤8 | height >8 |
| --- | --- | --- |
| TIMD above template height (as of ECM-3.1) | 2 | 4 |
| MRL list | {0, 1, 2} | {0, 2, 4} |

In Test 1 and Test 2, method 3 of [2] is used. LCU boundary area extension is performed using the 3rd method of [2] that utilized 5 reference lines above an LCU boundary. Specifically, in CCLM the same 2×3 down-sampling filter is applied on the above block boundary without checking whether a block is on the LCU boundary or not. Besides, MRLP is not disabled for a block on an LCU boundary. However, MRLP is constrained on the encoder side to use maximum of 5 reference lines adjacent to an LCU boundary. Test 2 is different from Test 1 in the following aspect: it does not comprise the extended MRL candidate list modifications (method 1) described in [3].

In Test 3 and Test 4, method 2 of [2] is used, i.e. only 3 reference lines above an LCU boundary are utilized. In these tests, when a block is on above LCU boundary (and not on the slice/tile boundary), if TIMD is enabled, TIMD above template side is set equal to 2 without consideration of a block height. Besides, in CCLM the same 2×3  down-sampling filter is applied on the above block boundary without checking whether a block is on the LCU boundary or not. Besides, MRLP is not disabled for a block on an LCU boundary. Test 4 is different from Test 3 in the following aspect: it does not comprise the extended MRL candidate list modifications (method 1) described in [3].

# Results

This combination was implemented on top of test EE2-2.1. The performance of Test 1 is evaluated in accordance with [1] and shown in Table 3 and Table 4.

Table 3. Performance of Test 1 over ECM-3.1 anchor (AI configuration), 5 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.06% | -0.08% | 101% | 97% |
| Class A2 | -0.13% | -0.05% | -0.06% | 100% | 97% |
| Class B | -0.22% | -0.24% | -0.25% | 101% | 100% |
| Class C | -0.20% | -0.21% | -0.10% | 100% | 101% |
| Class E | -0.19% | -0.26% | 0.06% | 102% | 100% |
| Overall | -0.17% | -0.17% | -0.11% | 101% | 99% |
| Class D | -0.07% | -0.07% | -0.17% | 101% | 101% |
| Class F | -0.14% | -0.05% | -0.09% | 100% | 100% |
| Class TGM | -0.31% | -0.22% | -0.24% | 99% | 99% |

Table 4. Performance of Test 1 over ECM-3.1 anchor (RA configuration), 5 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class B | -0.08% | -0.01% | -0.02% | 101% | 100% |
| Class C | -0.08% | 0.12% | 0.15% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | -0.03% | 0.42% | -0.04% | 101% | 100% |
| Class F | -0.11% | -0.33% | -0.36% | 101% | 100% |
| Class TGM | -0.24% | -0.19% | -0.11% | 101% | 100% |

Test results of performance evaluation of Test 2 are given in Table 5 and Table 6.

Table 5. Performance of Test 2 over ECM-3.1 anchor (AI configuration), 5 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.04% | 99% | 97% |
| Class A2 | -0.09% | -0.01% | -0.04% | 99% | 97% |
| Class B | -0.15% | -0.19% | -0.25% | 102% | 101% |
| Class C | -0.15% | -0.17% | -0.17% | 101% | 100% |
| Class E | -0.17% | -0.16% | -0.09% | 102% | 100% |
| Overall | -0.13% | -0.13% | -0.13% | 101% | 99% |
| Class D | -0.05% | -0.14% | -0.13% | 102% | 101% |
| Class F | -0.18% | -0.03% | -0.15% | 101% | 99% |
| Class TGM | -0.23% | -0.15% | -0.20% | 101% | 99% |

Table 6. Performance of Test 2 over ECM-3.1 anchor (RA configuration), 5 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class B | -0.06% | -0.08% | 0.06% | 100% | 100% |
| Class C | -0.03% | -0.10% | 0.11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | 0.00% | 0.29% | -0.15% | 101% | 101% |
| Class F | -0.04% | 0.06% | -0.25% | 101% | 100% |
| Class TGM | -0.13% | -0.10% | -0.10% | 101% | 100% |

The performance of Test 3 is evaluated in accordance with [1] and shown in Table 7 and Table 8.

Table 7. Performance of Test 3 over ECM-3.1 anchor (AI configuration), 3 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.04% | -0.12% | 101% | 97% |
| Class A2 | -0.12% | -0.03% | -0.04% | 100% | 98% |
| Class B | -0.20% | -0.22% | -0.27% | 101% | 100% |
| Class C | -0.18% | -0.16% | -0.22% | 100% | 100% |
| Class E | -0.15% | -0.14% | -0.03% | 101% | 100% |
| Overall | -0.15% | -0.13% | -0.15% | 101% | 99% |
| Class D | -0.07% | -0.01% | -0.19% | 101% | 101% |
| Class F | -0.17% | -0.04% | -0.19% | 99% | 99% |
| Class TGM | -0.31% | -0.20% | -0.22% | 99% | 99% |

Table 8. Performance of Test 3 over ECM-3.1 anchor (RA configuration), 3 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | -0.21% | 98% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class B | -0.10% | -0.09% | -0.01% | 101% | 100% |
| Class C | -0.04% | -0.11% | 0.17% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | -0.03% | 0.06% | 0.04% | 101% | 101% |
| Class F | -0.08% | -0.17% | -0.23% | 101% | 100% |
| Class TGM | -0.15% | -0.18% | -0.15% | 101% | 100% |

The performance of Test 4 is evaluated in accordance with [1] and shown in Table 9 and Table 10.

Table 9. Performance of Test 4 over ECM-3.1 anchor (AI configuration), 3 reference lines available in the extended MRL list

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.18% | -0.02% | 101% | 98% |
| Class A2 | -0.08% | 0.00% | 0.01% | 101% | 99% |
| Class B | -0.14% | -0.15% | -0.15% | 100% | 100% |
| Class C | -0.12% | -0.16% | -0.27% | 101% | 101% |
| Class E | -0.10% | -0.22% | -0.03% | 102% | 103% |
| Overall | -0.10% | -0.14% | -0.11% | 101% | 100% |
| Class D | -0.04% | 0.10% | -0.12% | 101% | 98% |
| Class F | -0.06% | 0.01% | -0.09% | 101% | 101% |
| Class TGM | -0.25% | -0.15% | -0.20% | 99% | 97% |

Table 10. Performance of Test 4 over ECM-3.1 anchor (RA configuration), 3 reference lines available in the extended MRL list

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class B | -0.06% | 0.02% | 0.04% | 101% | 100% |
| Class C | -0.02% | 0.01% | 0.26% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | -0.03% | 0.10% | -0.20% | 102% | 101% |
| Class F | -0.03% | -0.03% | -0.21% | 102% | 100% |
| Class TGM | -0.14% | -0.12% | -0.12% | 102% | 100% |

# Conclusions

This contribution presents a combination of the extension of LCU boundary reference area and the modifications of the extended MRL list studied in tests EE2-2.1. We recommend to adopt one of the proposed methods into the ECM reference software.

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-X2017, October 2021.

- A. Filippov, V. Rufitskiy, D. Ruiz Coll “EE2-related: On the LCU boundary processing by intra-prediction tools”, ISO/IEC JTC1/SC29/WG11 JVET-Y0140, January 2022.

- A. Filippov, V. Rufitskiy, E. Dinan “EE2-related: Modifications of the extended MRL candidate list”, ISO/IEC JTC1/SC29/WG11 JVET-Y0149, January 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).