| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0209 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0088 (EE2-related: IBC with Template Matching) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Na Zhang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-Y0088 which is about IBC with Template Matching. It is confirmed that the available results match with those provided in JVET-Y0088.

# Experimental Results

JVET-Y0088 [1] was implemented on top of ECM-3.1 [2] and was evaluated following the common test conditions [3]. The crosscheck results are summarized in Table 1 to Table 2.

Table 1: Results of IBC-TM on top of ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.59% | -0.45% | -0.42% | 106% | 102% |
| Class TGM | -0.47% | -0.43% | -0.44% | 104% | 108% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.37% | -0.49% | 104% | 100% |
| Class TGM | -0.36% | -0.31% | -0.26% | 102% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.81% | -0.72% | 107% | 100% |
| Class TGM | -0.20% | -0.06% | -0.14% | 104% | 102% |

Table 2: Combination of EE2-3.13 test and IBC-TM on top of ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.83% | -0.74% | -0.77% | 105% | 100% |
| Class TGM | -1.53% | -1.37% | -1.41% | 105% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.58% | -0.38% | -1.00% | 104% | 99% |
| Class TGM | -1.29% | -1.20% | -1.16% | 102% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.41% | -1.45% | -1.63% | 107% | 100% |
| Class TGM | -1.13% | -1.04% | -1.05% | 104% | 101% |

# References

[1] A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin, “EE2-related: IBC with Template Matching,” JVET-Y0088, Jan. 2022.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-3.1

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-X2017, Oct. 2021.