| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0210-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0091 Non-EE2: MVP refinement for regular AMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Chen | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-Y0091[1]. It is confirmed that the results match with the results reported in JVET-Y0091.

Introduction

JVET-Y0091 presents a method of MVP refinement for regular AMVP mode, in which before using template-based searching to refine the MVP, two changes have been made. First, two out of five candidates are chosen from the candidate list by comparing the template matching costs. Second for each of the two selected candidates, template-based searching process is operated with a search pattern indicated by MMVD to derive an initial refined MVP with minimum template matching cost. Then the template-based searching process in ECM is conducted to the initial refined MVP.

Three tests are reported in JVET-Y0091. Test 1 directly applies the proposed method. Test 2 does some simplification on top of Test 1, and Test 3 conditionally applies method in of Test 2.

Crosscheck results

All of the three tests of JVET-Y0091 are crosschecked in this contribution. The tests are conducted under CTC[2] and results are confirmed to be identical to those reported in JVET-Y0091.

Table 1. Test1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.29% | -0.48% | 107% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.09% | -0.13% | -0.08% | 108% | 101% |
| Class C | -0.12% | -0.18% | -0.20% | 110% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | 0.06% | -0.17% | 110% | 102% |
| Class F | -0.09% | -0.21% | -0.15% | 105% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.20% | -0.28% | -0.52% | 139% | 105% |
| Class E | -0.06% | 0.16% | -0.14% | 130% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.41% | -0.92% | 140% | 105% |
| Class F | -0.09% | -0.23% | -0.23% | 121% | 101% |

Table 2. Test2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.31% | -0.56% | 104% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.08% | -0.24% | -0.06% | 104% | 101% |
| Class C | -0.11% | -0.11% | -0.16% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.09% | -0.09% | -0.08% | 106% | 102% |
| Class F | -0.06% | -0.01% | -0.15% | 103% | 100% |

Table 3. Test 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.17% | -0.25% | 99% | 99% |
| Class A2 | -0.18% | -0.14% | -0.18% | 100% | 100% |
| Class B | -0.13% | -0.07% | -0.06% | 99% | 100% |
| Class C | -0.17% | -0.07% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.11% | -0.12% | 100% | 100% |
| Class D | -0.19% | -0.10% | -0.18% | 100% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

This contribution is a crosscheck report of JVET-Y0091. It is confirmed that the results match with the results reported in JVET-Y0091.

References

C. Zhou, Z. Lv, J. Zhang, “Non-EE2: MVP refinement for regular AMVP mode”, JVET-Y0091, Jan. 2022

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021