| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0051-v1 |
| --- | --- |

| Title: | EE2-1.2: On chroma intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test 1.2. Two aspects of chroma intra prediction are proposed: 1) a DIMD chroma mode, which derives a chroma intra prediction mode based on the collocated reconstructed luma samples by reusing the DIMD derivation method is proposed; 2) a non-LM mode can be fused with the MMLM_LT mode. The coding performance of each aspect and the combination of the two aspects are investigated. Furthermore, a new DMD chroma mode derivation method that can reduce the process by using neighboring reconstructed samples is proposed. It is reported that on top of ECM-4.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is:

- Test 1.2a (DIMD chroma mode):

AI {-0.08%, -0.59%, -0.51%, 102%, 102%}, RA {-0.04%, -0.37%, -0.20%, 102%, 100%}

- Test 1.2b (Fusion of chroma intra prediction modes):

AI {-0.01%, -0.66%, -0.61%, 102%, 100%}, RA {-0.01%, -0.27%, -0.13%, 101%, 100%}

- Test 1.2c (DIMD chroma mode + Fusion of chroma intra prediction modes):

AI {-0.09%, -1.31%, -1.16%, 103%, 101%}, RA {-0.04%, -0.64%, -0.45%, 101%, 100%}

- Test 1.2d (DIMD chroma mode with reduced processing + Fusion of chroma intra prediction modes):

AI {-0.07%, -1.17%, -1.02%, 103%, 100%}, RA {-0.04%, -0.52%, -0.40%, 101%, 100%}

Introduction

The intra prediction modes enabled for the chroma components in ECM-4.0 [1] are six cross-component linear model (LM) modes including CCLM_LT, CCLM_L, CCLM_T, MMLM_LT, MMLM_L and MMLM_T modes, the direct mode (DM), and four default chroma intra prediction modes. The four default modes are given by the list {0, 50, 18, 1} and if the DM mode already belongs to that list, the mode in the list will be replaced with mode 66.

A decoder-side intra mode derivation (DIMD) method for luma intra prediction is included in ECM-4.0. First, a horizontal gradient and a vertical gradient are calculated for each reconstructed luma sample of the L-shaped template of the second neighboring row and column of the current block to build a Histogram of Gradients (HoG). Then, the two intra prediction modes with the largest and the second largest histogram amplitude values are blended with the Planar mode to generate the final predictor of the current luma block.

In order to improve the coding efficiency of chroma intra prediction, two methods are proposed in the last JVET meeting [2], and studied in EE2 Test 1.2 [3], including a decoder-side derived chroma intra prediction mode (DIMD chroma) and a fusion of a non-LM mode and the MMLM_LT mode.

Proposed method

Test 1.2a: DIMD chroma mode

In the Test 1.2a, a DIMD chroma mode is proposed. The proposed DIMD chroma mode uses the DIMD derivation method to derive the chroma intra prediction mode of the current block based on the collocated reconstructed luma samples. Specifically, a horizontal gradient and a vertical gradient are calculated for each collocated reconstructed luma sample of the current chroma block to build a HoG, as shown in Figure 1. Then the intra prediction mode with the largest histogram amplitude values is used for performing chroma intra prediction of the current chroma block.

Figure 1 the used collocated reconstructed luma samples

When the intra prediction mode derived from the DIMD chroma mode is the same as the intra prediction mode derived from the DM mode, the intra prediction mode with the second largest histogram amplitude value is used as the DIMD chroma mode.

A CU level flag is signaled to indicate whether the proposed DIMD chroma mode is applied as shown in Table 1.

Table 1. The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 1100 | list[0] |
| 1 | 1101 | list[1] |
| 2 | 1110 | list[2] |
| 3 | 1111 | list[3] |
| 4 | 10 | DIMD chroma |
| 5 | 0 | DM |

Test 1.2b: Fusion of chroma intra prediction modes

In the Test 1.2b, it is proposed that the DM mode and the four default modes can be fused with the MMLM_LT mode as follows:

pred=w0*pred0+w1*pred1+(1≪(shift-1))≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the MMLM_LT mode and pred is the final predictor of the current chroma block. The two weights, w0 and w1 are determined by the intra prediction mode of adjacent chroma blocks and shift is set equal to 2. Specifically, when the above and left adjacent blocks are both coded with LM modes, {w0, w1}={1, 3}; when the above and left adjacent blocks are both coded with non-LM modes, {w0, w1}={3, 1}; otherwise, {w0, w1}={2, 2}.

For the syntax design, if a non-LM mode is selected, one flag is signaled to indicate whether the fusion is applied. And the proposed fusion is only applied to I slices.

Test 1.2c: Test 1.2a + Test 1.2b

In the Test 1.2c the DIMD chroma mode and the fusion of chroma intra prediction modes are combined. Specifically, the DIMD chroma mode described in Test 1.2a is applied, and for I slices, the DM mode, the four default modes and the DIMD chroma mode can be fused with the MMLM_LT mode using the weights described in Test 1.2b, while for non-I slices, only the DIMD chroma mode can be fused with the MMLM_LT mode using equal weights.

Test 1.2d: Test 1.2a with reduced processing + Test 1.2b

In the Test 1.2d the DIMD chroma mode with reduced processing and the fusion of chroma intra prediction modes are combined. Specifically, the DIMD chroma mode with reduced processing derives the intra mode based on the neighboring reconstructed Y, Cb and Cr samples in the second neighboring row and column as shown in Figure 2. Other parts are the same as Test 1.2c.

Figure 2 the used neighboring reconstructed samples

Simulation results

The described methods are implemented on top ECM-4.0 software [4] and are tested under CTC for enhanced compression tool testing [5].

Table 2. Simulation results of Test 1.2a

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.53% | -0.27% | 101% | 102% |
| Class A2 | -0.09% | -0.59% | -0.70% | 103% | 102% |
| Class B | -0.07% | -0.40% | -0.52% | 102% | 101% |
| Class C | -0.07% | -0.38% | -0.36% | 102% | 101% |
| Class E | -0.13% | -1.25% | -0.72% | 102% | 102% |
| Overall | -0.08% | -0.59% | -0.51% | 102% | 102% |
| Class D | -0.05% | -0.39% | -0.46% | 102% | 101% |
| Class F | -0.08% | -0.16% | -0.27% | 101% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.30% | 0.01% | 102% | 100% |
| Class A2 | -0.02% | -0.44% | -0.30% | 102% | 100% |
| Class B | -0.03% | -0.34% | -0.27% | 101% | 99% |
| Class C | -0.06% | -0.41% | -0.20% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.37% | -0.20% | 102% | 100% |
| Class D | -0.03% | -0.37% | -0.32% | 104% | 101% |
| Class F | -0.04% | -0.30% | -0.20% | 101% | 100% |

Table 3. Simulation results of Test 1.2b

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.75% | -0.62% | 101% | 101% |
| Class A2 | 0.00% | -0.58% | -0.51% | 103% | 101% |
| Class B | -0.02% | -0.57% | -0.50% | 101% | 100% |
| Class C | -0.01% | -0.67% | -0.62% | 103% | 100% |
| Class E | 0.01% | -0.76% | -0.86% | 102% | 100% |
| Overall | -0.01% | -0.66% | -0.61% | 102% | 100% |
| Class D | -0.02% | -0.25% | -0.30% | 103% | 100% |
| Class F | -0.09% | -0.29% | -0.21% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.40% | 0.09% | 101% | 101% |
| Class A2 | -0.01% | -0.35% | -0.18% | 101% | 100% |
| Class B | -0.02% | -0.13% | -0.22% | 100% | 100% |
| Class C | -0.01% | -0.31% | -0.14% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.27% | -0.13% | 101% | 100% |
| Class D | 0.03% | -0.55% | -0.24% | 102% | 100% |
| Class F | -0.05% | -0.50% | -0.25% | 101% | 100% |

Table 4. Simulation results of Test 1.2c

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -1.25% | -0.83% | 103% | 102% |
| Class A2 | -0.08% | -1.22% | -1.23% | 105% | 102% |
| Class B | -0.09% | -1.08% | -1.13% | 102% | 101% |
| Class C | -0.08% | -1.14% | -1.04% | 103% | 100% |
| Class E | -0.10% | -2.04% | -1.66% | 102% | 100% |
| Overall | -0.09% | -1.31% | -1.16% | 103% | 101% |
| Class D | -0.07% | -0.67% | -0.69% | 103% | 101% |
| Class F | -0.14% | -0.79% | -0.65% | 102% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.48% | -0.06% | 102% | 100% |
| Class A2 | -0.04% | -0.75% | -0.51% | 102% | 100% |
| Class B | -0.03% | -0.62% | -0.59% | 101% | 100% |
| Class C | -0.04% | -0.70% | -0.52% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.64% | -0.45% | 101% | 100% |
| Class D | -0.02% | -0.51% | -0.69% | 102% | 100% |
| Class F | -0.04% | -0.63% | -0.45% | 101% | 100% |

Table 5. Simulation results of Test 1.2d

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -1.17% | -0.75% | 102% | 101% |
| Class A2 | -0.05% | -1.13% | -0.96% | 105% | 101% |
| Class B | -0.08% | -1.00% | -1.03% | 102% | 100% |
| Class C | -0.07% | -1.01% | -0.94% | 104% | 99% |
| Class E | -0.08% | -1.70% | -1.42% | 103% | 100% |
| Overall | -0.07% | -1.17% | -1.02% | 103% | 100% |
| Class D | -0.06% | -0.60% | -0.56% | 104% | 100% |
| Class F | -0.08% | -0.60% | -0.56% | 102% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.55% | -0.13% | 102% | 100% |
| Class A2 | -0.05% | -0.52% | -0.36% | 102% | 100% |
| Class B | -0.05% | -0.58% | -0.60% | 101% | 100% |
| Class C | -0.05% | -0.40% | -0.38% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.52% | -0.40% | 101% | 100% |
| Class D | -0.05% | -0.58% | -0.70% | 102% | 100% |
| Class F | -0.07% | -0.64% | -0.42% | 101% | 100% |

References

- M. Coban, et. al., “Algorithm description of Enhanced Compression Model 4 (ECM 4),” JVET-Y2025, April 2022.

- X. Li, et. al., “Non-EE2: On chroma intra prediction mode,” JVET-Y0092, January 2022.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-Y2024, January 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).