| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0052-v1 |
| --- | --- |

| Title: | AHG9: NNR post-filter SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Maria Santamaria, Francesco Cricri, Emre B. Aksu, Hamed. R. Tavakoli Nokia Technologies Tampere, Finland Takeshi Chujoh, Yukinobu Yasugi, Tomohiro Ikai Sharp Corporation Chiba, Japan | Email: | firstname.lastname@nokia.com chujoh.takeshi@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Nokia and Sharp |  |  |

_____________________________

# Abstract

This contribution proposes an NNR post-filter SEI message into a working draft of a VSEI amendment. The NNR post-filter SEI message may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter. The NNR post-filter SEI message proposed in this contribution combines aspects of JVET-Y0073, JVET-Y0074, JVET-Y0075, and JVET-Y0115.

# Introduction

The contributions related to Neural Network Representation (NNR, ISO/IEC 15938-17, also known as Neural Network Coding, NNC) based post-filter SEI messages presented in the 25th JVET meeting can be summarized as follows:

- JVET-Y0073 [1] proposed the input and output chroma sampling formats of the post filter.

- JVET-Y0074 [2] proposed indicating input and output tensor format information with separate syntax elements and a picture-wise input/output conversion process.

- JVET-Y0075 [3] proposed to include information of neural network complexity in the SEI message.

- JVET-Y0115 [4] proposed indicating input and output tensor format information with a single syntax element and patch-wise input/output processing.

As a response to reviewing JVET-Y0073, JVET-Y0074, JVET-Y0075, and JVET-Y0115 in the 25th JVET meeting, it was suggested to combine everything into one SEI message. This contribution responds to the suggestion by combining aspects of JVET-Y0073, JVET-Y0074, JVET-Y0075, and JVET-Y0115 into a single NNR post-filter SEI message.

In the 25th JVET meeting it was also requested to clarify the relation to NNR. This request has been considered and addressed as follows in this contribution:

- Since the finalization of NNR version 2 is likely to take place before the finalization of the next VSEI amendment, this contribution requires the update of the neural network (nnrpf_mode_idc equal to 3) to conform to NNR version 2. Consequently, the NNR update bitstream uses the incremental weight update mechanisms specified in NNR version 2. It is clarified in the text that an NNR decoder takes care of updating the neural network by decoding the NNR version 2 bitstream.

- NNR specifies the interface for (de)compressing a neural network but not the input/output format of the neural network or how to run it (e.g., what is the required preprocessing of the input data). Thus, the input and output format of the neural network have been specified in the proposed NNR post-filter SEI message. This includes the following aspects:

  - Conversions from sample values to neural network input and from neural network output back to sample values have been specified. The representation format of numbers, including their precision or bit count, have been included in the specified conversions.

  - The batch index has been added as the first dimension in the input and output tensors to follow the practice in many neural network frameworks, such as TensorFlow and PyTorch. While the semantics of this SEI message use batch size equal to 1, it is up to the post-processing engine to determine the batch size used as input to the neural network inference.

  - It is asserted that there are varying practices whether to include the channel (e.g., the colour component) as the second dimension (after the batch index) or as the last dimension in the input and output tensors. Hence, a flag (nnrpf_component_last_flag) has been added in the syntax.

Section 2 contains the proposal for an amendment of VSEI. Section 3 contains the proposed VVC specification text for interfacing with the NNR post-filter SEI message. Section 4 includes a brief review of earlier contributions and some observations drawn from them.

# VSEI proposal

## Overview

This contribution proposes an NNR post-filter SEI message into a working draft of a VSEI amendment. The NNR post-filter may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter. The following changes have been made compared to JVET-Y0115:

- In addition to quality enhancement, the following filtering purposes have been added:

  - Chroma upsampling (e.g., from 4:2:0 and 4:4:4) has been added as one of the filtering purposes, as suggested by JVET-Y0073, and is controlled by two flags (nnrpf_out_sub_width_c_flag and nnrpf_out_sub_height_c_flag) indicating if vertical or horizontal chroma sampling is modified compared to the chroma format of the input picture.

  - Super resolution has been re-introduced as one of the filtering purposes.

  - Joint super resolution and chroma format upsampling has also been enabled.

- nnrpf_component_last_flag has been added in the syntax to indicate whether the channel is indicated as the second or last dimension in the input and output tensors.

- The input and output tensor formats are indicated with separate syntax elements, as proposed in JVET-Y0074.

- The normalization of the sample values to the input and output tensors as well as the number representation format of the normalized values is indicated with two syntax elements to respond to a comment expressed on JVET-Y0074.

- A flag (nnrpf_constant_patch_size_flag) has been added to indicate, when equal to 0, that the patch size used as input to the filter can be any positive integer multiple of the signalled size, or, when equal to 1, that the patch size used as input to the filter must be exactly the signalled size. Value 0 can typically be used for convolutional filters.

- A syntax element nnrpf_complexity_idc has been added to indicate whether and which type of complexity description is present, as inspired by JVET-Y0075. The content for the complexity description is a topic of another contribution.

## NNR post-filter SEI message

| nnr_post_filter( payloadSize ) { | Descriptor |
| --- | --- |
| nnrpf_id | u(21) |
| nnrpf_mode_idc | u(2) |
| if( nnrpf_mode_idc > 0 ) { |  |
| nnrpf_persistence_flag | u(1) |
| if( nnrpf_mode_idc = = 1 ¦ ¦ nnrpf_mode_idc = = 2 ) { |  |
| nnrpf_purpose | ue(v) |
| if( nnrpf_purpose = = 1 ¦ ¦ nnrpf_purpose = = 3 ) { |  |
| nnrpf_out_sub_width_c_flag | u(1) |
| nnrpf_out_sub_height_c_flag | u(1) |
| } |  |
| if( nnrpf_purpose = = 2 ¦ ¦ nnrpf_purpose = = 3 ) { |  |
| nnrpf_pic_width_in_luma_samples | ue(v) |
| nnrpf_pic_height_in_luma_samples | ue(v) |
| } |  |
| nnrpf_component_last_flag | u(1) |
| nnrpf_inp_sample_idc | ue(v) |
| nnrpf_inp_order_idc | ue(v) |
| nnrpf_out_sample_idc | ue(v) |
| nnrpf_out_order_idc | ue(v) |
| nnrpf_constant_patch_size_flag | u(1) |
| nnrpf_patch_size_minus1 | ue(v) |
| nnrpf_overlap | ue(v) |
| nnrpf_complexity_idc | ue(v) |
| if( nnrpf_complexity_idc > 0 ) { |  |
| nnrpf_complexity_element( nnrpf_complexity_idc ) |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nnrpf_reserved_zero_bit | u(1) |
| } |  |
| if( nnrpf_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ¦ ¦ nnrpf_mode_idc = = 3 ) { |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnrpf_payload_byte[ i ] | b(8) |
| } |  |
| } |  |
| } |  |

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnrpf_id).

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by InpPicWidthInLumaSamples and InpPicHeightInLumaSamples, respectively.

–	Luma sample array CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, of the cropped decoded output picture for vertical coordinates y and horizontal coordinates x, where the top-left corner of the sample array has coordinates y equal to 0 and x equal to 0.

–	Bit depth BitDepthY for the luma sample array of the cropped decoded output picture.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the cropped decoded output picture.

–	Chroma subsampling ratio relative to luma denoted as InpSubWidthC and InpSubHeightC.

–	When nnrpf_inp_order_idc is equal to 3, the initial luma quantization parameter value SliceQPY.

When this SEI message specifies a neural network that may be used as a post-processing filter, the semantics specify the derivation of the luma sample array FilteredYPic[ y ][ x ] and chroma sample arrays FilteredCbPic[ y ][ x ] and FilteredCrPic[ y ][ x ], as indicated by the value of nnrpf_out_order_idc, that contain the output of the post-processing filter.

nnrpf_id contains an identifying number that may be used to identify a post-processing filter.

Values of nnrpf_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnrpf_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

nnrpf_mode_idc equal to 0 indicates that no post-processing filter associated with the nnrpf_id value is specified in this SEI message and that the persistence of the previous post-processing filter with the same value of nnrpf_id is cancelled. nnrpf_mode_idc equal to 1 specifies that the post-processing filter associated with the nnrpf_id value is a neural network identified by a given Uniform Resource Identifier (URI), with syntax and semantics as specified in IETF Internet Standard 66. nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnrpf_id value is a neural network represented by the ISO/IEC 15938-17 bitstream with general_profile_idc equal to 0 contained in this SEI message. nnrpf_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnrpf_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream with general_profile_idc equal to 1 contained in this SEI message.

nnrpf_persistence_flag specifies the persistence of this SEI message for the current layer.

nnrpf_persistence_flag equal to 0 specifies that this SEI message applies to the current decoded picture only.

nnrpf_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnrpf_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and nnrpf_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnrpf_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and any nnrpf_mode_idc value is output.

nnrpf_purpose indicates the purpose of post-processing filter as specified in Table XYZ1. The value of nnrpf_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnrpf_purpose that do not appear in Table XYZ1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_purpose.

Table XYZ1 – Definition of nnrpf_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Visual quality improvement |
| 1 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 2 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |

NOTE 1 – When a reserved value of nnrpf_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnrpf_purpose being equal to that value.

nnrpf_out_sub_width_c_flag and nnrpf_out_sub_height_c_flag are used to derive the variables outSubWidthC and outSubHeightC, respectively, which specify the chroma subsampling ratio relative to luma in the picture resulting from the post-processing filtering. When not present, nnrpf_out_sub_width_c_flag and nnrpf_out_sub_height_c_flag are both inferred to be equal to 0. When nnrpf_out_sub_width_c_flag and nnrpf_out_sub_height_c_flag are present, the sum of nnrpf_out_sub_width_c_flag and nnrpf_out_sub_height_c_flag shall be greater than 0.

outSubWidthC = InpSubWidthC − nnrpf_out_sub_width_c_flag	(1)

outSubHeightC = InpSubHeightC − nnrpf_out_sub_height_c_flag	(2)

It is a requirement of bitstream conformance that outSubWidthC and outSubHeightC are both greater than 0.

nnrpf_pic_width_in_luma_samples and nnrpf_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by nnrpf_id to a cropped decoded output picture. When nnrpf_pic_width_in_luma_samples and nnrpf_pic_height_in_luma_samples are not present, they are inferred to be equal to InpPicWidthInLumaSamples and InpPicHeightInLumaSamples, respectively.

nnrpf_component_last_flag equal to 0 specifies that the second dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for the channel. nnrpf_component_last_flag equal to 1 specifies that the last dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for the channel.

NOTE 2 – The specified values of nnrpf_inp_sample_idc and nnrpf_out_sample_idc specify that the first dimension in the input tensor and in the output tensor, respectively, is used for the batch index, which is a practice in some neural network frameworks. While the semantics of this SEI message use batch size equal to 1, it is up to the post-processing implementation to determine the batch size used as input to the neural network inference.

NOTE 3 – A colour component is an example of a channel.

nnrpf_inp_sample_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the post-processing filter. When nnrpf_inp_sample_idc is equal to 0, 1, 2, or 3, the input values to the post-processing filter are binary16, binary32, binary64, or binary128 floating point values, respectively, as specified in IEEE 754-2019, and the functions InpY and InpC are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )		(3)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) ) − 1 )		(4)

The value of nnrpf_inp_sample_idc shall be in the range of 0 to 255, inclusive. Values of nnrpf_inp_sample_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_inp_sample_idc.

nnrpf_inp_order_idc indicates the method of ordering the sample arrays of a cropped decoded output picture as the input to the post-processing filter. Table XYZ2A contains an informative description of nnrpf_inp_order_idc values. The semantics of nnrpf_inp_order_idc in the range of 0 to 3, inclusive, are specified in Table XYZ2B, which specifies a process for deriving the input tensors inputTensor for different values of nnrpf_inp_order_idc and a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors. When the chroma format of the cropped decoded output picture is not 4:2:0, nnrpf_inp_order_idc shall not be equal to 3. The value of nnrpf_inp_order_idc shall be in the range of 0 to 255, inclusive. Values of nnrpf_inp_order_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_inp_order_idc.

Table XYZ2A – Informative description of nnrpf_inp_order_idc values

| nnrpf_inp_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the input tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the input tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the input tensor, thus the number of channels is 3. |
| 3 | Four luma matrices, two chroma matrices, and a quantization parameter matrix are present in the input tensor, thus the number of channels is 7. This nnrpf_inp_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | reserved |

nnrpf_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnrpf_patch_size_minus1 as input. nnrpf_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnrpf_patch_size_minus1 as input.

nnrpf_patch_size_minus1 + 1 specifies the horizontal and vertical sample counts of patch sizes of the post-processing filter. The value of nnrpf_patch_size_minus1 shall be in the range of 0 to 32766, inclusive.

nnrpf_overlap specifies the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnrpf_overlap shall be in the range of 0 to 16383, inclusive.

The variables inpPatchWidth, inpPatchHeight, outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

inpPatchWidth = nnrpf_patch_size_minus1 + 1
inpPatchHeight = nnrpf_patch_size_minus1 + 1
outPatchWidth = ( nnrpf_pic_width_in_luma_samples * inpPatchWidth ) / InpPicWidthInLumaSamples
outPatchHeight = ( nnrpf_pic_height_in_luma_samples * inpPatchHeight ) / InpPicHeightInLumaSamples
horCScaling = InpSubWidthC / outSubWidthC
verCScaling = InpSubHeightC / outSubHeightC						(5)
outPatchCWidth = outPatchWidth * horCScaling
outPatchCHeight = outPatchHeight * verCScaling
overlapSize = nnrpf_overlap

It is a requirement of bitstream conformance that outPatchWidth * InpPicWidthInLumaSamples shall be equal to nnrpf_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * InpPicHeightInLumaSamples shall be equal to nnrpf_pic_height_in_luma_samples * inpPatchHeight.

Table XYZ2B – Process for deriving the input tensors inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnrpf_inp_ order_idc | Process DeriveInputTensors( ) for deriving input tensors |
| --- | --- |
| 0 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { y = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP ) x = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP ) if( nnrpf_component_last_flag = = 0 ) inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ y ][ x ] ) else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ y ][ x ] ) } |
| 1 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { y = Clip3( 0, InpPicHeightInLumaSamples / InpSubHeightC – 1, cTop + yP ) x = Clip3( 0, InpPicWidthtInLumaSamples / InpSubWidthC – 1, cLeft + xP ) if( nnrpf_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ y ][ x ] ) } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpC( CroppedCrPic[ y ][ x ] ) } } |
| 2 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yY = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP ) xY = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP ) yC = yY / InpSubHeightC xC = xY / InpSubWidthC if( nnrpf_component_last_flag = = 0 ) inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yY ][ xY ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ yC ][ xC ] ) inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ yC ][ xC ] ) else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ yY ][ xY ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpC( CroppedCbPic[ yC ][ xC ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = InpC( CroppedCrPic[ yC ][ xC ] ) } |
| 3 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yTL = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP * 2 ) xTL = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP * 2 ) yBR = Clip3( 0, InpPicHeightInLumaSamples – 1, yTL + 1 ) xBR = Clip3( 0, InpPicWidthtInLumaSamples – 1, xTL + 1 ) y = Clip3( 0, InpPicHeightInLumaSamples / 2 – 1, cTop / 2 ) x = Clip3( 0, InpPicWidthtInLumaSamples / 2 – 1, cLeft / 2 ) if( nnrpf_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yTL ][ xTL ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yTL ][ xBR ] ) inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yBR ][ xTL ] ) inputTensor[ 0 ][ 3 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yBR ][ xBR ] ) inputTensor[ 0 ][ 4 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ 5 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ y ][ x ] ) inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(SliceQPY – 42)/6 } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ yTL ][ xTL ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpY( CroppedYPic[ yTL ][ xBR ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = InpY( CroppedYPic[ yBR ][ xTL ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 3 ] = InpY( CroppedYPic[ yBR ][ xBR ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 4 ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 5 ] = InpC( CroppedCrPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 6 ] = 2(SliceQPY – 42)/6 } } |
| 4..255 | reserved |

nnrpf_out_sample_idc equal to 0, 1, 2, or 3 indicates that the sample values output by the post-processing filter are binary16, binary32, binary64, or binary128 floating point values, respectively, as specified in IEEE 754-2019, in the range of 0, inclusive, to 1, exclusive. Functions OutY and OutC for converting luma sample values and chroma sample values, respectively, output by the post-processing, to integer values at bit depths BitDepthY and BitDepthC, respectively, are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, Round( x * ( ( 1  <<  BitDepthY ) − 1 ) ) )	(6)

OutC( x )= Clip3( 0, ( 1  <<  BitDepthC ) − 1, Round( x * ( ( 1  <<  BitDepthC ) − 1 )) )	(7)

The value of nnrpf_out_sample_idc shall be in the range of 0 to 255, inclusive. Values of nnrpf_out_sample_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_out_sample_idc.

nnrpf_out_order_idc indicates the output order of samples resulting from the post-processing filter. Table XYZ3A contains an informative description of nnrpf_out_order_idc values. The semantics of nnrpf_out_order_idc in the range of 0 to 3, inclusive, are specified in Table XYZ3B, which specifies a process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for different values of nnrpf_out_order_idc and a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors. When nnrpf_purpose is equal to 1 or 3, nnrpf_out_order_idc shall not be equal to 3. The value of nnrpf_out_order_idc shall be in the range of 0 to 255, inclusive. Values of nnrpf_out_order_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_out_order_idc.

Table XYZ3A – Informative description of nnrpf_out_order_idc values

| nnrpf_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnrpf_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | reserved |

Table XYZ3B – Process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnrpf_out_ order_idc | Process StoreOutputTensors( ) for deriving sample values in the filtered picture from the output tensors |
| --- | --- |
| 0 | for ( yP = 0; yP < outPatchHeight; yP++) for ( xP = 0; xP < outPatchWidth; xP++ ) yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP if ( yY < nnrpf_pic_height_in_luma_samples && xY < nnrpf_pic_width_in_luma_samples ) if( nnrpf_component_last_flag = = 0 ) FilteredYPic[yY ][ xY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) else FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) |
| 1 | for ( yP = 0; yP < outPatchCHeight; yP++) for ( xP = 0; xP < outPatchCWidth; xP++ ) { xSrc = cLeft * horCScaling + xP ySrc = cTop * verCScaling + yP if ( ySrc < nnrpf_pic_height_in_luma_samples / outSubHeightC && xSrc < nnrpf_pic_width_in_luma_samples / outSubWidthC ) if( nnrpf_component_last_flag = = 0 ) { FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) } else { FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) } } |
| 2 | for ( yP = 0; yP < outPatchHeight; yP++) for ( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP yC = yY / outSubHeightC xC = xY / outSubWidthC yPc = ( yP / outSubHeightC ) * outSubHeightC xPc = ( xP / outSubWidthC ) * outSubWidthC if ( yY < nnrpf_pic_height_in_luma_samples && xY < nnrpf_pic_width_in_luma_samples) if( nnrpf_component_last_flag = = 0 ) { FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCbPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ 1 ][ yPc ][ xPc ] ) FilteredCrPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ 2 ][ yPc ][ xPc ] ) } else { FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCbPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 1 ] ) FilteredCrPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 2 ] ) } } |
| 3 | for( yP = 0; yP < outPatchHeight; yP++ ) for( xP = 0; xP < outPatchWidth; xP++ ) { ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP if ( ySrc < nnrpf_pic_height_in_luma_samples / 2 && xSrc < nnrpf_pic_width_in_luma_samples / 2 ) if( nnrpf_component_last_flag = = 0 ) { FilteredYPic[ ySrc * 2 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 ][ xSrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ 2 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 + 1] = OutY( outputTensor[ 0 ][ 3 ][ yP ][ xP ] ) FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 4 ][ yP ][ xP ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 5 ][ yP ][ xP ] ) } else { FilteredYPic[ ySrc * 2 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredYPic[ ySrc * 2 ][ xSrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 2 ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 + 1] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 3 ] ) FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 4 ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 5 ] ) } } } |
| 4..255 | reserved |

A base post-processing filter for a cropped decoded output picture picA is the filter that is identified by the NNR post-filter SEI message that has a particular nnrpf_id value, has nnrpf_mode_idc equal to 1 or 2, and pertains to the picture picA. If there is another NNR post-filter SEI message that has the same nnrpf_id value, has nnrpf_mode_idc equal to 3, and pertains to the picture picA, the base post-processing filter is updated by decoding the ISO/IEC 15938-17 bitstream in that NNR post-filter SEI message to obtain a post-processing filter PostProcessingFilter( ). Otherwise, the post-processing processing filter PostProcessingFilter( ) is assigned to be the same as the base post-processing filter.

The following process is used to filter the cropped decoded output picture with the post-processing filter PostProcessingFilter( ) to generate the filtered picture, which contains Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnrpf_out_order_idc.

if( nnrpf_inp_order_idc  = = 0 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

else if( nnrpf_inp_order_idc  = = 1 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples / InpSubHeightC; cTop += inpPatchHeight )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples / InpSubWidthC; cLeft += inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

else if( nnrpf_inp_order_idc  = = 2 ) {
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight)
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
}

else if( nnrpf_inp_order_idc  = = 3 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
}

nnrpf_reserved_zero_bit shall be equal to 0.

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string, as specified in IETF Internet Standard 63. The UTF-8 character string contains a URI, with syntax and semantics as specified in IETF Internet Standard 66, identifying the neural network used as the post-processing filter.

nnrpf_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnrpf_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

## Other

In subclause 8.1, add the following, yellow-highlighted table row in Table 4:

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| ... |  |
| NNR post-filter | Specified by the syntax of the SEI message |

# VVC proposal

## Payload type

In subclause D.2.1, add the following, yellow-highlighted table row:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = 208 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nnr_post_filter( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| ... |  |

In subclause D.2.2, replace 207 with 208 as indicated with yellow highlights:

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 180, 200 to 202, inclusive, and 204 to 208207, inclusive.

## Use of NNR post-filter SEI message (subclause D.9.x of VVC)

For purposes of interpretation of the NNR post-filter SEI message, the following variables are specified:

–	The variables InpPicWidthInLumaSamples and InpPicHeightInLumaSamples are derived for the referenced PPS as follows:

InpPicWidthInLumaSamples = pps_pic_width_in_luma_samples –	SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset )	(1)

InpPicHeightInLumaSamples = 	pps_pic_height_in_luma_samples –	SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset )	(2)

–	The variables CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, are set to be the 2-dimensional arrays of decoded sample values of the 0th, 1st, and 2nd component, respectively, of the cropped decoded output picture to which the NNR post-filter SEI message applies.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	InpSubWidthC is set equal to SubWidthC.

–	InpSubHeightC is set equal to SubHeightC.

–	SliceQPY is set equal to SliceQPY.

When an NNR post-filter SEI message with nnrpf_mode_idc equal to 1 or 2 and an NNR post-filter SEI message with nnrpf_mode_idc equal to 3 are present in the same picture unit and both NNR post-filter SEI messages have the same value of nnrpf_id, both NNR post-filter SEI messages shall be present in the same SEI NAL unit.

# Background

## Related contributions

JVET-T0076 [5] proposed enabling the usage of ALF as a post-filter and presented additional sharpening and bokeh effect as use cases. The contribution pointed out that ALF post-filtering enables selective region-wise sharpening or blurring.

JVET-T0092 [6] proposed neural network (NN) based super resolution post-filtering and presented a concept to convey the NN parameters in an SEI message. MPEG NNR, ONNX (Open Neural Network eXchange), and NNEF (Neural Network Exchange Format) were mentioned as potential formats for the NN parameters in the SEI message.

JVET-U0091 [7] presented syntax and semantics for a neural-network-based post-filtering SEI message, which included the following aspects:

- Purpose for the post-filtering

| nn_postfilter_type_idc | Post filtering type |
| --- | --- |
| 0 | Visual quality improvement with single input picture |
| 1 | Visual quality improvement with multiple input pictures |
| 2 | Super resolution with single input picture |
| 3 | Super resolution with multiple input pictures |
| 4..15 | Reserved |

- Neural network topology and parameter storage formats, e.g. NNEF or ONNX

- Either a URI identifying the neural network topology or a coded representation of the topology

- Either a URI identifying the neural network parameter values or a coded representation of the parameter values

JVET-V0058 [8] proposed a post-filter nesting SEI message, which contains inter-related SEI messages jointly specifying a post-filtering operation. The contribution envisioned that the nested SEI messages could include the following:

- An SEI message specifying the purpose for filtering, such as quality improvement or super-resolution, and parameters specific to the purpose, such as an applicable scaling ratio range for a super-resolution filter.

- An SEI message specifying the filter itself. Such an SEI message could for example contain or reference (through a URI) an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream.

JVET-V0058 was discussed in a joint meeting during which the following guidance was noted (quote from the meeting minutes):

It was suggested to continue with the intent to not significantly extend existing SEI messages (at least if there was not a clear plan to do so that was established in advance, and that the existence of a reserved value is not sufficient to indicate such a plan). A NOTE in the text to describe future plans could be an approach to consider for new SEI messages.

JVET-W0063 [9] proposed a NN-based super-resolution post-filter. Two sets of results were provided, where test 1 used constant neural network parameters and test 2 used content-adaptive neural network parameters per each sequence and QP that were coded with the NCTM of MPEG NNR. Test 1 was shown to provide an average BD-rate reduction over the use of VVC RPR for upsampling. Test 2 was shown to provide a consistent BD-rate reduction compared to test 1. An SEI message or systems layer signaling was proposed to convey the network parameters to the decoder.

JVET-X0111-v1 [11] proposed a content-adaptive NN-based quality enhancement post-filter where the NN parameters are coded with MPEG NNR. The results are compared with the same NN parameters coded with Lempel-Ziv-Markov chain algorithm 2 (LZMA2). A consistent BD-rate reduction with the MPEG NNR coding compared to LZMA2 coding is reported in all tested classes (B, C, D, F).

JVET-X0112 [12] proposed two SEI messages:

- An NNR post-filter SEI message for the carriage of an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream that specifies a neural-network-based post-filter.

- A post-filter purpose SEI message that currently specifies quality enhancement and super resolution as purposes and allows other purposes to be specified in the future.

JVET-Y0059 [13] applied the content-adaptation technique proposed in JVET-X0111-v1 [11] to the in-loop filter proposed in JVET-W0131 [10] when used as a post-filter. The contribution shows that content-adaptation provides consistent coding gains also for a very different neural network architecture than the one in JVET-X0111-v1 [11].

## Observations

We have the following observations based on the related contributions:

- Post-filtering can be performed for multiple purposes, including but not limited to those mentioned in JVET-U0091 and JVET-T0076.

- Content-adaptive NN-based post-processing for quality enhancement and super-resolution has been shown to provide a BD-rate reduction compared to pre-trained NN-based post-processing with the same neural network topology.

- Coding neural network parameters with MPEG NNR has been shown to be a viable approach for content-adaptive NN-based post-processing for quality enhancement and super-resolution.

# References

- Takeshi Chujoh, Yukinobu Yasugi, Keiichiro Takada, and Tomohiro Ikai, "AHG9: Colour component description for post-filter purpose SEI message," JVET-Y0073, Jan. 2022.

- Yukinobu Yasugi, Takeshi Chujoh, Keiichiro Takada, and Tomohiro Ikai, "AHG9: Data conversion description for NNR post-filter SEI message," JVET-Y0074, Jan. 2022.

- Keiichiro Takada, Yukinobu Yasugi, Takeshi Chujoh, and Tomohiro Ikai, "AHG9: Complexity description for NNR post-filter SEI message," JVET-Y0075, Jan. 2022.

- Miska M. Hannuksela, Maria Santamaria, Francesco Cricri, Emre B. Aksu, and Hamed R. Tavakoli, "AHG9: On post-filter SEI", JVET-Y0115, Jan. 2022.

- Han Boon Teo, Hai Wei Sun, Chong Soon Lim, "AHG9: ALF control SEI message", JVET-T0076, Oct. 2020.

- Takeshi Chujoh, Eiich Sasaki, and Tomohiro Ikai, "AHG9/AHG11: Neural network based super resolution SEI", JVET-T0092, Oct. 2020.

- Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", JVET-U0091, Jan. 2021.

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, and Hamed R. Tavakoli, "AHG9: On post-filter SEI", JVET-V0058, Apr. 2021.

- Takeshi Chujoh, Yukinobu Yasugi, and Tomohiro Ikai, "EE1.2.2: NN-based super resolution (JVET-V0073)", JVET-W0063, Jul. 2021.

- Hongtao Wang, Jianle Chen, Kevin Reuze, Anand Meher Kotra, Marta Karczewicz, “EE1-related: Neural Network-based in-loop filter with constrained computational complexity”, JVET-W0131, Jul. 2021.

- Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Alireza Zare, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela, "AHG11: MPEG NNR compressed bias update for the CNN based post-filter of EE1-1.1", JVET-X0111, Oct. 2021.

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, Hamed R. Tavakoli, and Maria Santamaria, "AHG9: On post-filter SEI", JVET-X0112, Oct. 2021.

- Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela, "AHG11: Content-adaptive post-processing filter", JVET-Y0059, Jan. 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).