| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0056-v1 |
| --- | --- |

| Title: | EE2-2.4: Template matching based reordering for GPM split modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, 2Han Huang, Yan Zhang, Zhi Zhang, Yao-Jen Chang, 3Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | 1chunchic@qti.qualcomm.com 2hanhuang@qti.qualcomm.com 3vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the testing results of the Test 2.4 method (Template matching based reordering for GPM split modes) in the context of Exploration Experiments 2 (JVET-Y2024). Given the motion information of the current GPM block, the test computes the respective TM cost values of GPM split modes. Then, it reorders all GPM split modes in ascending ordering based on the TM cost values. Instead of sending GPM split mode, this test signals an index using Golomb-Rice code to indicate where the exact GPM split mode is located in the reordering list. Compared with ECM-4.0, it is reported that the contribution achieves BD-rate saving of {0.30% (Y), 0.49% (U), 0.41% (V)} for Random Access and {0.44% (Y), 0.61% (U), 0.59% (V)} for Low-delay B with 3% encoding time increase. Around 2% encoding time increase on average is observed with minor decoding time fluctuation.

# Introduction

As reported previously in JVET-Y0135, both VVC spec (JVET-T2001) and ECM-4.0 regard the 64 split modes of Geometric Partitioning Merge (GPM) mode as equal-probable events and a fixed-length binary code is thus used for signaling. However, as JVET-Y0135 reported, this equal-probable assumption may not hold when the respective template matching (TM) cost values of all GPM split modes are considered, leading to the study of Test 2.4.

Test 2.4 infers the respective TM cost values of all GPM split modes, trying to rank the effectiveness of each GPM split mode for reordering. Accordingly, all the options of GPM split modes in a CU are no longer equal-probable events after reordering, and a variable-length code could thus be assigned to signaling the use of GPM split mode. As compared with ECM-4.0 under common test conditions of Exploration Experiments 2 (JVET-Y2024), Test 2.4 achieves BD-rate saving of {0.30% (Y), 0.49% (U), 0.41% (V)} for Random Access and {0.44% (Y), 0.61% (U), 0.59% (V)} for Low-delay B.

# Reordering method

The reordering method for GPM split modes as defined in Test 2.4 is a two-step process performed after the respective reference templates of the two GPM partitions in a coding unit are generated, as follows:

- blending the reference templates of the two GPM partitions using the respective weights of split modes (i.e., resulting in 64 blended reference templates) and computing the TM cost value for each of the blended reference templates;

- reordering GPM split modes based on their TM cost values in ascending order and marking the best 32 as available split modes.

The edge on the template is extended from that of the current CU, as Figure 1 illustrates. The corresponding weights used in the blending process of templates are computed using the same GPM weight derivation process, except that the weights are mapped to 0 and 8 before use depending on whichever is closer.

- The edge on templates

After ascending reordering using TM cost, an index is signaled using Golomb-Rice code (with divisor 4) to indicate the use of GPM split mode.

# Test results

The proposed method was implemented on top of the ECM-4.0 software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-Y2024). The test results are summarized below.

- Performance results of Test 2.4 method relative to ECM-4.0 anchor.

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.67% | -0.26% | 102% | 99% | x | x | x | x | x |
| Class A2 | -0.28% | -0.51% | -0.31% | 102% | 97% | x | x | x | x | x |
| Class B | -0.27% | -0.30% | -0.40% | 102% | 101% | -0.35% | -0.57% | -0.78% | 103% | 99% |
| Class C | -0.39% | -0.56% | -0.60% | 103% | 99% | -0.58% | -0.62% | -0.95% | 104% | 101% |
| Class E | x | x | x | x | x | -0.40% | -0.66% | 0.22% | 102% | 99% |
| Overall | -0.30% | -0.49% | -0.41% | 102% | 99% | -0.44% | -0.61% | -0.59% | 103% | 100% |
| Class D | -0.27% | -0.29% | -0.38% | 103% | 99% | -0.34% | 0.15% | 0.18% | 104% | 102% |
| Class F | -0.26% | -0.41% | -0.33% | 101% | 101% | -0.28% | -0.07% | -0.65% | 100% | 100% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).