| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0061-v1 |
| --- | --- |

| Title: | EE2-2.3: Template matching based OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuoyi Lv Chuan Zhou Jinrong zhang No.168 Weiwo Rd., Chang'an, Dongguan, Guangdong | Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com jinrong.zhang@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2-2.3 related to a template matching based OBMC, which was proposed in JVET-Y0076 [1]. Instead of directly using the weighted prediction, the prediction value of CU boundary samples derivation approach is decided according to the template matching costs, including using current block’s motion information only, or using neighboring block’s motion information as well with one of the blending modes.

It is reported that on top of ECM-4.0, the overall coding performance for {Y, U, V, EncT, DecT} is:

RA: BDrate : -0.12%, -0.12%, -0.13%; EncT: 99% DecT: 97%

LB: BDrate : -0.09%, -0.24%, -0.28%; EncT: 93% DecT: 93%

Introduction

In ECM, when OBMC is applied, the top and left boundary samples of a CU are refined using neighboring block’s motion information with a weighted prediction as described in JVET-L0101[1], JVET-U0100[2], JVET-K0258[3]. The OBMC sample blending operation is processed for each top/left block at the top/left CU boundaries (top-direction OBMC of top 4×4 row, and left-direction OBMC of left 4×4 column).

Proposed Method

In EE2-2.3, a template matching-based OBMC scheme is proposed. As shown in Fig. 1, for each top block with a size of 4×4 at the top CU boundary, the above template size equals to 4×1. If N adjacent blocks have the same motion information, then the above template size is enlarged to 4N×1 since the MC operation can be processed at one time, which is in the same manner in ECM-OBMC. For each left block with a size of 4×4 at the left CU boundary, the left template size equals to 1×4 or 1×4N.

Figure 1. Template

For each 4×4 top block (or N 4×4 blocks group), the prediction value of boundary samples is derived following the below steps.

Take block A as the current block and its above neighboring block AboveNeighbor_A for example. The operation for left blocks is conducted in the same manner.

First, three template matching costs (Cost1, Cost2, Cost3) are measured by SAD between the reconstructed samples of a template and its corresponding reference samples derived by MC process according to the following three types of motion information:

Cost1 is calculated according to A’s motion information.

Cost2 is calculated according to AboveNeighbor_A’s motion information.

Cost3 is calculated according to weighted prediction of A’s and AboveNeighbor_A’s motion information with weighting factors as 34 and 14 respectively.

Second, choose one approach to calculate the final prediction results of boundary samples by comparing Cost1, Cost2 and Cost 3.

The original MC result using current block’s motion information is denoted as Pixel1, and the MC result using neighboring block’s motion information is denoted as Pixel2. The final prediction result is denoted as NewPixel.

- If Cost1 is minimum, then NewPixeli,j= Pixel1i,j.

- If (Cost2 + (Cost2 >> 2) + (Cost2 >> 3)) <= Cost1, then blending mode 1 is used.

For luma blocks, the number of blending pixel rows is 4.

NewPixeli,0=26×Pixel1i,0+6×Pixel2i,0+16≫5

NewPixeli,1=7×Pixel1i,1+Pixel2i,1+4≫3

NewPixeli,2=15×Pixel1i,2+Pixel2i,2+8≫4

NewPixeli,3=31×Pixel1i,3+Pixel2i,3+16≫5

For chroma blocks, the number of blending pixel rows is 1.

NewPixeli,0=26×Pixel1i,0+6×Pixel2i,0+16≫5

- If Cost1 <= Cost2, then blending mode 2 is used.

For luma blocks, the number of blending pixel rows is 2.

NewPixeli,0=15×Pixel1i,0+Pixel2i,0+8≫4

NewPixeli,1=31×Pixel1i,1+Pixel2i,1+16≫5

For chroma blocks, the number of blending pixel rows/columns is 1.

NewPixeli,0=15×Pixel1i,0+Pixel2i,0+8≫4

- Otherwise, blending mode 3 is used.

For luma blocks, the number of blending pixel rows is 4.

NewPixeli,1=7×Pixel1i,1+Pixel2i,1+4≫3

NewPixeli,2=15×Pixel1i,2+Pixel2i,2+8≫4

NewPixeli,3=31×Pixel1i,3+Pixel2i,3+16≫5

For chroma blocks, the number of blending pixel rows is 1.

NewPixeli,0=7×Pixel1i,0+Pixel2i,0+4≫3

Simulation results

The proposed method was implemented on top of the ECM-4.0 [2] and tested using the common test conditions as defined in JVET-Y2017 [3]. Table 1 shows the overall BD-rate performance.

Table 1. BD-rate performance

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.14% | -0.03% | 98% | 100% |
| Class A2 | -0.12% | -0.21% | -0.18% | 98% | 100% |
| Class B | -0.12% | -0.01% | -0.14% | 101% | 100% |
| Class C | -0.14% | -0.16% | -0.13% | 96% | 90% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.12% | -0.13% | 99% | 97% |
| Class D | -0.18% | -0.17% | -0.08% | 98% | 90% |
| Class F | -0.14% | -0.26% | -0.32% | 101% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.27% | -0.48% | 100% | 98% |
| Class C | -0.07% | -0.22% | -0.36% | 82% | 83% |
| Class E | -0.14% | -0.23% | 0.15% | 98% | 98% |
| Overall | -0.09% | -0.24% | -0.28% | 93% | 93% |
| Class D | -0.08% | 0.37% | 0.14% | 94% | 96% |
| Class F | -0.29% | 0.30% | -0.06% | 101% | 100% |

Conclusion

This contribution reports the results of EE2-2.3 which improve the coding efficiency of OBMC. With the proposed methods, 0.12% gain and 0.09% gain can be achieved in RA and LDB configurations, respectively. It is recommended to adopted Test 2.3 into the next version of ECM software.

References

[1]	Z. Lv, C. Zhou, J. Zhang, “Non-EE2: Template Matching-based OBMC Design,” JVET-Y0076, January 2022.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

[4] V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-Y2024, January 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).